DOCUMENTATION FOR

VV     VV MM     MM DDDDDDD   EEEEEEEEE BBBBBBB   UU     UU   GGGGGG

VV     VV MMM   MMM DDDDDDDD  EE        BB     BB UU     UU  GG    GG

VV   VV  MMMM MMMM DD     DD EE        BB     BB UU     UU GG

VV   VV  MM MMM MM DD     DD EEEEEEE   BBBBBBB   UU     UU GG

VV VV   MM  M  MM DD     DD EE        BB     BB UU     UU GG   GGGG

VVV    MM     MM DDDDDDDD  EE        BB     BB UU     UU  GG    GG

V     MM     MM DDDDDDD   EEEEEEEEE BBBBBBBB   UUUUUUU    GGGGGGG

Copyright (c) 1988 by William Couture.  All rights reserved.

Published by DDI, 55 Lakeland Shores, St. Paul MN  55432   (612) 436-1000

Page   1

Trademarks used in this documentation include IBM, PC, PC-JR, PC-XT, PC-AT,

Intel, Microsoft, Tandy, The Norton Utilities, JRAM, Tall Trees Systems,

Sidekick, Zenith, Z150, Standard Brand 286, Atari ST, Adaptec,

Commodore Amiga, Quadram, Concurrent CPM-86, Core International, Lotus 123,

Hercules, NSI, and Vutek.

Page   2

Table of Contents:

Chapter 1:   Introduction.......................................  6

Chapter 2:   Getting started....................................  7

Program requirements............................  7

The VMDEBUG package.............................  8

Program overview................................  8

Compatibility................................... 10

Notes on speed.................................. 11

Chapter 3:   Memory usage and mapping........................... 14

PC-XT memory usage.............................. 14

VMDEBUG program................................. 14

Virtual memory map........................... 15

DMA mapping..................................... 17

DMA memory map............................... 17

Chapter 4:   VMDEBUG I/O mapping and hardware interrupts........ 19

I/O mapping..................................... 19

Keyboard handling............................ 19

DMA mapping.................................. 20

Speaker handling............................. 20

Hardware interrupts............................. 20

Chapter 5:   VMDEBUG instruction set............................ 24

8088 instruction set............................ 24

Flags........................................... 24

Instruction set overkill........................ 24

8087 instructions............................... 25

80286 instructions implemented.................. 25

8088 vs 80286................................... 26

Undefined opcodes............................... 27

Special opcodes: HLT and WAIT................... 27

Instruction set mnemonics....................... 28

Chapter 6:   Using VMDEBUG...................................... 30

Starting VMDEBUG................................ 30

Information common to all commands.............. 31

Breakpoints..................................... 34

Break on Memory Reference.................... 34

Break on Port Address........................ 36

Break on Instruction Type.................... 36

Break on Instruction Count................... 37

Page   3

Available Commands.............................. 37

Quick Reference.............................. 38

;  (Comment)................................. 44

A  (Assembler)............................... 45

B  (set sticky Breakpoints).................. 48

BL (List sticky Breakpoints)................. 49

BO (sticky Breakpoints OFF).................. 50

C  (Create file)............................. 51

D  (Dump).................................... 53

DS (Dump Stack).............................. 54

DW (Dump Word)............................... 55

E  (Enter)................................... 56

F  (Fill).................................... 57

G  (Go)...................................... 58

H  (Help).................................... 60

I  (Input from port)......................... 61

L  (Load file)............................... 62

M  (Calculate a mathematical expression)..... 64

N  (Name of file)............................ 65

O  (Output to port).......................... 66

P  (Proceed)................................. 67

Q  (Quit).................................... 68

R  (Registers)............................... 69

S  (Search).................................. 70

T  (Trace)................................... 71

U  (Unassemble).............................. 72

V  (View virtual screen)..................... 73

W  (Where Was I?)............................ 74

X  (eXecute, stop on special conditions)..... 75

Z7 (8087 instruction handling)............... 77

ZB (toggle Break on I/O status).............. 78

ZC (toggle PC-XT clone status)............... 79

ZD (load virtual copy of DOS)................ 80

ZE (toggle EGA/VGA screen swapping).......... 82

ZF (File browse)............................. 84

ZG (Get a directory)......................... 86

ZH (set Hotkey).............................. 87

ZI (redirect input).......................... 88

ZK (toggle Keyboard interrupt handling)...... 89

ZL (Load virtual machine image).............. 90

ZM (display size of virtual Memory).......... 92

ZN (specify Name of virtual machine image)... 93

ZO (echo output)............................. 94

ZP (Patch virtual DOS, pre-DOS 3.0).......... 95

ZR (display video Registers)................. 97

ZS (toggle Screen swapping).................. 98

ZT (set Timer (Clock) and IRQ divisors)...... 99

ZU (toggle destination of Unused IRQs).......100

ZV (set long Video retrace mode).............101

ZW (Write virtual machine image).............102

ZY (Toggle DMA remap ZZ command).............104

ZZ (Replace system ROMs).....................105

The VMDEBUG.CFG configuration file..............108

Page   4

Chapter 7:   VMDEBUG and DOS....................................109

Finding the patch for pre-DOS 3.0 COMMAND.COM...109

File buffers and disk conflicts.................111

I/O redirection.................................112

Chapter 8:   Examples...........................................114

Tracing a DOS command...........................114

Debugging a resident program as it runs.........120

Debugging a device driver as is runs............125

Booting DOS virtually........................125

Finding the device driver....................126

Working with copy protected software............132

Chapter 9:   Compatibility problems and solutions...............136

HP Vectra.......................................136

Bus mice........................................136

RAM disks.......................................137

Known quirks....................................137

Appendix A:  Format of VMDEBUG image files......................141

Image loader program............................145

Page   5

Chapter 1

Introduction

------------

VMDEBUG stands for Virtual Machine DEBUGger.  The program is an

interpreter whose language is 8088 machine code extended with the real-mode

instructions of an 80286, with provisions to simulate and/or allow access

to the hardware of an IBM PC-XT, in an addressing space independent of the

normal hardware.  However, unlike the familiar BASIC interpreter, VMDEBUG

is specialized as a debugging tool, not a development tool.  Software

development capabilities have been left out in favor of additional

debugging options.

VMDEBUG was created because a program gone wrong can, and usually will,

take out everything it can, including the interrupt vector table.  This

renders even debuggers that use NMI (Non-Maskable Interrupt) hardware useless.

The hardware solution to this problem is either a logic analyzer or an

in-circuit emulator, both of which are very expensive and require hardware

attachments to the system under test.  VMDEBUG, on the other hand, presents

a simple software solution with most of the power of complex hardware.

The VMDEBUG interpreter can stop execution at any point, examine or alter

memory or registers, examine the program, determine where the program has

been, and much more.  Unlike normal debuggers (such as the well-known DOS

DEBUG) these functions are accomplished by a program OUTSIDE the addressing

space of the program or system under test, so that VMDEBUG can never be

altered or destroyed by the errant program.  Unlike DEBUG and other

debuggers, VMDEBUG also has the ability to trace DOS itself, or the ROMs,

and set breakpoints within ROM.

The price for VMDEBUG's power is speed.  Each instruction must be

fetched from virtual memory, decoded, checked for validity, interpreted,

data modified in virtual memory (if the instruction accesses memory),

external interrupts checked, and special conditions considered.  Given

these requirements, the program is considerably slower than actual machine

language.

Page   6

Chapter 2

Getting started

---------------

Program requirements

--------------------

VMDEBUG requires an IBM PC, IBM PC-XT, IBM PC-AT, or a close clone.  It

has been tried on an IBM PC-JR, and found not to work, due to the hardware

differences.  Also, a Tandy 1000 will probably not work unless it has a DMA

controller installed.

An IBM PC-AT or clone is recommended, because of the speed of the

interpreter.

The memory requirements are 256K for VMDEBUG to do anything, and 320K for

interpreting a minimal DOS (no ramdisks, resident programs, etc.) and

applications program.  640K is HIGHLY recommended.

Also note that a 704K DOS space (some people patch their operating system

to recognize an additional 64K at A000:0000) is not provided for.

The program has beem tested under PC-DOS versions 3.3, 3.2, 3.1, 3.0, 2.1,

and 2.0.  Dos 1.x is not supported, and will display an error message and

return to the command prompt if you try to use VMDEBUG.  If you find that some

version of DOS is not compatible, please let me know.  To use VMDEBUG with DOS

versions before 3.0, see Chapter 7 for information on patching a bug in

COMMAND.COM.

VMDEBUG uses file handles to redirect I/O within VMDEBUG itself, and a

minimum of FILES=7 is required to allow for both command line redirection

and internal redirection.

VMDEBUG has not been tested on an expanded memory board, such as an Intel

Aboveboard or one of the LIM expanded memory boards, and thus it is not known

if it will work.

Note, however, that such memory is not properly mapped into the virtual

memory space, and will be directly accessed by VMDEBUG (see chapter 3).

Any data or ramdisk stored in this memory will be subject to the whims of

VMDEBUG.

The JRAM-2 memory board by Tall Trees Systems for the PC is another bank

switching memory board, and will work with VMDEBUG, but again the expanded

memory is directly accessed and modified by VMDEBUG.

Disk caching software should not be used with VMDEBUG.  See chapter 7 for

details.

Page   7

The VMDEBUG package

-------------------

VMDEBUG is distributed on a single DSDD (Double Sided, Double Density)

floppy disk, and is not copy protected.  For your own safety, do not use

VMDEBUG from the distribution disk.   Instead, make a copy of the

distribution disk and put the original in a safe place.  If a hard disk

is being used, the VMDEBUG program may be copied to the hard disk, and

placed where a PATH search can find it.

There are 5 files in the VMDEBUG package, but only VMDEBUG.EXE is needed

to run VMDEBUG.  The rest are test and demonstration programs talked about

in this manual.

The files on the disk include:

VMDEBUG.EXE - The VMDEBUG interpreter.  VMDEBUG.EXE does not require any

supporting files, although a configuration file, VMDEBUG.CFG, may be

used.  See chapter 6 for details on VMDEBUG.CFG.

CLICKER.ASM - The example resident program from Chapter 8, complete with

mistakes.

NULLDEV.ASM - The example null device driver from Chapter 8.

LOADER.ASM - The image loader program from Appendix A.

TOSCREEN.DAT - An example program for masochists, TOSCREEN.DAT is a program

intended to be interpreted by VMDEBUG being interpreted by VMDEBUG, etc.

The program sends the alphabet to the 1st line of a CGA display.  It

ends with an undefined opcode to stop the interpreter, and thus should

not be used under DOS.

Program overview

----------------

Upon entry to the program, the user is presented with a DEBUG-like

interface that can be used to load and execute programs, to load a virtual

memory copy of DOS, and to execute programs that require DOS.  Commands may

be entered that will dump memory, unassemble code, change values in memory

or registers, etc., but these commands WILL ONLY AFFECT THE VIRTUAL MEMORY

SPACE.  No command will look at or alter "real" system memory.

Optionally, the program will access the I/O hardware of an IBM PC-XT

and work with real-world devices such as printers, disk drives (both floppy

and hard disks), and the keyboard.  Special provisions have been made for

access to several of these devices.

Page   8

Access to the disk drives uses DMA (Direct Memory Access) to bypass the

CPU when writing to memory.  This is good, in that VMDEBUG is not fast enough

to keep up with a disk controller, and bad, in that the program does not

reside in the memory that it thinks it owns, and thus the requested transfer

addresses are wrong, and can wipe out the operating system, programs, etc.

As a result, VMDEBUG will re-direct DMA requests on the fly to send the data

to the correct area in virtual memory, or to ROM (non-writeable memory)

when the requested address does not exist in virtual memory.

Access to the keyboard (via hardware interrupt) could be directly passed

to VMDEBUG, but instead interrupts are trapped and passed to VMDEBUG one at

a time.  The user may type input as fast as is comfortable in an application

under VMDEBUG.  The interpreted BIOS will process them when it can.

This also allows VMDEBUG to be invisible to the application, as control

is returned to VMDEBUG via a programmable hotkey (the default is hotkey is

Right-Shift and Esc together) that is invisible to the program being

interpreted.  Also, the Right-Shift is not passed to VMDEBUG until it is

determined that the next key is not the hotkey.  If the right-shift is

pressed, various keys are pressed, and then the hotkey, all keystrokes up

to the hotkey will be in the queue.  Additionally, the use of Left-Shift

and hotkey (the hotkey itself, not Right-Shift and hotkey) will suspend

execution of the interpreter, and wait for any keystroke.  An Esc will

will return to VMDEBUG, while anything else will resume execution.  The

above comments on shift-key transparency apply.

Direct access to the video display adapter memory is allowed in all video

modes.  The screen may become messy as VMDEBUG and the application both write

to the screen, but the application will not crash.  Optionally, the program

will swap both screen memory and video modes at entry and exit from the

interpreter for CGA, MDA, EGA, and VGA displays.  See chapter 6 for details.

All hardware interrupts (timer, keyboard, floppy disk, hard disk, serial

port, parallel port, and the reserved lines) are intercepted, noted by

VMDEBUG, and any special processing done (such as slowing down the timer,

or buffering the keyboard).  In the case of the timer and keyboard, the

interrupt is passed on to the "real" system.  The other interrupts (initiated

by programs running under VMDEBUG) merely flag the interrupt to VMDEBUG and

perform an IRET.

The instruction prefetch queue of the 8088 is not supported.  This might

make itself felt in programs that are VERY self-modifying, though it should

be invisible under normal circumstances.  As the instruction prefetch queue

is included in the hardware for increased throughput, it seems justifiable

to leave it out of the software for the same reason.

One of the more powerful features of VMDEBUG is the ability to interrupt

execution on memory reference.  Unlike most/all debuggers that depend on

the INT 3 instruction (with all its limitations, such as the need to be the

1st byte in an instruction, destroyed by self-modifying code, dependent

on the integrity of both the interrupt vector and the interrupt handling

routine, etc.) for breakpoints, VMDEBUG looks at memory addresses while doing

Page   9

virtual memory mapping and determines whether or not this is a protected

address/range of addresses.  This means that ANY memory reference may be

used to signal an interrupt to VMDEBUG, including ANY byte within an

instruction, a PUSH, a POP, a read or write or modify to an address, or

even an LEA to a protected range.  Also, such interrupts cannot be defeated

by nor interfere with self-modifying code.

To promote silence and serenity, the status of the speaker I/O port is

saved when VMDEBUG returns to the command level and the speaker is turned off.

When the program starts execution again, the speaker status is restored.

Compatibility

-------------

VMDEBUG may not be a perfect emulation of an 8088 processor, since every

8088 program has not been tried with VMDEBUG.  The known instructions have

been implemented and tested, and a large number of programs and operations

have been tried under VMDEBUG, among them:

Running DOS

Booting DOS  (Note that the version of DOS booted under VMDEBUG does not

have to be the version that VMDEBUG is running under.)

Booting and running Concurrent CPM for the IBM-PC

Booting and running Wendin-DOS (a multi-user, multi-tasking OS)

Lotus 123, Release 1A

Lotus 123, Release 2

Flight Simulator   (Providing that the BIOS and hardware of the machine

VMDEBUG is running on will support these programs.)

The Norton Utilities

CHKDSK

DEBUG

FORMAT

CORETEST   (The Core International hard disk test program)

VMDEBUG    (Yes, VMDEBUG will interpret itself, and will interpret

VMDEBUG interpreting a program.  For the curious, the

program TOSCREEN.DAT is included, that sends the alphabet

to the first line of the CGA screen.  It ends with an

undefined opcode to stop the interpretation.)

etc...

The only program that has been found not to work is a program that

sets the DOS time and date from a real time clock.  There is a status port

on the board that says that some value has changed (down to 1/1000 of a

second) since the last data read, and this port is checked after the data

has been read and processed, VMDEBUG simply is not fast enough to do the

required processing.  This is an extreme case, and almost no other programs

have timing requirements this stringent.

Copy protection schemes that depend on timing events upon the disk

probably will not work.

Page  10

Notes on Speed

--------------

The first thing that people will note about VMDEBUG is its speed, or

lack thereof.  VMDEBUG will execute, on a standard 4.77MHz PC-XT, about 541

instructions per second (ips), which is .000541 MIPS.  A 10MHz AT clone

will give about 2732 ips (.002732 MIPS).  This is hardly what one would

expect from any sort of hardware, but when it is considered as a piece

of software, things take on a different perspective.

The SI program from the Norton utilities reports a performance index of

8.4 running VMDEBUG on a standard XT, with a clock divisor of 2000.  This

translates into a real speed factor of 238, which means that, on the

average, 238 machine instructions are used to fetch an opcode from virtual

memory, decode it, call the function to execute that instruction, execute

the function, set flags, store results back into virtual memory, check for

special conditions, and check for interrupts.  The instruction stream is

continually being watched for trap addresses (if any), invalid instructions,

and the activation of the hotkey; as well as the execution of interrupt

handlers by both the "real" operating system and VMDEBUG.

There will be no substantial changes to the basic design of the

instruction executor, so the current speed for VMDEBUG is a "worst case"

time.  Considerable optimization has already been done, though there

could be as much as a 10% increase in the speed of future versions.

When memory range trapping is used, VMDEBUG must do additional processing

at every virtual memory reference (including instruction fetch, PUSH/POP

instructions, INT's, any instruction that references or changes memory,

etc.) to determine the validity of the address(es) in question.  This means

additional processing time, and thus slower execution speed.  Note, however,

that it takes no more time to watch a range of memory addresses than to

watch a single byte.

For comparison, some real numbers come from a standard "operating system

mix" test, which involves performing an INT 20h to return from a freshly

loaded DOS 3.1 to running under DOS 3.1.  This executes the interrupt, checks

keyboard interrupts, resets vectors, deallocates memory, does a checksum

on memory to test for the transient portion of COMMAND.COM, searches the

environment space for the prompt string, and displays the prompt, plus

nobody-knows-what else deep inside DOS.  All times are measured to the

nearest multiple of 5 seconds for ease of bookkeeping.

The results for 3 machines are summarized below:

Page  11

IBM PC-XT       IBM PC-AT        Standard Brand 286

6MHz,               10MHz,

1 wait state         1 wait state

number of        ---------      ------------      ------------------

breakpoints

-----------

0            541 ips          1587 ips             2732 ips

(1m, 40s)        (40 sec)             (25 sec)

1            440 ips          1285 ips             2197 ips

(2m, 5s)         (45 sec)             (30 sec)

2            401 ips          1163 ips             1995 ips

(2m, 20s)        (50 sec)             (30 sec)

3            369 ips          1062 ips             1810 ips

(2m, 30s)        (55 sec)             (35 sec)

4            341 ips           975 ips             1682 ips

(2m, 40s)         (1 min)             (35 sec)

5            317 ips           903 ips             1540 ips

(2m, 55s)         (1m, 5s)            (40 sec)

Actual execution speeds depend on the type of instruction being executed.

NOP's, for example, are much faster than IMUL's.

One obvious question is why not run programs at full speed by executing

the actual program, and then switch to interpretation during the critical

pieces?  The answer is that such an approach would prevent the use of virtual

memory, one of the most powerful features of VMDEBUG.  A normally executing

program will have control of the CPU, and thus will have access to all of

memory, which includes VMDEBUG.  Thus a mistake in the part of the program

that is considered "safe" could wipe out the program, VMDEBUG, the system,

and anything else.

Later, if VMDEBUG takes control, it must set up a virtual memory space

in an area that is not being used.  However, DOS allocates all available

memory to a program, and so the program must be a "nice" program that

returns unused memory to the system (which very few programs do, as

multi-tasking is not available to take advantage of the memory).  Also

note that the piece returned must be large enough to accommodate the

virtual memory space, including DOS, VMDEBUG, the program, and all its

memory requirements.  This is a most unlikely event.

Another approach might be to have VMDEBUG allocate sufficient space for

the virtual memory (a "hole"), and let the program run in the area above

that.  However, when attempting to set up the virtual memory space, it

would be necessary to put the program, VMDEBUG, DOS, and the "hole" in the

"hole!"  Trying to eliminate VMDEBUG and the "hole" from the virtual memory

area would necessitate changing all DOS pointers that point to VMDEBUG and

the missing memory, and all pointers in the program that refer to an absolute

memory location, as well as having large "gaps" in memory addressing.

Page  12

The difference between VMDEBUG and programs for other machines that do

8088 emulation and are faster than VMDEBUG is that VMDEBUG is a true

interpreter, rather than a run-time translator.  Having written both sorts

of program (both VMDEBUG and 8080 emulation on the 8088), I would like to

comment on this.  Emulation programs work on the principle of taking the

opcode and using it as an index into a jump/call table to skip to the

processing of an instruction (or instructions) that perform the equivalent

function on the host processor, using the host processors memory, flags,

registers, etc.  Such programs, written in machine language, can be very

fast, but are no more of a debugging tool than running the original chip

(if even that good), and will gleefully destroy themselves.

VMDEBUG, on the other hand, is a true interpreter.  Each instruction is

processed according to the rules of the 8088, oblivious to the program

executing it and the memory mapping of the computer.  Each instruction may

be individually watched, or any location in memory may be set as a trap

point.  VMDEBUG allows a blow-by-blow execution of the program, watching

errors as they happen.  Both types of program perform the "same" function,

executing 8088 machine code, but only VMDEBUG is set up to watch and to

analyze as an omnipotent observer.

Page  13

Chapter 3

Memory usage and mapping

------------------------

PC-XT memory usage

------------------

Following the lead of IBM, the industry has settled on a memory map

for 8088/8086 and 80286 machines that has the following broad outlines:

hex addresses     usage

-------------     ----------------------------------

00000-9FFFF       RAM

A0000-AFFFF       Reserved for Video expansion

B0000-B3FFF       Monochrome Video display adapter

B4000-B7FFF       Reserved for Video expansion

B8000-BBFFF       CGA Video display adapter

BC000-BFFFF       Reserved for Video expansion

C0000-C7FFF       ROM expansion (EGA ROM)

C8000-CBFFF       PC-XT Hard Disk Controller ROM

CC000-EFFFF       ROM expansion

F0000-FFFFF       BASIC ROM and BIOS ROM

This arrangement allows for 640K of addressable memory, and despite

"memory hungry" programs, much of it is often used as a ramdisk to speed

up disk accesses.

VMDEBUG program

---------------

The VMDEBUG program uses the large amount of memory available to set up

a virtual machine that runs independently of the real operating system.

As a result, VMDEBUG cannot be crashed while the program being debugged is

watched.

The VMDEBUG program itself is a small memory model C program, occupying

a maximum of 64K code and 64K data.  The program is restricted to the small

memory model, because there are times it must know the location of another

part of the program.  Using a larger memory model would mean that the

code segment would not be known in all cases.

VMDEBUG must assume that it "owns" all memory above itself to set up the

virtual memory workspace.  As resident programs and ramdisks are set up in

the DOS allocation chain at low memory, they will not be affected by VMDEBUG.

Also, note that the use of the RAM above VMDEBUG as virtual memory

precludes the possibility of a "shell escape," running another copy of

COMMAND.COM to process DOS commands while VMDEBUG is active.

Page  14

VMDEBUG can, and usually does interpret DOS as well as any application

program.  In order to do this, a copy of DOS is placed in virtual memory

and the application program is loaded normally.  Note, however, that this

requires sufficient memory to copy DOS and all resident programs (device

drivers, ramdisks, etc.) into virtual memory and STILL have enough to run

the application.   VMDEBUG requires that there be 64K of free memory above

the virtual DOS space to load a virtual copy of DOS.

Note that VMDEBUG works with memory in 64K blocks.  Since 64K and 256K

chips are the current standard, this should not be a problem.  However,

several machines, such as the HP Vectra (an AT compatible) set aside some

RAM at the top of memory for use by their BIOS.  See chapter 9 for more

details.

If large ramdisks are used, interpreting DOS may not be possible without

booting a copy of DOS virtually.  For example, DOS V3.1, 30 file buffers, a

160K ramdisk, and no resident programs on a 640K IBM PC-XT leaves VMDEBUG

with a virtual memory size of 256K, which is not sufficient to load another

copy of DOS (which includes the file buffers and the ramdisk) from the

real DOS.

When working above 640K DOS space, VMDEBUG allows direct access to the

address space.  This allows virtual applications and virtual DOS to write

to the video memory and to use the ROM's located there.

Virtual memory map

------------------

The relationship between real memory and virtual memory is shown in the

following figure.

Page  15

Actual Memory                           Virtual Memory

Segment ����������������ͻ                    �������������������ͻ Segment

�                �                    �  �                � maps to

0000  � DOS            �                    �  � Virtual DOS    �  3000

����������������͹                    �  ����������������͹

�                �                    �  �                �

1000  � VMDEBUG code   �                    �  � Available RAM  �  4000

����������������͹                    �  ����������������͹

�                �                    �  �                �

2000  � VMDEBUG data   �                    �  � Available RAM  �  5000

����������������͹�Ŀ                 �  ����������������͹

� Segment 0000   �  �                 �  �                �

3000  � Virtual RAM    �  �        ��������Ĵ  � Available RAM  �  6000

����������������͹  �        �        �  ����������������͹

� Segment 1000   �  �        �        �  �                �

4000  � Virtual RAM    �  �        �        �  � Available RAM  �  7000

����������������͹  �        �        �  ����������������͹

� Segment 2000   �  �        �        �  �                �

5000  � Virtual RAM    �  �        �        �  � Available RAM  �  8000

����������������͹  �        �        �  ����������������͹

� Segment 3000   �  �        �        �  �                �

6000  � Virtual RAM    �  ����������        �  � Available RAM  �  9000

����������������͹  �                 �������������������͹

� Segment 4000   �  �                 �  � "Non-Existant" �

7000  � Virtual RAM    �  �                 �  � RAM            �  D000

����������������͹  �                 �  ����������������͹

� Segment 5000   �  �                 �  � "Non-Existant" �

8000  � Virtual RAM    �  �        ��������Ĵ  � RAM            �  D000

����������������͹  �        �        �  ����������������͹

� Segment 6000   �  �        �        �  � "Non-Existant" �

9000  � Virtual RAM    �  �        �        �  � RAM            �  D000

top of ����������������͹���        �        �������������������͹

RAM   ����������������͹�Ŀ        �        �������������������͹

� Video          �  �        �        �  � Video          �

A000  � Expansion      �  �        �        �  � Expansion      �  A000

����������������͹  �        �        �  ����������������͹

�                �  �        �        �  �                �

B000  � Video          �  �        �        �  � Video          �  B000

����������������͹  �        �        �  ����������������͹

� EGA BIOS and   �  �        �        �  � EGA BIOS and   �

C000  � Hard Disk BIOS �  ��������ĳ�������Ĵ  � Hard Disk BIOS �  C000

����������������͹  ��Ŀ     �        �  ����������������͹

� ROM            �  �  �     �        �  � ROM            �

D000  � Expansion      �  �  �������        �  � Expansion      �  D000

����������������͹  ����              �  ����������������͹

� ROM            �  �                 �  � ROM            �

E000  � Expansion      �  �                 �  � Expansion      �  E000

����������������͹  �                 �  ����������������͹

�                �  �                 �  �                �

F000  � BIOS           �  �                 �  � BIOS           �  F000

����������������ͼ���                 �������������������ͼ

Page  16

DMA mapping

-----------

VMDEBUG also maps DMA requests from the interpreted DOS and applications

program into virtual memory.  Without going into details here (see Chapter 4

on I/O ports), the DMA mapping requires that virtual memory mapping starts

on a 64K boundary.  Usually a chunk of memory is left between the end of

VMDEBUG's data segment and the start of virtual memory, which can be as

large as 65520 bytes (64K - 16 bytes (1 paragraph) ), but this cannot be

helped if disk access is to be supported.

Of the 4 DMA channels on a PC-XT, 3 are mapped into virtual memory,

corresponding to channels 1, 2, and 3 of the 8237 DMA controller. Channel 0,

used for memory refresh, is not mapped.

To support VMDEBUG's use on an AT, DMA channels 5, 6, and 7 are also

mapped, although ALL channels are limited to a 1M (20 bit) address space.

DMA memory map

--------------

For the DMA map used, see the figure below.  Note that this is the same

mapping as the memory mapping used by VMDEBUG.

Page  17

Actual Memory                             DMA Memory

Segment ����������������ͻ                    �������������������ͻ Segment

�                �                    �  �                � maps to

0000  � DOS            �                    �  � Virtual DOS    �  3000

����������������͹                    �  ����������������͹

�                �                    �  �                �

1000  � VMDEBUG code   �                    �  � Available RAM  �  4000

����������������͹                    �  ����������������͹

�                �                    �  �                �

2000  � VMDEBUG data   �                    �  � Available RAM  �  5000

����������������͹�Ŀ                 �  ����������������͹

� Segment 0000   �  �                 �  �                �

3000  � Virtual RAM    �  �        ��������Ĵ  � Available RAM  �  6000

����������������͹  �        �        �  ����������������͹

� Segment 1000   �  �        �        �  �                �

4000  � Virtual RAM    �  �        �        �  � Available RAM  �  7000

����������������͹  �        �        �  ����������������͹

� Segment 2000   �  �        �        �  �                �

5000  � Virtual RAM    �  �        �        �  � Available RAM  �  8000

����������������͹  �        �        �  ����������������͹

� Segment 3000   �  �        �        �  �                �

6000  � Virtual RAM    �  ����������        �  � Available RAM  �  9000

����������������͹  �                 �������������������͹

� Segment 4000   �  �                 �  � "Non-Existant" �

7000  � Virtual RAM    �  �                 �  � RAM            �  D000

����������������͹  �                 �  ����������������͹

� Segment 5000   �  �                 �  � "Non-Existant" �

8000  � Virtual RAM    �  �        ��������Ĵ  � RAM            �  D000

����������������͹  �        �        �  ����������������͹

� Segment 6000   �  �        �        �  � "Non-Existant" �

9000  � Virtual RAM    �  �        �        �  � RAM            �  D000

top of ����������������͹���        �        �������������������͹

RAM   ����������������͹�Ŀ        �        �������������������͹

� Video          �  �        �        �  � Video          �

A000  � Expansion      �  �        �        �  � Expansion      �  A000

����������������͹  �        �        �  ����������������͹

�                �  �        �        �  �                �

B000  � Video          �  �        �        �  � Video          �  B000

����������������͹  �        �        �  ����������������͹

� EGA BIOS and   �  �        �        �  � EGA BIOS and   �

C000  � Hard Disk BIOS �  ��������ĳ�������Ĵ  � Hard Disk BIOS �  C000

����������������͹  ��Ŀ     �        �  ����������������͹

� ROM            �  �  �     �        �  � ROM            �

D000  � Expansion      �  �  �������        �  � Expansion      �  D000

����������������͹  ����              �  ����������������͹

� ROM            �  �                 �  � ROM            �

E000  � Expansion      �  �                 �  � Expansion      �  E000

����������������͹  �                 �  ����������������͹

�                �  �                 �  �                �

F000  � BIOS           �  �                 �  � BIOS           �  F000

����������������ͼ���                 �������������������ͼ

Page  18

Chapter 4

VMDEBUG I/O mapping and hardware interrupts

-------------------------------------------

I/O mapping

-----------

When run in PC-XT clone mode, VMDEBUG allows direct access to the I/O

ports, with a few small exceptions for keyboard handling and DMA.

Keybaord handling

-----------------

To allow a comfortable typing rate and implement an invisible "hotkey"

return to the VMDEBUG program, keyboard interrupts and the scancode values

at port 60 hex are buffered within VMDEBUG and are passed to the interpreted

BIOS when the interpreted BIOS has finished processing the current interrupt.

This buffer is 60 (decimal) bytes long, which corresponds to 30 keypresses

and releases (20 on the new, extended keyboard with 12 function keys, as

it sends 1 make code and 2 break codes), which must include such things as the

shift, ctrl, and alt keys as well as the regular keys.  Any keystrokes beyond

the end of the buffer are ignored.

The keyboard command port on a PC-XT at I/O address 61 hex (port B of the

8255 PPI) is watched, and if the command byte has the high bit set, VMDEBUG

is flagged to "turn off" the keyboard.  The high bit is cleared before it is

sent to the hardware, so that the keyboard is NOT turned off.  This prevents

the user from hitting a breakpoint while the keyboard is off, leaveing a

command prompt with a dead keyboard.

When the keyboard is enabled by clearing the high bit of port 61 hex,

this is flagged to VMDEBUG, which "turns on" the keyboard and, if available,

presents the next interrupt from the keyboard queue.  The byte is passed

modified to the hardware.

The keyboard on a PC-AT works in a different fashion, and is handled with

its own special requirements.  The port that is watched is 64 hex, the I/O

address for the 8042 keyboard controller.  When a command of AD hex is sent

to the keyboard, the fact is flagged to VMDEBUG, which "turns off" the

keyboard and DOES NOT pass the command to the keyboard, for the reasons noted

above.

The keyboard is enabled by a value of AE hex sent to port 64 hex, which is

passed to the keyboard.  The next interrupt "from" the AT keyboard, however,

is not presented until BOTH the keyboard is enabled and the 8259 is enabled.

Note that BOTH methods of watching the keyboard are employed

simultaneously, no matter which machine you are running on.

Page  19

DMA mapping

-----------

DMA mapping intercepts the values sent to the DMA page registers at I/O

addresses 81 to 83 hex, corresponding to DMA channels 2, 1, and 3, and

maps the values into the virtual memory space.  Each DMA page is a 64K block

of memory, so the page registers contain a 4-bit value, that combines with

a 16-bit value from the DMA controller to generate a 20-bit address.

To support VMDEBUG's use on an AT, the additional DMA channels at I/O

addresses 89 to 8B hex, corresponding to DMA channels 6, 7, and 5, also

are mapped into the virtual memory space.

DMA channel 0 is used for memory refresh on a PC-XT, and is not mapped

into the virtual address space, but merely passed through.  Also, the AT

refresh register is not mapped.

All DMA transfers, including those on the AT, are limited by VMDEBUG to

a 20-bit (1 megabyte) address space.  As the program is supposed to be an

emulation of a real mode 8088, not a virtual mode 80286, there should be no

programs attempting to address memory or DMA outside this range.

Speaker handling

----------------

To promote silence and serenity, the status of the speaker I/O port is

saved when VMDEBUG returns to the command level and the speaker is turned off.

When the program starts execution again, the speaker status is restored.

Hardware interrupts

-------------------

When in PC-XT clone mode and using the G, P, or X commands to interpret

programs (in order to have applications and DOS running under VMDEBUG

properly talk to the hardware of the system) the hardware interrupts are

noted as they occur so that VMDEBUG may simulate each interrupt.

Two of the interrupts (the keyboard and timer tick) are also passed

to the "real" BIOS, as well as the interpreted BIOS.  The other interrupts,

as they originate from the application or DOS running under VMDEBUG, are

not passed to the "real" operating system but terminated within VMDEBUG.  The

interpreted BIOS, DOS, and applications take care of the required processing.

The interrupt structure on the PC-AT is slightly different from the PC-XT.

Since the XT interrupts are a subset of the AT interrupts, VMDEBUG watches

the interrupts as implemented on the AT, with the priority considerations

noted below.

Page  20

IRQ 0, the clock interrupt, is handled specially.  The interrupt is

counted by VMDEBUG, and when the count is greater than the divisor (which

can be set by the ZT command), a clock interrupt is passed to the

interpreter.  This prevents the clock interrupt from taking too much

processing time from the interpreter.

The clock interrupt is also passed to the real BIOS, so that the real

system keeps track of the current time.  However, to prevent the real

BIOS from turning off the floppy disk drive motor while the interpreter

is using the floppy drive, the count-down byte for BIOS, located at 40:40,

is set to its maximum value of FF (about a 14 second delay) at every timer

tick.  Besides keeping the motor on while VMDEBUG is interpreting a program,

it also allows you to stop the interpreter, execute commands, and return

to the program with the motor still running, if you take under 14 seconds.

The interrupt requests (IRQs) 2 through 15 (2 through F hex) can also

have divisors associated with them via the ZT command, implemented

primarily to handle devices such as bus mice (see chapter 9).  The default

is no division, and unused IRQs (those not passed into VMDEBUG due to

division) are passed along to the normal interrupt handler (this may be

changed by the ZU command).

For all interrupts using IRQs 2 through 15, if the interrupt is passed

into VMDEBUG for processing by the interpreter, the end of the interrupt

chain is VMDEBUG.

Page  21

The hardware interrupts watched are:

interrupt  IRQ  address    function      comments

(hex)           (hex)

---------  ---  ---------  ------------  ------------------------------

8       0   0:20-23    Time of day   passed to "real" BIOS,

floppy motor turn off disabled

9       1   0:24-27    Keyboard      passed to "real" BIOS

A       2   0:28-2B    Reserved

B       3   0:2C-2F    Comm port 2

C       4   0:30-33    Comm port 1

D       5   0:34-37    Hard disk     function is Alternate printer

on the PC-AT

E       6   0:38-3B    Floppy disk

F       7   0:3C-3F    Printer

70       8   0:1C0-1C3  Realtime clock

71       9   0:1C4-1C7  Reserved

72      10   0:1C8-1CB  Reserved

73      11   0:1CC-1CF  Reserved

74      12   0:1D0-1D3  Reserved

75      13   0:1D4-1D7  Coprocessor

76      14   0:1D8-1DB  AT Hard disk

77      15   0:1DC-1DF  Reserved

PC-XT interrupts are handled so that the lowest number IRQ signal has the

highest priority.  On the PC-AT, IRQs 8-15 are assigned the same priority

level as IRQ 2 (and, in fact, are routed through the IRQ2 interrupt handler).

Page  22

IBM PC-XT IRQ PRIORITY

----------------------

IRQ 0  (highest priority)

IRQ 1           IBM PC-AT IRQs

IRQ 2  <----------------------

|  IRQ 8   (highest priority)

|  IRQ 9

|  IRQ 10

|  IRQ 11

|  IRQ 12

|  IRQ 13

|  IRQ 14

|  IRQ 15  (lowest priority)

--------------

IRQ 3

IRQ 4

IRQ 5

IRQ 6

IRQ 7  (lowest priority)

Inside VMDEBUG, putting IRQs 8-15 at the same level as IRQ 2 would

create excessive overhead during interrupt processing, so interrupts are

handled strictly in the reverse order of their IRQ number (i.e. IRQ 15 has

the lowest priority, IRQ 0 has the highest priority).

The Non-maskable interrupt (NMI, which uses the vector of INT 2), is

not watched by VMDEBUG.  The NMI is used to indicate parity error on

memory (which probably means that some chip has failed), or to indicate

coprocessor exception.  Since VMDEBUG should not continue if memory fails,

and the coprocessor is not used, there is no reason to watch the NMI, since

excluding the NMI makes the program simpler.  Hardware assisted debuggers

that use NMI breakout switches to regain control of the system will have to

do without their switch when running under VMDEBUG.

Page  23

Chapter 5

VMDEBUG instruction set

-----------------------

8088 instruction set

--------------------

The entire 8088 instruction set has been implemented.  For details on

what instructions are available, see any reference on 8088/8086 assembly

language.

Flags

-----

The flags of the 8088 have been implemented so that all defined flags

are calculated.  However, if an instruction leaves some flag or flags

undefined, VMDEBUG will not change the current state of the flag.  If

someone has found that these flags have some definition not listed by Intel

and uses that, their program will probably not work.

Instruction set overkill

------------------------

In addition to the standard instruction set, some additional

instructions have been implemented to fill in the instruction space.  These

instructions are:

hex code          instruction

--------------    -------------------------------------------

82 mod 001 r/m    OR  mem/reg,data (byte)

82 mod 100 r/m    AND mem/reg,data (byte)

82 mod 110 r/m    XOR mem/reg,data (byte)

83 mod 001 r/m    OR  mem/reg,data (sign extended)

83 mod 100 r/m    AND mem/reg,data (sign extended)

83 mod 110 r/m    XOR mem/reg,data (sign extended)

8C mod 001 r/m    MOV CS,mem/reg

8F mod xxx r/m    POP mem/reg          (xxx not equal to 000)

C6 mod xxx r/m    MOV mem,data (byte)  (xxx not equal to 000)

C7 mod xxx r/m    MOV mem,data (word)  (xxx not equal to 000)

Note that some of the above opcodes (such as the 82 mod reg r/m codes) are

defined according to one source, and not defined according to another source.

The standard reference book for creating VMDEBUG was "The 8086 Book" by

Russell Rector and George Alexy (Osborne/McGraw-Hill).

Page  24

8087 Instructions

-----------------

The 8087 instructions are only minimally supported (i.e., VMDEBUG

produces ESC instructions while decoding/unassembling that act as NOP's

to the interpreter).  8087 emulation is not planned.

80286 Instructions implemented

------------------------------

All of the real-mode instructions from the expanded set implemented on

the 80188, 80186, 80286 (by the way, has anyone ever seen the 80288?)

and 80386 processors have been implemented within VMDEBUG to support the ROM

BIOS on those machines.  They are not intended for general use, and programs

being developed for the 8088 should avoid using them:

hex code                instruction

--------------------    -----------

60                      PUSHA

61                      POPA

62 mod reg r/m          BOUND reg,mem

68 iijj                 PUSH immediate

69 mod reg r/m iijj     IMUL immediate

6A ii                   PUSH immediate (sign extended)

6B mod reg r/m ii       IMUL immediate (sign extended)

6C                      INSB

6D                      INSW

6E                      OUTSB

6F                      OUTSW

C0 mod 000 r/m count    ROL mem/reg,count (byte)

C0 mod 001 r/m count    ROR mem/reg,count (byte)

C0 mod 010 r/m count    RCL mem/reg,count (byte)

C0 mod 011 r/m count    RCR mem/reg,count (byte)

C0 mod 100 r/m count    SHL/SAL mem/reg,count (byte)

C0 mod 101 r/m count    SHR mem/reg,count (byte)

C0 mod 111 r/m count    SAR mem/reg,count (byte)

C1 mod 000 r/m count    ROL mem/reg,count (word)

C1 mod 001 r/m count    ROR mem/reg,count (word)

C1 mod 010 r/m count    RCL mem/reg,count (word)

C1 mod 011 r/m count    RCR mem/reg,count (word)

C1 mod 100 r/m count    SHL/SAL mem/reg,count (word)

C1 mod 101 r/m count    SHR mem/reg,count (word)

C1 mod 111 r/m count    SAR mem/reg,count (word)

C8 iijjkk               ENTER jjii,kk

C9                      LEAVE

The interrupt generated by the BOUND instruction exception is INT 5, which

is used for the print-screen function on IBM's and compatibles.  As the

address placed on the stack is the START of the BOUND instruction, a return

Page  25

from the print-screen function will generate another BOUND instruction

exception and another print-screen, in an endless loop.  Thus, the BOUND

instruction is not normally used in 80286 code.

8088 vs 80286

-------------

VMDEBUG is set up to emulate an 8088 processor, rather than an 80286.

Although the two processors are object code compatible, there are some

small distinctions between the two.  In these cases, VMDEBUG is programmed

to emulate the 8088 behavior, rather than that of the 80286.

PUSH SP - On the 8088, SP is decremented, then pushed.  On the 80286,

a temporary value is set to SP, SP is decremented, and the temporary value

is pushed.  Thus, the sequence:

PUSH SP

POP AX

CMP AX,SP

will yield a zero result on an 80286, and non-zero on an 8088.  VMDEBUG

follows the 8088 method of decrementing SP, then pushing SP.

SEGMENT WRAP AROUND - On the 8088, a word reference to offset FFFF (hex)

within a segment will use offset FFFF as the low byte, and offset 0000 as

the high byte, wrapping around in the segment.  On the 80286, a word reference

to offset FFFF will cause an exception interrupt of type 0D (hex) (i.e., an

INT 0D will be executed).  VMDEBUG follows the 8088 rule of wrapping the high

byte of the word reference back to 0000.

Note that INT 0D is used for the disk on a PC-XT, and alternate printer

on a PC-AT, and so that this exception will probably crash an 80286 based

machine.

SHIFTS - On the 8088, a multi-bit shift (number of bits is specified in

the CL register) will shift by the specified number of bits.  On the 80286,

the number in the CL register is taken modulo 32 (decimal) to limit the time

spent on the instruction.  Thus, the code sequence:

MOV CL,21

MOV AL,FF

SHR AL,CL

(with all numbers in hex) will leave AL=3F on a 80286, and AL=0 on the 8088.

VMDEBUG follows the 8088 rule, and performs all specified shifts.

MULTIPLE INSTRUCTION PREFIXES ON STRING PRIMITIVES AND INTERRUPTS - On

the 8088, if more than one prefix is attached to a string primitive (one

of them must be the REPZ or REPNZ prefix), and the instruction is interrupted

by an external interrupt (such as the clock interrupt on the PC), the

instruction pointer placed on the stack corresponds to the LAST prefix.  On

the 80286, the instruction pointer placed on the stack corresponds to the

FIRST prefix.  VMDEBUG follows the 8088 rule, so that prefixes can be dropped

during repeated string operations.

Page  26

Undefined opcodes

-----------------

The undefined opcode space for VMDEBUG is as follows:

hex code                       Comments

----------------               ---------------------------

0F                             Would be POP CS on an 8088, this is

the first byte of the extended opcode

space on the 80286 and 80386.

62 11 reg r/m                  BOUND reg,reg (i.e. BOUND AX,BX etc.)

63                             80286 ARPL instruction

64                             80386 FS segment override

65                             80386 GS segment override

66                             80386 operand size prefix

67                             80386 address size prefix

C0 mod 110 r/m

C1 mod 110 r/m

D0 mod 110 r/m

D1 mod 110 r/m

D2 mod 110 r/m

D3 mod 110 r/m

D4 xx  (xx not equal to 0A)    D4 0A is AAM

D5 xx  (xx not equal to 0A)    D5 0A is AAD

D6

F1

F6 mod 001 r/m

F7 mod 001 r/m

FE mod 010 r/m

FE mod 011 r/m

FE mod 100 r/m

FE mod 101 r/m

FE mod 110 r/m

FE mod 111 r/m

FF 11 011 r/m                  CALL FAR SI, or some such

FF 11 101 r/m                  JMP FAR CX, or some such

FF mod 111 r/m

Special opcodes: HLT and WAIT

-----------------------------

The opcodes HLT and WAIT (called FWAIT when used with an 8087) are

treated specially.  Although valid opcodes, they send the actual 8088 into

a potentially infinite wait.

Under VMDEBUG, the HLT instruction is treated as a jump to itself, so

that the interpreter is in an infinite loop.  The only way to break out of

this via software is by an external interrupt, which pushes the address of

the instruction after the HLT on the stack, so that returning from the

interrupt falls into the next instruction.  This mimicks the behavior of

the HLT instruction on an 8088 chip (except that HLT stops the chip, rather

than looping), so that code that uses the HLT instruction to wait for

Page  27

external interrupts, such as the BIOS on the Adaptec hard disk controller,

functions properly.  Note that using the HLT instruction to wait for an

interrupt opens a window of vulnerability during which the desired interrupt

can occur before entry to the HLT instruction unless extreme care is taken.

This is usually a VERY rare occurance, because of the speed of the

processor, but will occur more frequently under VMDEBUG.

The WAIT instruction waits for a completion code from a coprocessor before

proceeding.  However, as the 8087 is not simulated, there can be no completion

signal, and the wait will be infinite.  It is not clear, however, whether

the instruction should be treated as a breakpoint (stop and inform user) or

as a NOP (proceed to next instruction).  The Z7 command (see chapter 6)

will toggle VMDEBUG between treating WAIT as a NOP (the default), and as a

breakpoint.

Instruction set mnemonics

-------------------------

The mnemonics used are (almost) the standard IBM macro assembler

mnemonics.  The difference is that the FAR versions of the JMP, and CALL

instructions are called JMPF and CALLF (one word), rather than JMP FAR

and CALL FAR.  A complete list of all mnemonics used by VMDEBUG is given

in alphabetical order below:

AAA     AAD     AAM     AAS     ADC     ADD     AND     BOUND

CALL    CALLF   CBW     CLC     CLD     CLI     CMC     CMP

CMPSB   CMPSW   CS:     CWD     DAA     DAS     DEC     DIV

DS:     ENTER   ES:     ESC     HLT     IDIV    IMUL    IN

INC     INSB    INSW    INT     INTO    IRET    JA      JAE

JCXZ    JG      JGE     JMP     JMPF    JNA     JNAE    JNG

JNGE    JNO     JNS     JNZ     JO      JPE     JPO     JS

JZ      LAHF    LDS     LEA     LEAVE   LES     LOCK    LODSB

LODSW   LOOP    LOOPNZ  LOOPZ   MOV     MOVSB   MOVSW   MUL

NEG     NOP     NOT     OR      OUT     OUTSB   OUTSW   POP

POPA    POPF    PUSH    PUSHA   PUSHF   RCL     RCR     REPZ

REPNZ   RET     RETF    ROL     ROR     SAHF    SAR     SBB

SCASB   SCASW   SHL     SHR     SS:     STC     STD     STI

STOSB   STOSW   SUB     TEST    WAIT    XCHG    XLAT    XOR

Note that the Intel mnemonics, found in the Intel reference books,

differ slightly from the mnemonics used by IBM, Microsoft, and VMDEBUG.

In most cases, there is only one possible mnemonic for a given instruction.

However, in the case of some of the conditional jumps, the string primitives,

etc, there are several possibilities.  The mnemonics used in VMDEBUG for

assembly, unassembly and break on instruction type are as follows:

Page  28

mnemonic used     instruction name or alternate forms

-------------     -----------------------------------

JNAE              JC, JB

JAE               JNC, JNB

JZ                JE

JNZ               JNE

JNA               JBE

JA                JNBE

JPE               JP

JPO               JNP

JNGE              JL

JGE               JNL

JNG               JLE

JG                JNLE

LODSB             LODS BYTE, LODB

LODSW             LODS WORD, LODW

MOVSB             MOVS BYTE, MOVB

MOVSW             MOVS WORD, MOVW

STOSB             STOS BYTE, STOB

STOSW             STOS WORD, STOW

CMPSB             CMPS BYTE, CMPB

CMPSW             CMPS WORD, CMPW

SCASB             SCAS BYTE, SCAB

SCASW             SCAS WORD, SCAW

INSB              INS  BYTE

INSW              INS  WORD

OUTSB             OUTS BYTE

OUTSW             OUTS WORD

LOOPNZ            LOOPNE

LOOPZ             LOOPE

CALLF             CALL FAR

JMPF              JMP FAR

RETF              RET FAR

REPZ              REPE

REPNZ             REPNE

Page  29

Chapter 6

Using VMDEBUG

-------------

Starting VMDEBUG

----------------

To start the VMDEBUG program, all that is necessary is to have the

VMDEBUG.EXE program available in the current directory, or where a PATH

search can find it, and then type VMDEBUG followed by a return at the DOS

prompt.

After VMDEBUG has been started, it will automatically perform several

warm-up functions:

* The screen mode will be reset (thus clearing the screen) to insure

that VMDEBUG has a known mode for screen swapping.  If you are in a

standard mode (25 x 80 text), you can skip the reset by using ANY

command line parameter when starting VMDEBUG.  The value of the

command line parameter is not checked, merely its presence.

* Memory mapping for the "non-existant" RAM is initalized to the segment

at D000 (hex).  This may be changed by the ZM command, discussed

below.

* If a configuration file named VMDEBUG.CFG is present in the current

directory, it is read and processed by VMDEBUG.  See the section on

VMDEBUG.CFG in this chapter for more information.

When these functions are completed, VMDEBUG is ready for user commands.

At initalization, VMDEBUG emulates a "naked" 8088 processor.  There is

no operating system loaded, I/O ports are not present, the interrupt

controller is not present, and screen swapping is not enabled.  This allows

the user to test code that is not dependent on (or does not want) PC

hardware or DOS.  VMDEBUG will be allowed to use more of the PC by use

of several commands:

* ZC - Toggle PC-Clone status.  This allows access to the I/O ports.

* ZS - Toggle screen swapping.

* ZD - Set PC-Clone status, enable the interrupt controller, save the

current screen contents in the virtual screen, and load DOS.

These and other commands are discussed fully in the following sections.

Page  30

Information common to all commands

----------------------------------

All commands are 1 or 2 letters long, are entered after the '>' prompt,

and may be in upper or lower case.  A Return terminates the command line,

and must be typed before the command will be processed.  The maximum

length of the command line, including all arguments, is 127 characters.

Spaces before the command are ignored, as are spaces between the letters of

a 2 character command.  No spaces are required between a command and its

arguments, although spaces are required between arguments.

Arguments can be addresses, lengths, ranges, values, strings, or

filenames.  The syntax for each type of argument is given in the following

table, with the notation segment for a segment address, and offset for an

offset address:

Argument      Syntax

---------     --------------------------------------------------

Address       offset or segment:offset

Length        Lnumber.  A length may not be entered by itself, but

instead must follow an offset.

Range         offset or offset1,offset2 or segment:offset

or segment:offset1,offset2.  Offset2, if used, must

be larger than offset1.  When used as a memory breakpoint,

it can be followed by a 'hit count', in the form

range=count

Value         hex number (if a byte is required, it must be < 100)

String        "A string must be quoted with either double"

'or single quotes.  It may contain any character except a'

"quote of the same type as the delimiting quotes."

'It cannot be blank (two quotes together).'

Filename      An unquoted string, it may contain both a drive

specifier (such as A:) and backslashes (\) for the

pathname.

All numbers must be hexadecimal (hex), and may consist of expressions

with hex numbers (the digits 0-9 and the letters A-F), the names of the

16-bit registers (AX, BX, CX, DX, SI, DI, BP, SP, DS, ES, SS, CS, IP or PC),

and the following operators (in order of lowest precedence to highest):

XOR: ^   OR: |   AND: &   Rotate Left: <   Rotate Right: >   Subtraction: -

Addition: +   Modulo (MOD): %   Division: /   Multiplication: *   NOT: ~

Any letters (hex constants or register names) can be in upper or lower case.

Numeric expressions cannot contain any spaces, or the expression will be

read as 2 expressions.  It is illegal to enter negative numbers or signed

positive numbers.  The NOT operator (~) is a unary operator, and is illegal

when used in binary operations.

Parenthesis (), square brackets[], and curly brackets {} are allowed

in expressions, and each modifies the order of evaluation.  Parenthesis

simply change the order of evaluation, with the expression in the innermost

parenthesis being evaluated first.

Page  31

Square brackets are an operator, which perform a word memory fetch from

virtual memory.  The square brackets may enclose an offset, in the format

[OFFSET] with the current DS used as the segment, or a segment and offset

in the format [SEGMENT:OFFSET].  Both segment and offset may be expressions

of arbitrary complexity.

Curly brackets are also an operator, like square brackets, except that a

byte fetch is performed, and used as a word with a high byte of 0.

For example:

Legal Expressions    Illegal Expressions   Comments

-----------------    -------------------   ----------------------------

1234                 qrst                  Not hexadecimal

1+a+3+B              +1                    signed positive illegal

23-17                -4                    signed negative illegal

SS^AX|6&4            ss^ax|6& 4            no spaces allowed

[DS:AX]<{BX:CX}      1+[BX)                bracket/parenthesis mismatch

33*7-2               33x7-2                x not hex or operator

1+~2                 1~2                   ~ is a unary operator

The precedence of operators is parenthesis/brackets first, then NOT (~)

multiplication (*), division (/), modulo (%), addition (+), subtraction (-),

rotate right (>), rotate left (<), AND (&), OR (|), and then XOR (^).  If

there is more than one operator of a given type in an expression, they are

processed from left to right.

The command line may be edited using the standard DOS editing keys.

As a review, they are:

key          function

---------    --------------------------------------------

F1 or -->    copy 1 character from the template

F2 x         copy characters from template up to x

F3           copy all characters from template

F4 x         skip characters in template up to x

F5           clears the template

Ins          insert characters without affecting template

Del          delete 1 character in the template

Esc          start again without affecting template

Where the template is the previous command line.

Examples of commands:

Command                       Comments

------------------            ------------------------------------

N moon.com                    Give VMDEBUG a filename

D 100:30 L20                  Dump 20 (hex) bytes, starting at the

given address

F 100 200 "Test!"             Fill the memory range 100 to 200 in the

current data segment (DS) with a string

G ds:80,ff                    Execute program with a breakpoint on the

range DS:80 to DS:FF

T 1c                          Trace program for 1C (hex) instructions

ZH 0e                         Set hotkey to right-shift + backspace

(0E is the scancode, in hex, for backspace)

Page  32

Most error messages are self-explanatory, and will be discussed with the

associated commands.  However, some messages are common to all routines and

are explained here:

Error         Explanation

---------     -----------------------------------------------------

command?      VMDEBUG could not recognize the command.

number?       A number was expected but not received, or was illegal.

range?        An address or range was expected, but not received.

Note that, at its simplest, this could be a number.

string?       A parameter that could be either a number or a string

was neither a number nor a string.

filename?     A filename was not received, or has not yet been set,

or the file was not found.

The most common display in VMDEBUG is the register and current instruction

display.  The registers are displayed in the form "REGNAME=VALUE", along with

the current state of the flags, on the first two lines.  The next line (and

possibly on several lines, if there are instruction prefixes) contain the

address of the instruction (in segment:offset form), the hexcode of the

instruction, the unassembled instruction, and possibly the address and value

of an operand (if the instruction references memory).  As an example,

immediately after DOS is loaded, the register display might look like this:

AX=0000  BX=0000  CX=0000  DX=0000  SP=FFFE  BP=0000  SI=0000  DI=0000

DS=1234  ES=1234  SS=1234  CS=1234  IP=0100   NV UP EI NT PL NZ NA PO NC

1234:0100  CD20         INT     20

The flags are displayed in the order:

overflow, direction, interrupt, trap, sign, zero,

auxiliary carry, parity, and carry

with the following two letter abbreviations for their on and off states:

flag            ON abbreviation             Off abbreviation

-------------   -------------------------   -----------------------

overflow        OV (OVerflow)               NV (No oVerflow)

direction       DN (DowN, auto-decrement)   UP (UP, auto-increment)

interrupt       EI (Enable Interrupts)      DI (Disable Interrupts)

trap            TR (TRap (single step))     NT (No Trap)

sign            NG (NeGative)               PL (PLus, positive)

zero            ZR (ZeRo)                   NZ (Not Zero)

aux. carry      AC (Auxiliary Carry)        NA (No Auxiliary carry)

parity          PE (Parity Even)            PO (Parity Odd)

carry           CY (CarrY)                  NC (No Carry)

These are the same abbreviations used by DEBUG, except for the trap flag,

which is not displayed by DEBUG.  The above list of ON and OFF abbreviations

is also used when setting flags via the R command.

When a program is executed via the G, P, or X commands, the interpretation

is stopped and return to the command level through the hotkey, which defaults

to Right-Shift and Esc.  All other keypresses are passed to the interpreted

program.  The Right-Shift is coded into the VMDEBUG program, but the ESC key

may be substituted with almost any other key by means of the ZH command.

Page  33

A second hotkey is also set up with a default of Left-Shift and Esc, and

is changed via the ZH command along with the Light-Shift hotkey.  The

Left-Shift hotkey causes the interpreter to pause and wait for another

keypress before proceeding.  If the keypress is an Esc, VMDEBUG returns to

the command level; otherwise, the program resumes execution.

When a program is being traced via the T command, or X with the T option,

return to the command prompt is by the Esc key.  This cannot be changed by

the user.

Breakpoints

-----------

There are four types of breakpoints available in the VMDEBUG program:

break on memory reference, break on port reference, break on instruction

type, and break on instruction count.  They are discussed below.

When a breakpoint of any type is encountered, VMDEBUG stops at the start

of the instruction which caused the breakpoint.  For example, if the last

byte of stack to be used for a PUSHA instruction (from the 80286 instructions

implemented) is a breakpoint, NOTHING will be pushed to stack.  Similarily,

if the last byte of stack to be used for a POPA instruction is a breakpoint,

NOTHING will be popped from the stack; all register values will remain

unchanged.

Break on Memory Reference

-------------------------

Break on memory reference is set by having VMDEBUG look for references to

those memory ranges as the program executes.  They are NOT set by placing

INT 3 (1-byte interrupt) instructions in the code.  This means that they may

be ANY byte ANYWHERE in memory, not simply the first byte of an instruction.

Also, they will cause a break on any reference to that address, including

execution, read, write, push to, pop from, or an LEA.  They will not

interfere with or be destroyed by self-modifying code.

Also, memory breakpoints are not limited to a single byte.  Entered in

[segment:]offset[,offset2][=count] format, they may encompass up to 64K as

a single breakpoint.  It takes no more time to watch a range of bytes as a

breakpoint than it takes to watch a single byte as a breakpoint.

Up to 20 memory ranges may be watched at once, including the breakpoints

set by the B (set sticky breakpoints) and P (proceede, stop after the next

instruction) commands.

Break on memory reference may also have a count associated with the

breakpoint.  When the specified count of "hits" has occurred, VMDEBUG

generates the break.  The format for this feature is:

RANGE=COUNT

Page  34

with no spaces between the range, the equals sign, and the count.  For

example, some typical breakpoints with counts might be:

1234=5

DS:SI=2

0:84,87=8

etc...

If a count is not given, a count of 1 (break on any reference) is used.

It must be noted, however, that since VMDEBUG generates breakpoints at

the bus level (during address generation), the count of memory references

may not be what is expected by the user.  For example, the instruction:

ADD [1234],AL

references the byte at DS:1234 twice - once for the fetch, and once to store

the result - while the instruction

ADD AL,[1234]

only makes one reference to DS:1234.  Word references and breakpoint on

memory range can add to the count.  For example, with a breakpoint of

DS:1234,1235 the instruction

ADD [1234],AX

will count 4 memory references - there will be one count for each byte on

the fetch, and one count for each byte on the store.

If code is executing in a breakpoint range, the count can become even more

convoluted.  For example, if DS and SS are equal to CS (a .COM program) and

the breakpoint is set as CS:0,FFFF=2A the following code will produce memory

counts as shown:

hex code  instruction          number of counts

--------  -------------------  -----------------------------------------

40        INC AX               1 (for the code byte)

FEC3      INC BL               2 (for the code bytes)

0004      ADD [SI],AL          4 (2 for the code, 2 for the memory ref.)

FF063412  INC WORD PTR [1234]  8 (4 for code, 2 for fetch, 2 for store)

3A1C      CMP BL,[SI]          3 (2 for code, 1 for memory fetch)

7413      JZ  0120             2 (for the code)

3B063412  CMP AX,[1234]        6 (4 for code, 2 for memory fetch)

740D      JZ  0120             2 (for the code)

89163412  MOV [1234],DX        6 (4 for code, 2 for memory store)

CD21      INT 21               8 (2 for code, 6 bytes pushed to stack)

Also, note that hardware interrupts will access the stack, giving extra

counts, as will stack access within DOS calls.

Placing a breakpoint on the address of an interrupt vector is the

preferred way to watch for interrupts.  If a hardware interrupt is being

watched, VMDEBUG will break when it tries to look at the vector address,

and the interrupt will not execute.  In this case, the interrupt flag is

not cleared, and the interrupt is tried again after execution of the next

instruction.

The use of memory breakpoints slows down the VMDEBUG interpreter, as

additional instructions must be executed at every memory access.  See

chapter 2 for some comparisons on speed versus number of breakpoints.

Page  35

When a breakpoint is set using a register name, the value used is that of

the register when the breakpoint was set.  The breakpoint does not change as

the contents of the registers change during program execution.

Break on Port Address

---------------------

Breakpoints may be set on port addresses via the ZB command.  These

breakpoints are sticky, and are disabled only by another ZB command, although

break on port address will be ignored by the T (trace) command.  Either all

ports will be watched, which stops VMDEBUG at any I/O instruction, or a list of

selected ports (up to 20) will be watched.  Like break on memory reference,

break on port address is watched at the bus level, and cannot be detected

by any program code.

The use of break on port address does not noticably slow down the VMDEBUG

interpreter.  Although additional processing must be done, I/O instructions

are relatively rare, and the additional overhead is not noticable.

Break on Instruction Type

-------------------------

Break on instruction type is implemented via the X command.  This is

done so that the G and P commands do not have to cope with the additional

overhead and so run faster.  When break on instruction type is used, the

instructions being decoded are watched, and if the desired instruction

type is found, a breakpoint occurs.

Breakpoint on instruction type has two variations.  The first type is not

sensitive to the arguments of the instruction.  Thus, setting a breakpoint

on a MOV instruction would break on

MOV  AX,1

MOV  DI,CS

MOV  WORD PTR [1234],5678

or ANY other MOV instruction.  Thus, this variation is best used with

instructions that have unique uses, such as LDS, IN, OUT, INT, etc.  It is

included as it does not noticably slow down the VMDEBUG interpreter, as the

overhead is always present in the X command.  This variation of break on

instruction type will not recognize segment override prefixes as breakpoints.

The second variation of break on instruction type uses a quoted string

for comparison with the instruction name and operands unassembled by VMDEBUG

as the progrm executes.  The strings are compared only to the length of

the quoted string entered by the user, thus as much or as little of the

instruction as desired may be used for the breakpoint.  For example, if

a string of "mov ax,[" is entered, VMDEBUG will break only on an instruction

that moves memory to the AX register.  Note that a segment override prefix

is a separate instruction string, and will not be recognized as part of the

above breakpoint, although a segment override instruction is a valid string

for setting a breakpoint.

Page  36

When the string is entered, it is changed to upper case, and the spacing

between the opcode and the operands is adjusted to that used by VMDEBUG.

The string is assumed to start with the opcode.  VMDEBUG does not check the

string for a legal instruction type, that is the responsibility of the user.

This variation on breakpoint on instruction type slows down the VMDEBUG

interpreter by about half, because of the additional processing needed to

generate strings on instruction decode and compare them with the desired

string.

Break on Instruction Count

--------------------------

Break on instructions count is implemented via the X command.  This is

done so that the G and P commands do not have to cope with the additional

overhead and so run faster.  When break on instruction count is used, the

running count is compared to the desired count, and when the two are equal

a break occurs.

Break on instruction count is useful in situations where an error occurs

at a know location in program execution, but other breakpoints provide too

many "false alarms" to the user.

The use of break on instruction count does not noticably slow down

the VMDEBUG interpreter, as the associated overhead is always present in

the X command.

Available Commands

------------------

The following is assumed in all command descriptions:

* If a command can be stopped before it is finished, the ESC key will

do it.  Ctrl-C and Ctrl-Break are disabled inside VMDEBUG.

* These commands affect/look-at only virtual memory, not real memory.

* A parameter surrounded by square brackets [] is optional.  Do not type

the brackets.

* A set of parameters surrounded by curly brackets {} means: "pick one of

the enclosed."  Do not type the brackets.

* All numbers in this manual are in hex unless otherwise noted.

* All numbers entered into VMDEBUG must be in hex.

* All commands and numbers may be upper or lower case.

* All numeric values can be expressed as a hexadecimal number or the

name of a 16-bit register (AX, BX, CX, DX, SP, BP, SI, DI, ES, DS,

SS, CS, IP or PC), with the operators ^ (XOR), | (OR), & (AND), ~ (NOT),

< (rotate left), > (rotate right), -, +, % (MOD), /, and * allowed.

Parenthesis, square brackets, and curly brackets used in expressions

behave as described above.  Register names and hex numbers can be upper

or lower case.

Page  37

* A breakpoint set using a register name uses the value of the register

at the time of the command.  The breakpoint will not change as the

register value changes.

* Seg is short for segment, and ofs is short for offset.  Offset may

also appear in forms such as eofs (ending offset), startoffset

(starting offset), etc.

* A command will scan only as much of the command line as needed.

Any errors in the remaining command line will be ignored.

Quick Reference

---------------

The following quick reference gives the available commands, the syntax

of any parameters, and a brief description.  For more information and

examples, see the detailed descriptions below.

Page  38

;

A line beginning with a semi-colon is a comment, and is ignored.

A [[seg:]ofs]

Start the Assembler.  All 8088 instructions and 80286 real-mode instructions

are recognized.  To avoid confusion with addressing modes, hex and register

expressions are not allowed, only hex constants.

B [seg1:]ofs1[,endofs1][=count1]...[[seg10:]ofs10[,endofs10][=count2]]

Set sticky breakpoints.  Sticky breakpoints are not discarded when control

returns to VMDEBUG.  There is a limit of 10 sticky breakpoints, and 20

total breakpoints.  Entering a new set of sticky breakpoints clears the old

set.  The default segment is the CS at the time the command was issued.

B

Toggle active status of sticky breakpoints.  This allows you to turn them

off without clearning them.  If there are no sticky breakpoints, having them

on does nothing.

BL

List the current sticky breakpoints.

BO

Turn sticky breakpoints OFF.  This command clears sticky breakpoints.

C filename

Create filename on disk.  The length is in the BX:CX register pair, and the

the memory to write starts at CS:IP.  Any previous file contents are lost.

This command could be called Write, but then what would I call Where?

C [seg:]ofs drive sector numberofsectors

Write the specified memory (default segment is DS) to sectors on disk (drive

A=0, B=1, etc.).  Sectors are logical beginning with the boot sector at 0.

NOTE: SECTOR SIZE IS ASSUMED TO BE 512 BYTES!

D [[seg:]ofs [{ofs2 Llength}]]

Dump memory in BYTE (ASCII) format.  The default segment is the last dump

segment, or DS after a ZD command.  The default length is 80H (128 decimal)

bytes.  Use ESC to stop the dump.

DS [number]

Dump stack in WORD format, with top-of-stack closest to the next command

prompt. (i.e. The top-of-stack is at the "bottom," like the actual stack.)

The default is to dump 8 words, or the number can be specified.  Use ESC to

stop the dump.

DW [[seg:]ofs [{ofs2 Llength}]]

As D above, but dump in WORD format.  The default length is 40H (64 decimal)

words.

E [seg:]ofs {byte1 string1} [{byte2 string2} ...]

Enter values into memory, for the length of the given values.  The default

segment is DS.

Page  39

F [seg:]ofs {ofs2 Llength} {byte1 string1} [{byte2 string2} ...]

Fill the specified memory range with the given values, repeating the values

as necessary.  The default segment is DS.

G [=[CS:]IP] [[seg1:]ofs1[,endofs1][=cnt1]...[seg20:]ofs20[,endofs20][=cnt20]]

Start executing program at current or given CS:IP, with up to 20 breakpoints

on memory ranges.  The default segment for breakpoints is CS, the range is 1

byte if endofs is not given, and the count of 'hits' until the break occurs

defaults to 1.  If sticky breakpoints are used, the number of breakpoints

will be decreased so that there are only 20 total breakpoints.

H [name]

Help, optionally on a specific command or all commands staring with name.

? is another name for help.  For example, 'H R' shows the register command,

while 'H EX' shows the examples.

I portnumber

Get the value from the port and display it.

L

Load the file specified by the N command.  If the file is not an .EXE file,

it is loaded at the current CS:IP.

L [seg:]ofs drive sector numberofsectors

Read the specified sectors (drive A=0, B=1, etc) to memory (the default

segment is DS).  Sectors are logical beginning with the boot sector at 0.

NOTE: SECTOR SIZE IS ASSUMED TO BE 512 BYTES!

M expression

Evaluate and show the result of a mathematical expression.

N filename

Set the pathname and filename to load.  This is not copied into the PSP.

O portnumber value

Output the value to the specified port.

P [=[CS:]IP] [[seg1:]ofs1[,endofs1][=cnt1]...[seg19:]ofs19[,endofs19][=cnt19]]

As G above, but a breakpoint is automatically placed on the instruction after

the current instruction.

Q

Quit VMDEBUG and return to DOS.

R [regname [value]]

The default is to display all registers and the current instruction.  If a

register name is given, the new value can be on the command line, or you

will be prompted for a new value.  The register name can be any of the 16-

bit registers (AX, BX, CX, DX, SI, DI, BP, SP, DS, ES, SS, CS, IP or PC),

or F (the Flag register).  The flag names and the abbreviations used are:

Overflow, Direction, Interrupt, Trap, Sign, Zero, Aux Carry, Parity, Carry

with ON abbreviations of: OV, DN, EI, TR, NG, ZR, AC, PE, CY

and OFF abbreviations of: NV, UP, DI, NT, PL, NZ, NA, PO, NC

Page  40

S [seg:]ofs {ofs2 Llength} {byte1 string1} [{byte2 string2} ...]

Search the specified memory for a byte pattern.  The default segment is DS.

T [=[CS:]IP] [number]

Trace program, starting at the current or given CS:IP.  The default is to

trace 1 step, or the number may be specified.  Interrupts, breakpoints, and

Break-On-I/O are turned off for this command.  Use ESC to stop the trace.

U [[seg:]ofs [{ofs2 Llength}]]

Unassemble memory.  The default segment is the last Unassemble segment, or

CS after a ZD command, and the default length is 20H (32 decimal) bytes.

Use ESC to stop the disassembly.

V

View the virtual screen, if screen swapping is enabled.  Pressing any key

will restore the VMDEBUG screen.

W [number]

Where was I?  VMDEBUG can remember and display the last 2047 (7FFH) machine

instructions that were executed.  The default is to display the last 16 (10H)

instructions, or the number can be specified.  The Where queue is reset by

the L, ZL, and ZD commands, and by setting CS or IP.  Use ESC to stop.

X [=[CS:]IP] [T] [Qnumber] [N{insttype "instruction string"}]

[[seg1:]ofs1[,endofs1][=count1]...[seg20:]ofs20[,endofs20][=count20]]

eXecute program from the current or given CS:IP.  This is like the G

command, but more breakpoint options are available.  They are:

T - eXecute in trace mode, showing registers and instruction at every step.

When in trace mode, the ESC key, not the hotkey, returns to VMDEBUG.

Qnumber - Quit after a specified number of instructions have been executed.

N{insttype "instruction string"} - Quit on instruction type (e.g. ADD, MOV,

IRET, etc.) or when an instruction that matches "instruction string" is

decoded.  (e.g. "MOV AX" or "INC WORD PTR [SI+0ADC" or ...)

Z7

Toggle 8087 WAIT instruction between a NOP (default) and a breakpoint.

ZB [port1] ... [port20]

Toggle Break-on-I/O status.  The I/O instructions are IN, OUT, INSB, INSW,

OUTSB, and OUTSW.  The new status is displayed.  If a list of ports is

given, Break-on-I/O is set ON for those ports only.

ZC

Toggle PC-Clone status.  When enabled, access to the I/O ports is allowed.

The new status is displayed.  PC-Clone status is enabled by the ZD command.

ZD [commandline]

Load a virtual memory copy of DOS from the real DOS, and set up the PSP

fields from the command line, if given.  The program is set to an INT 20

instruction (terminate program).  PC-CLONE (see the ZC command) status is

enabled, and the current state of the screen is saved for screen swapping.

This command can be used any time a fresh start is desired.

Page  41

ZE [E [num_seq num_crt num_attrib num_graphics]]

Toggle EGA/VGA screen swapping.  The new status is displayed.  This is for

boards that emulate CGA/MDA at the hardware level, so that VMDEBUG will not

attempt to use EGA/VGA screen swapping when it is not appropriate.  Use the

ZE command before the ZD command, as saving the screen state uses mode

swapping for EGA/VGA boards.

ZF [filename]

Enter showfile screen.  The specified file is displayed on a text screen.

If no name is given, the last file used is re-displayed.

ZG [filespec]

Get a directory, optionally with a path and/or a wildcard filename.

ZH number

Set the hotkey (used to interrupt VMDEBUG during a G, P, or X command) to

Right-Shift and hotkey, where number is the SCANCODE of the hotkey.  The

default hotkey is Right-Shift and ESC (scancode = 1).  Some suggestions

for hotkeys are 0E (backspace), 1C (return), and 39 (spacebar).

ZI [name]

Redirect input to come from the file or device given by name.  If no name

is given, input comes from CON (the keyboard).  Pressing ESC will return

input to CON, as will a blank line in the input file.  This redirection is

invisible to the virtual DOS loaded by the ZD command.

At the start of VMDEBUG, the current directory is checked for a file named

VMDEBUG.CFG.  If found, it is used by the ZI logic to configure VMDEBUG.

ZK

Toggle passing of the keyboard interrupt into VMDEBUG.  The default is to

pass it.  This allows you to isolate the program under test from accidental

keystrokes.

ZL

Load the saved state of the virtual machine from the file specified by the

ZN command.  This allows you to continue a debugging session later, or to

start a debugging session from a known state several times.

ZM [seg]

Display the size of virtual memory and the unused RAM mapping.  Optionally,

the 64K segment for unused memory may be set to seg (C000, D000, E000, or

F000).  This mapping allows you to avoid conflicts with memory mapped

devices or buffers in your machine.

ZN imagename

Set the filename for the save/restore of the virtual machine.  See the ZW

and ZL commands.

ZO [name]

Echo output to the file or device given by name.  If no name is given or

the name is CON, any previous ZO command is canceled.  If the file already

exists, the output is appended to the old file.  This echoing is invisible

to the virtual DOS loaded by the ZD command.  This command allows you to

save a record of your debugging session.

Page  42

ZP seg1:ofs1 [seg2:ofs2] ... [seg20:ofs20]

Patch the current memory image at the specified seg:ofs locations for

pre-DOS 3.0 or incompatible memory mappings (such as the HP VECTRA),

and re-apply the patch at every ZD command.

ZR

Show the current contents of the video display registers.  These values are

the actual register contents, not the saved BIOS values, and include the

contents of the write-only registers.

ZS

Toggle screen swapping (screen is swapped for G, P, and X (without T)

program execution).  The new status is displayed.  When enabled, screen

swapping gives VMDEBUG its text-mode debugging screen, while the application

may do anything it desires, including custom reprogramming of the video

hardware.  This allows VMDEBUG and the application to share the video display

without worrying about video mode or over-writing needed information.

ZT [number [irqnumber]]

Set the current clock (or IRQ) divisor to number+1.  Using number=FFFF will

turn off the interrupt passed by VMDEBUG to programs.  If no numbers are

given, the current divisors are displayed.  The default clock divisor is

7D0H (2000 decimal) for a PC, and 1F4H (500 decimal) for an AT, and all IRQ

divisors default to 0 (no division).  Legal IRQ numbers are 02H through 0FH.

ZU

Toggle the final destination of IRQs not passed into VMDEBUG between passing

them to the system or internal handling within VMDEBUG (the default).

ZV [number]

Toggle extended (1 second long) video retrace mode.  This allows programs

that try to do everything during retrace time to finish.  If a number is

given, the retrace length is set to number clock ticks (18 ticks = 1 second).

ZW

Save the current state of the virtual machine in the file specified by

the ZN command.  This allows you to continue debugging later, or to have

several debugging sessions with the same initial conditions.

ZY

Toggle remapping of DMA to and from remapped ROMs during the ZZ command.

The default is not to remap DMA, so that remapped ROMs are safe from disk

reads (to ROM) but cannot be written to disk by DMA running under VMDEBUG.

ZZ [file1 [seg1 [file2 [seg2 [file3 [seg3 [file4 [seg4 [commandline]]]]]]]]]

Replace the 64K ROM segments at C000, D000, E000, and/or F000 (the default

segment is F000) with the specified files.  If no names are given, the

original ROMs are restored.  In any case, the mapping of DMA and unused

memory is not changed.  This command also performs a ZD command.

Page  43

;

Starting a line with a semicolon denotes a command line as a comment.

Anything on that line will be ignored.  It is primarily used to add comments

or remove lines from a command file (processed via the ZI command, or the

VMDEBUG.CFG file).

Example:

; any text you want              This line is ignored.

Page  44

A [[seg:]ofs]

Start the assembler.  The default location to assemble to is CS:IP after

either a ZD or an L command, and thereafter defaults to 1 above the last

location used by the Assemble command.

To avoid confusion with addressing modes, the assembler does not use the

register and hex expressions discussed in previously, but instead expects

hex constants.  There is no label or symbol capability.

If the instruction being assembled references memory without using a

a register or having an implied size, the assembler must be told whether a

byte or a word is being addressed by using "BYTE" or "BYTE PTR" or "WORD" or

"WORD PTR" in one of the operands.  If conflicting byte/word types are used

(i.e. MOV BYTE [1234],AX), an error is returned.  Note that a count of CL on

rotate/shift instructions is not a reference to a register for determining

whether the destination is a byte or a word.

The assembler recognizes all 8088 instructions, and all the 80286 real-

mode instructions, and is insensitive to upper/lower case.  If there is a

special form of an instruction (generally involving the AL or AX

accumulator), the special form will be generated when appropriate.  In

addition, the assembler will recognize the DB and DW pseudo-ops, and both

DB and DW will accept quoted strings.  Comments beginning with a semi-colon

(a ';') can be on a line by themselves, or after an instruction.

The instruction names recognied by the assembler are those used by

VMDEBUG for disassembly, and are the standard mnemonics with the exception

that CALLF, JMPF, and RETF are used for the FAR forms of CALL, JMP, and RET.

Note that the ESC instruction is not recognized for assembly.  Since it is

a NOP to VMDEBUG, it was decided to simplify the assembler by omitting it.

The list of acceptable mnemonics is:

AAA     AAD     AAM     AAS     ADC     ADD     AND     BOUND

CALL    CALLF   CBW     CLC     CLD     CLI     CMC     CMP

CMPSB   CMPSW   CS:     CWD     DAA     DAS     DEC     DIV

DS:     ENTER   ES:     HLT     IDIV    IMUL    IN      INC

INSB    INSW    INT     INTO    IRET    JA      JAE     JCXZ

JG      JGE     JMP     JMPF    JNA     JNAE    JNG     JNGE

JNO     JNS     JNZ     JO      JPE     JPO     JS      JZ

LAHF    LDS     LEA     LEAVE   LES     LOCK    LODSB   LODSW

LOOP    LOOPNZ  LOOPZ   MOV     MOVSB   MOVSW   MUL     NEG

NOP     NOT     OR      OUT     OUTSB   OUTSW   POP     POPA

POPF    PUSH    PUSHA   PUSHF   RCL     RCR     REPNZ   REPZ

RET     RETF    ROL     ROR     SAHF    SAR     SBB     SCASB

SCASW   SHL     SHR     SS:     STC     STD     STI     STOSB

STOSW   SUB     TEST    WAIT    XCHG    XLAT    XOR

The 80286 specific instructions recognized by the assembler are:

Page  45

BOUND reg,mem

ENTER stack size,level   (stack size is 16-bit immediate data,

level is 8-bit immediate data)

IMUL reg,mem/reg,immediate (sign extended)

IMUL reg,mem/reg,immediate

INSB

INSW

LEAVE

POPA

PUSH immediate (sign extended)

PUSH immediate

PUSHA

OUTSB

OUTSW

RCL mem/reg,count

RCR mem/reg,count

ROL mem/reg,count

ROR mem/reg,count

SAR mem/reg,count

SHL mem/reg,count

SHR mem/reg,count

In addition, the following mnemonics are recognized as alternate forms

for some of the above names:

Standard mnemonic  Alternate forms recognized

-----------------  --------------------------

JA                 JNBE

JAE                JNB, JNC

JG                 JNLE

JGE                JNL

JNA                JBE

JNAE               JB, JC

JNG                JLE

JNGE               JL

JNZ                JNE

JPE                JP

JPO                JNP

JZ                 JE

LOOPNZ             LOOPNE

LOOPZ              LOOPE

REPNZ              REPNE

REPZ               REP, REPE

SHL                SAL

The pseudo-ops DB and DW are also recognized, and both may contain a list of

hex values separated by spaces or commas, or quoted ASCII strings.

If the assembler is used when input is redirected via the ZI command, it

must be remembered that a blank line to terminate the A command will also

terminate the ZI command.  To stop the assembler alone, put a space on

the line.

Page  46

Examples of the A command:

A - Start the assembler at the current default address.

A 100 - Start the assembler at offset 100 in the default segment.

A IP - Start the assembler at the offset given by IP in the default

segment.

A B800:0 - Start the assembler at B800:0.  This is in the CGA video

memory.

Examples of assembler statements:

MOV AX,1234          ;move the hex constant 1234 into the AX register

XOR [-4+BX],CH       ;XOR memory with a register

CALLF [BX+SI+77-44]  ;note that the double word memory reference is

;implicit in the CALLF mnemonic

SHL BYTE [BX+DI],CL  ;the assembler must be told the size of [BX+DI]

ES:                  ;a segment over-ride

IMUL CX,[BP],2323    ;IMUL immediate, an 80286 instruction.  Note that

;the byte offset of +0 on the BP addressing mode is implicit

ADD CX,FFFC    ;add a -4 to CX (assembles to the special case ADD CX,-04)

ADD CX,-4      ;same as above

DB  D A "Hi!" d a 0  ;a CR/LF, a string, a CR/LF, and a 0 terminator

Error messages:

opcode? - The mnemonic used is not legal.

operand? - There was a mistake in an operand to the instruction.  This

covers a variety of problems, including illegal addressing modes,

mis-matched byte/word operands, missing operands, illegal hex

constants, and others.  Sometimes this will point to the 2nd (the

source) operand, even though the mistake is in the 1st (the

destination) operand, since the problem found is a mis-match between

operands.

what? - Extra operands were entered.  This could be due to a missing

semi-colon on the comment at the end of the line.

Page  47

B [[seg1:]ofs1[,eofs1][=count1] ... [seg10:]ofs10[,eofs10][=count10]]

- Set sticky breakpoints.

Using B by itself on a line will toggle the active status of sticky

breakpoints, and the new status will be displayed.  Also, setting sticky

breakpoint ranges forces sticky breakpoints to enabled, and clearing

sticky breakpoints disables them.

If sticky breakpoints are enabled and there are no sticky breakpoints,

VMDEBUG will behave as if sticky breakpoints were disabled.

When a list of ranges are given with the B command, they are checked

to see if they are all valid (up to the limit of 10 sticky breakpoints).

If they are, then the old set of sticky breakpoints is cleared and

replaced with the new set, and the saved list of stick breakpoints is

replaced as well.  As mentioned above, sticky breakpoints are enabled

by setting a list of ranges.

The general format for a breakpoint range is

[segment:]offset[,offset2][=count]

The default segment for breakpoints is the current code segment (CS), and is

not copied into the saved breakpoint list.  The default length of a range is

1 byte, unless a second offset is specified.  The default breakpoint count

is 1 (break on any reference), and the count is reset everytime a G, P, or

X command is used.

There is a limit of 10 (decimal) sticky breakpoints, and 20 (decimal)

active breakpoints of all types.  If more than 10 sticky breakpoints are

entered, only the first 10 are set.  If sticky breakpoints are disabled,

they are not counted in the active breakpoints.

The use of sticky breakpoints will slow down the interpreter.  See chapter

2 for some timing examples.

VMDEBUG responds by the message "Sticky breakpoints set." if everything

is valid.

Examples:

B 110,120 DS:SI 0:40 - Set sticky breakpoints to the range of offsets

110 to 120 (inclusive) in the current code segment (CS), the

byte at the current value of DS:SI, and the first byte of the

INT 10 vector.  Sticky breakpoints are enabled.

B 0:84=2 - Set a sticky breakpoint to the 2nd reference to the byte

at 0:84 (the 1st byte of the INT 21 vector).

B - Disable sticky breakpoints after setting them above.

Error message:

range? - An invalid range was entered.

Page  48

BL - List sticky breakpoints.

The list of sticky breakpoints is a copy of the last valid B command

to set breakpoints.  Thus, the default segment information, as well as

the values of registers used, are not available.  Also, if the command

line contained more than 10 ranges, they are all listed, even though only

the first 10 are used.

It is not possible to list the actual values of the sticky breakpoints,

as they are not stored in a form that can be reconstructed to the original

segment:offset form.

Example:

BL - lists the breakpoints from the last B command line to set

sticky breakpoints.

Error message:

No sticky breakpoints. - No sticky breakpoints have been set since

the start of VMDEBUG or the last BO command.

Page  49

BO - Sticky breakpoints OFF.

The BO command clears the sticky breakpoint addresses, disables sticky

breakpoints, and clears the list of sticky breakpoints, totally resetting

sticky breakpoints.  VMDEBUG responds with the message "Sticky breakpoints

cleared."

Example:

BO - Resets sticky breakpoints.

Page  50

C filename

or

C [seg:]ofs drive sector number

The first form of this command creates a file on disk and writes the

memory located at the current CS:IP, for a length of BX:CX bytes, to the

file.  Any previous contents of the file are lost.  This command could be

called Write, but then what would I call Where?

To avoid accidentally over-writing another file, the filename specified

by the N command (which is used by the L command) is not used, the name

must be specified along with the C command.

The file written is a straight dump of the specified memory, and thus

an .EXE file cannot be written.  If a complete .EXE file (the .EXE header,

along with the body of the file) is in memory, write the file out with

another extension (such as .DAT), and use the DOS RENAME command to change

the extension to .EXE.

The second version of the Create file command writes data to logical

disk sectors.  Thus, such things as the FAT's or the boot sector may be

modified and written back to disk.  Again, logically I would like to call

this command W, but that is being used already.

NOTE:  DO NOT USE THIS COMMAND ON A DISK THAT DOES NOT HAVE 512-BYTE

SECTORS!

All four parameters are required, or an error is returned.  If only

one parameter is given, it is assumed to be a filename and a file will be

written to disk (as noted above).

The first parameter is the address of the write.  It may contain both a

segment and offset, and if only the offset is given the current DS is used

as the segment.

The second parameter is the drive to write to, with 0 corresponding to

drive A, 1 corresponding to drive B, and so forth.

The third parameter is the starting logical sector number, which starts

at 0 for the boot sector of the disk.  The maximum available sector depends

on the type of disk used.

The fourth parameter is the number of sectors to write.  Although any

number may be specified, only 64K of data (128 (80H) sectors) is available

in the segment, and higher numbers will cause data to be re-used.

Page  51

Examples:

RCS D80 - Set CS for the starting location.

RIP 100 - Set IP for the starting location.

RBX 0 - Set high word of length.

RCX 44 - Set low word of length.

C A:\SAVE\SOMEFILE.COM - Create the file.  The specified directory must

exist.

C 100 1 0 1 - Write the data at DS:100 as the boot sector on drive B.

C DS:BX 0 5 1 - Write the data at DS:BX as the first root directory

sector on drive A.

Error messages:

filename? - a filename was not specified on the command line, or could

not be created.

Cannot create an .EXE file. - The filename given has an .EXE extension.

Try again with a different extension.

Error writing file. - There is a problem writing to the new file.  Check

for problems like disk full or write protect, and try again.

number? - a parameter is missing, or is not a number.

Error writing to disk - Not all of the sectors could be written.  This

could be caused by an empty floppy drive, an invalid drive, a non-

existant sector, or a bad sector on the disk.

Page  52

D [[seg:]startoffset [{endoffset Lleng}]] - dump memory.

The Dump command dumps memory in byte and ASCII format.  Pressing Esc

stops the dump at the end of the current line.

At the start of VMDEBUG or after a load DOS (ZD) command the default

segment is DS.  Thereafter, the default segment is the last segment used

by the Dump command.

If the starting offset is not specified, default is 100 at either the

start of VMDEBUG or after a ZD command, and thereafter is 1 above the last

address dumped (including Dump's interrupted by the ESC key).  Note that

D and DW share the address of the last dump, so that DW will pick up where

D left off, and vice versa.

The ending offset is optional, and may be specified either as an absolute

offset or as a length in the form Lleng, where leng is the number of bytes

to dump.  If an absolute offset is used and is less than the starting offset,

nothing is dumped.  If neither an ending offset nor a length is specified,

VMDEBUG dumps the default of 80 bytes.

A typical line from a Dump command looks like this:

4321:0100  64 61 74 61 20 61 6E 64-20 73 74 75 66 66 0D 0A   data and stuff..

The line starts with the address in segment:offset form, followed by the

memory displayed as byte values.  The dash in the middle simply separates

the first 8 values from the second 8 values.  Next come the ASCII characters

corresponding to the memory.  If the bytes are <20 or >7F, a period is

displayed instead of a character.  If only part of the line is needed for

the dump, the unused parts are filled with spaces.

Examples:

D - Dump 80 bytes starting at the current (or initial) default

dump address.  After the dump is over, the default dump

address will be the 1st byte beyond the end of the dump.

D 100 - Dump 80 bytes, starting at offset 100 in the current

dump segment.

D 40:23 - Dump 80 bytes, starting at the given absolute address.

When the dump is over, the default dump address will be

segment=40, offset=A3.

D 100 120 - Dump the specified range in the default dump segment.

After the dump is over, the default address will be at offset 121.

D DS:BX l2 - Dump 2 bytes, starting at the address given by DS:BX.

After the dump is over, the default address will be DS:BX+2.

D 0:24 lCX - Dump CX bytes starting at the given absolute address.

After the dump is over, the default address 0:24+CX.

Error message:

range? - an invalid parameter was entered in the address range.

Page  53

DS [number] - Dump stack.

Unlike the other Dump commands (D and DW), DS dumps ONLY the current stack.

Number is the number of words of stack to dump.  If not specified, or if

larger than 7FFF, the default of 8 words is dumped.  The dump starts at

number words up from the current SS:SP and proceeds down to the current

SS:SP, so the most recent words on the stack are at the bottom of the display,

close to the next command prompt (and will not scroll off if a large part of

the stack is dumped).

Note that there is no confusion with the D command in the following cases:

DS   - dump stack

DSS  - dump memory at the offset given by the SS register

DSSS - dump stack for the number of words given in the SS register

(note that this is parsed as DS SS.  The parsing D SS S would

say to dump memory at starting offset SS, but the ending offset

S is illegal.)

For example, if 3 words are dumped just before an IRET instruction is

executed, they will look something like this:

1234:5678 F244

1234:5676 32A1

1234:5674 2317

Where the 2317 is at the top of the stack, and is the IP to be restored by

the IRET instruction.  The 32A1 is next on the stack and is the CS to be

restored.  The F244 is last in the dump and will be the flags when restored.

Examples:

DS - Dump 8 words from the top of the stack.  Nothing is changed by this

command, so the next DS command would dump the same 8 words.

DS 3 - Dump 3 words from the top of the stack.  As in the above example,

this could be used to check the information for an IRET instruction.

Error message:

number? - the parameter specified was not a number.

Page  54

DW [[seg:]startoffset [{endoffset Lleng}]] - dump memory in word format.

All comments and examples on the D command above apply, except that the

format of the dumped memory is changed.  Memory is displayed as 16-bit words

instead of bytes and ASCII.  Note that this means reversing the byte order,

as the 8088 stores words with the low-byte first.  The length to dump is in

words.

The format of the dumped memory is:

1ABC:DEF0  1234  5678  9ABC  DEF0--0FED  CBA9  8765  4321

It starts with the address in segment:offset format, followed by the memory

displayed as word values.  The two dashes simply serve to separate the

first 4 words from the second 4 words.  If only part of the line is needed

for the dump, the unused pieces are filled with spaces.

If the dump occurs on an odd byte boundary, the dashes will not be present

in the dump.

Examples:

DW - Dump 40 words starting at the current (or initial) default

dump address in word format.  After the dump is over, the default

dump address will be the 1st byte beyond the end of the dump.

DW 100 L1 - Dump the word at offset 100 in the current dump segment.

After the dump, the new offset is 102.

Error message:

range? - an invalid parameter in the address range was entered.

Page  55

E [seg:]offset {byte1 string1} [{byte2 string2}  ... ]

- enter data into memory.

The E command enters values into memory.  Before memory is changed, all

values to be entered are checked for validity.  If any errors are found, no

values are entered into memory.

Only as many bytes are set as are specified on the command line as

data.  At least one byte must be specified.

The default segment is the current data segment (DS).  There is no

default offset.

Examples:

E 100 CD 20 - Enter the byte CD at DS:100, and the byte 20 at DS:101.

This puts an INT 20 instruction at DS:100.

E 0:52B 1 0 - This puts a 1 at location 0:52B, and a 0 at 0:52C.

E 80 "This is a test" - Enter the ASCII values of the characters in

the quoted string at location DS:80 through DS:8D.

E SS:SI AX - If the value in AX is a byte (AX < 100), enter that value

into the location pointed to by SS:SI, otherwise a number? error is

given.

E 322 'a' 1 'b' 2 'c' 3 'd' 4 - Enter a 61 (the 'a') at DS:322, a

1 at DS:323, a 62 at DS:324, a 2 at DS:325, a 63 at DS:326,

a 3 at DS:327, a 64 at DS:328, and a 4 at DS:329.

Error messages:

range?  - an invalid address to start the enter was used.

number? - there was no data on the command line to enter into memory

(NOTE:  This error points to the end of the offset, which is the

end of the command line), or the data was not a byte value (was > FF).

string? - the value to be entered is not a number and not a string, either.

Page  56

F [seg:]startoffset {endoffset Lxxxx} {byte1 string1} [{byte2 string2} ...]

- fill an area of memory with the specified data.

The F command fills an area of memory with the specified values.  All the

values in the fill command are checked for validity before any change to

memory occurs.  If any errors are found, nothing is changed.

If the fill data is shorter than the specified area of memory, the data

is repeated until the entire area is filled.  The ending offset may be

specified as either an absolute offset or a length.  If an absolute offset

is used that is less than the starting offset, the fill wraps around in the

current segment.

The default segment is the current data segment (DS).

Examples:

F 100 200 0 - Fill the area DS:100 to DS:200 with 0's.

F BX CS 23 17 - Fill the area DS:BX to DS:CS with the pattern

2317.

F 100 10F "testing " - Fill the area DS:100 to DS:10F with the

string "testing testing "

F 0:0 3FF 0 - Fill the area 0:0 through 0:3FF with 0's.  This example

wipes out the entire interrupt vector table.

F 222 l8 '"' - Fill the area starting at DS:222 with a length of 8

bytes with the value 22 (the ").

Error messages:

range?  - an invalid address or length was used.

number? - there was no data on the command line to fill into memory

(NOTE:  This error points to the end of the offset, which is the

end of the command line), or the data was not a byte value (was > FF).

string? - the value to be filled is not a number and not a string, either.

Page  57

G [=[CS:]IP] [[seg1:]ofs1[,eofs2][=cnt1]...[seg20:]ofs20[,eofs20][=cnt20]]

- start executing memory at the current or specified CS:IP, with optional

breakpoints.

The GO command starts interpretation of virtual memory.  Breakpoints

may be set at this time as command line parameters.

Program execution may be interrupted and returned to the command

prompt via the hotkey, with a default hotkey of right-shift + ESC.  The

hotkey may be changed via the ZH command.

Another hotkey is the combination left-shift + hotkey, which will stop

the interpreter without returning to the command level.  Pressing the ESC

key returns to the command line.  Any other key will resume interpretation,

with all breakpoints still active.

If screen swapping is enabled (via the ZS command), VMDEBUG will swap

screen memory and modes upon entering or leaving the G command.

When execution halts, VMDEBUG reports the number of instruction executed.

If a starting address (an IP or CS:IP) is given on the command line,

program execution starts from that point.  Two things should be noted,

however:

1) Setting either CS or IP clears out the information for the

"Where was I?" (the W) command.

2) If breakpoints are specified along with a starting CS value, the

default segment for the breakpoints will be the starting CS value,

not the current CS value.

If breakpoints are entered on the command line, they are watched as the

program executes.  The general format for a breakpoint range is

[segment:]offset[,offset2][=count]

The default segment for breakpoints is the current code segment (CS).  The

default length of a range is 1 byte, unless a second offset is specified.

If a count is not specified, a count of 1 is used (break on any reference).

Only 20 (decimal) total ranges may be watched at one time, including

sticky breakpoints.  If more are entered, the excess is ignored.  Note that

the use of breakpoints slows down the interpreter.  (See chapter 2.)

When a breakpoint has been accessed or the program has been interrupted

by the hotkey, the ranges are cleared out.  The ranges must be re-entered at

the next G, P, or X command to be watched again.

When VMDEBUG halts, the reason for the interrupt may not be obvious.

Memory reference via a XLAT or string primitive or PUSH or ... may not have

an obvious reference to a protected memory location, and a hardware

interrupt acknowledge may reference a protected address without anything

showing in the instruction stream.

Page  58

Examples:

G - Start execution at the current CS:IP.

G 105 - Start execution at the current CS:IP and stop when ANY

instruction references the byte location CS:105.

G 0:21*4 - Start execution at the current CS:IP, and stop when ANY

instruction references the byte at 0:84 (the low byte of the

INT 21 vector).  This is a good way to watch for INT 21 usage

in a program.

G DS:SI=4 - Start execution at the current CS:IP and stop at the 4th

reference the byte at DS:SI.

G 0:24,27 - Start execution at the current CS:IP and stop when ANY

instruction references ANY byte in the range 0:24 to 0:27.

Note that this example stops when any instruction looks at, changes,

or uses the INT 8 vector, including the hardware interrupt of the

clock tick.

G ES:BX,DI - Start execution at the current CS:IP and stop when

ANY instruction references ANY byte in the range ES:BX to ES:DI.

Note that DI must be larger than BX.

G 0:0,3 1234:5678 F000:0,3FFF - Start execution at the current CS:IP

and stop when ANY instruction references ANY of the bytes in any

of the ranges specified.

G=7654 - Set IP to 7654 and start execution.  Remember that setting IP

will erase the information for the "Where was I?" (the W) command.

G=1234:5678 6666 - Set CS to 1234, IP to 5678, and set a breakpoint at

1234:6666.  Note 2 things:  1) the default segment for the

breakpoint is the specified CS, and 2) the information for the

"Where was I?" (the W) commnad is erased.

Error message:

range?  -  An invalid range was entered, or the CS or IP specified was

invalid.

Page  59

H [command] - Help!

The Help command displays a list of all commands and their functions.

If a specific command is asked for, only that command is displayed.  If more

than one command matches the requested command, all matching commands are

displayed.

If the help list runs over 1 page, it pauses and asks for a keypress to

continue, or ESC to quit.

A synonym for H is ?, and works exactly like H.

Examples:

H - Get all help available.

?D - Get help on the D, DS, and DW commands.

HZ - Get help on all commands that begin with Z.

HEX - Show the examples.

Error message:

No help available on that subject. - The requested command was not found.

Page  60

I portnumber - Get a value from a byte I/O port.

The value of the specified I/O port is displayed.  If PC-XT clone status

is not enabled (via the ZC or ZD commands), the value returned is FF (nothing

there to respond).

Examples:

I 21 - Get and display the value from port 21.  This example shows

the interrupt mask register of the 8259 interrupt controller.

Note that, unless PC-XT clone status is enabled, the displayed

value will be FF.

I BP - Get the value of the port addressed by BP.

Error message:

number? - the parameter specified was not a number.

Page  61

L - load a file.

or

L [seg:]ofs drive sector numberofsectors

In the first form of the L command, the file is loaded at the current

CS:IP (for .COM or data files) or after the PSP (for .EXE files).  If DOS

has not been loaded, an .EXE file is loaded where the PSP would be if DOS

were loaded.  The file name is specified by use of the N command.

The file length is returned in the CX register if less than or equal to

FFFF and returned in BX:CX if greater than FFFF.

Loading a file clears pending interrupts and resets the "where was I?"

(the W command) information.

Also, note that in DOS 3.0 or higher, the name of any program that is

Loaded is not put into the environment space.  Some programs, mostly copy

protection schemes, check the name of the program in the environment to

make sure of who is executing.  If you run across a program like this, you

may either set the name in the environment space yourself (use DW 2C L1

immediately after a ZD command to find the segment address of the

environment, use the D command to scan its contents, and the E command to

set the program name), or load the program from the virtual DOS prompt.

The second version of the Load command reads logical disk sectors to

memory, so that things like the boot sector or the FAT's may be examined

and modified.

NOTE:  DO NOT USE THIS COMMAND ON A DISK THAT DOES NOT HAVE 512-BYTE

SECTORS!

All four parameters are required, or an error is returned.  If no

parameters are given, VMDEBUG will try to load the file specified by the

last N command.

The first parameter is the address of the read.  It may contain both a

segment and offset, and if only the offset is given the current DS is used

as the segment.

The second parameter is the drive to read from, with 0 corresponding to

drive A, 1 corresponding to drive B, and so forth.

The third parameter is the starting logical sector number, which starts

at 0 for the boot sector of the disk.  The maximum available sector depends

on the type of disk used.

The fourth parameter is the number of sectors to read.  Although any

number may be specified, only 64K of data (128 (80H) sectors) is available

in the segment, and higher numbers will cause data to be over-written.

Page  62

Examples:

N filename.com - The program to be loaded is FILENAME.COM

L - The actual load is performed here.

L 100 1 0 1 - Read the boot sector on drive B to DS:100.

L DS:BX 0 5 1 - Read the first root directory sector on drive A to DS:BX.

Error messages:

filename? - The filename has not been specified or does not exist.

invalid .EXE file. - The file was given an .EXE extension but does not

have a valid .EXE signature.

not enough memory. - For a .COM or data file, the file will not fit into

memory.  For a .EXE file, the minimum required memory is more than is

available.

number? - a parameter is missing, or is not a number.

Error reading disk - Not all of the sectors could be read.  This could

be caused by an empty floppy drive, an invalid drive, a non-existant

sector, or a bad sector on the disk.

Page  63

M expression - Evaluate and display the results of a Mathematical expresion.

The M command evaluates an expression according to the rules described

above for numeric arguments and displays the result.  Remember that spaces

separate expressions, and that ONLY the first expression is evaluated.

Examples:

M ax - Evaluate the expression "ax".  In this case, it is simply the value

of the AX register.

M SI^~DI - Evaluate the given expressions.  The precedence of operators

says that NOT DI will be done first, then the XOR.

M AX*AB+DX*CD - Multiply the registers by the given constants, and add the

results.  Remember that multiplication has precedence over addition.

M 3A3A<3 - Rotate the hex word 3A3A left 3 bits.  Note that since both

the low and high bytes are the same, each byte (low and high) of the

answer is the same as if a 3 bit rotate had been used on the byte 3A.

Error message:

number? - The argument was missing, or could not be evaluated as a number.

Page  64

N filename - specify a file name.

The N command allows a file name to be specified for loading.  Paths and

drive names are accepted.  When entered, the name is not checked for validity

or for existence of the file.

The specified filename is not forgotten after use, and only needs to be

reset when a different file is required.

The filename is not entered into the current PSP.  If you want to

specify a "command line" to a program, use the ZD command.  Also, for DOS 3.0

or higher, the specified name is not entered into the environment space.

Example:

N bigprog.exe - specify the file name to be loaded.

Error message:

filename? - no filename was specified.

Page  65

O portnumber value - Output a value to a byte port.

The O command sends the specified byte value to the specified port.  If

the value is not a byte (is > FF), an error is returned.  The data to be

output to the port is processed as described in chapter 4, I/O mapping and

hardware interrupts.  If PC-XT clone status is not enabled (by either the ZC

or ZD commands), this command has no effect.

This command can be used to turn on the floppy disk motor if interrupting

a floppy I/O operation turns off the motor.  To turn on the motor, use the

following values:

port   value  drive

----   -----  -----

3F2    1C     A

3F2    2D     B

3F2    4E     C

3F2    8F     D

Multiple drives may be turned on by ORing the above values.  However, it may

be better to set the byte at 40:3F to 0, telling BIOS that the motor is not

running.  It is possible to start the motor and have the real BIOS turn it

off just before execution of a subsequent G, P, or X command, leaving a

program in the floppy disk BIOS with the motor off.

Note that on the AT, the floppy BIOS forces a minimum motor start time

that is quite long when executed under VMDEBUG, so that the risk noted above

becomes more acceptable.  The impatient programmer could bypass this motor

start time by setting the low nibble of the byte at 40:3F to reflect a motor

on status, and turn on the drive via an O command before the BIOS checks for

a running disk motor.

Examples:

O 61 80 - Try to turn off the keyboard.  As discussed earlier, the

keyboard will not be turned off by an out from VMDEBUG.

O BP SI - If the value of the SI register is a byte (SI < 100), send

the byte to the port referenced by the BP register; otherwise return

a number? error.

O 3F2 5E - Turn on floppy drives A and C.

Error message:

number? - One of the two parameters is invalid or missing.  If missing,

the error will point to the end of the previous parameter.  Also,

the value might have been > FF.

Page  66

P [=[CS:]IP] [[seg1:]ofs1[,eofs1][=cnt1]...[seg19:]ofs19[,eofs19][=cnt19]]

- Proceed.

Proceed is a variation on the G command.  Proceed automatically sets

a breakpoint after the next instruction to be executed and is useful for

"stepping over" CALLs, INTs, repeated string instructions, and loops.

All comments above for the G command apply, except that 1 less breakpoint

is available to the user, because the breakpoint is set by VMDEBUG.

When an initial IP or CS:IP is specified for the P command, the

breakpoint is set on the instruction AFTER the instruction at the new CS:IP.

As with the G command, when either IP or CS is changed, the information for

the "Where was I?" (the W) command is lost, and breakpoints will use the new

CS for the default segment.

If you try to proceed through a data area that consists only of prefix

codes, VMDEBUG will take a long time to find the instruction.  Press Esc

while VMDEBUG is trying to find the address of the next instruction to

return to the command prompt.

Examples:

P 0:40 - Execute the program until either the next instruction or the byte

at 0:40 is referenced.

P 40:17=2 - Execute the program until either the next instruction or the

byte at 40:17 is accessed twice.

P=1234:5678 - Execute the program from 1234:5678, and set a breakpoint on

the first instruction after 1234:5678.

Error message:

range?  -  An invalid range was entered, or the IP or CS was invalid.

Page  67

Q - Quit.

Quit the VMDEBUG program.

Example:

Q - Quit the program and return to DOS.

Page  68

R [name [value]] - Display and modify registers.

The Register command has two forms, to allow inspection and modification

of the 8088 registers kept by VMDEBUG.

If a register name is not given, all registers, flags, and the current

instruction are displayed.

If a name is given, it must be the name of one of the 16-bit registers

(AX, BX, CX, DX, SI, DI, BP, SP, CS, DS, ES, SS, IP or PC). If a new value

was specified on the command line, the register is set to the value.  If

only the register name was given, the current value of the register and a

":" prompt to enter a new value are displayed.  Press return on a blank line

to retain the current value.  For either method, entering a value for either

CS or IP (even if the same as the current value) will cause VMDEBUG to forget

the "Where was I?" (the W command) information.

PC (program counter) is recognized as another name for IP (instruction

pointer).

If F is given for register name, VMDEBUG will use the flags register.

If any values are given on the command line, they will be used to set the

flags, else the current flags will be displayed and you will be prompted

for a list of flags.  A blank list will keep the current flags, and if

multiple values are specified for the same flag, the last (rightmost) value

will be used.  If an invalid value for a flag is specified, an error is

returned, although other values on the command line will be processed up

to the error.  The flag names and the abbreviations used are:

Overflow, Direction, Interrupt, Trap, Sign, Zero, Aux Carry, Parity, Carry

with ON abbreviations of: OV, DN, EI, TR, NG, ZR, AC, PE, CY

and OFF abbreviations of: NV, UP, DI, NT, PL, NZ, NA, PO, NC

For more on the flag abbreviations, see the discussion at the start of

chapter 6.

Examples:

R - Display all registers, and the instruction at the current CS:IP.

R AX   - Display the current value of AX in the form AX xxxx, and on the

next line, give a prompt of ':' and wait for a value to be entered.  If

a blank or invalid value was entered, do not change the current value.

R CX 1 - Set the value of the CX register to 1.

R ES AX^DI - Set the ES register to the value of the AX register XOR'ed

with the DI register.

R F NV PL CY - Set the overflow flag to OFF, the sign flag to OFF (plus),

and the carry flag to ON.

Error messages:

register? - The register name given was not valid.

number? - The value given for the register was not a number.

flag? - The flag value specified was not valid.

Page  69

S [seg:]ofs {endoffset Lleng} {byte1 string1} [{byte2 string2} ...]

- Search memory for pattern.

All the values of the search command are checked for validity before the

search occurs.  If any errors are found, the search does not occur.  When

a match is found, the address of the matching pattern is displayed in

segment:offset format.  If more than one match occurs, all addresses are

displayed.  Press ESC to terminate the search after the next address is

displayed.  If no matches are found, pressing ESC does nothing.

The ending offset may be specified as either an absolute offset or as a

length.  If an absolute offset is used that is less than the starting offset,

the search will wrap around in the current segment.

The default segment is the current data segment (DS).

Examples:

S 100 200 0 - Search the area DS:100 to DS:200 for 0's.

S 100 LCX "testing " - Search the area at DS:100 for CX bytes for the

string "testing "

S CS:IP LCX 0D 0A - Search the specified area of memory for CR/LF's.

If a program was just loaded, this is a good way to search for text.

S 3201:0 ffff '"' - Search an entire segment for double quotes.

Error messages:

range?  - an invalid address or length was used.

number? - there was no data on the command line to search for

(NOTE:  This error points to the end of the offset, which is the

end of the command line), or the data was not a byte value (was > FF).

string? - the value to be searched for is not a number and not a string,

either.

Page  70

T [=[CS:]IP] [num] - Trace program.

Trace a program for 1 step, or optionally for num steps (the maximum is

7FFFFFFF).  Interrupts are not turned on, although pending interrupts are

processed.  Break on I/O, sticky breakpoints, and breakpoints are not used

in the Trace command.

When a hardware interrupt is processed, a message of

"Hardware interrupt #xx" is displayed.

If a starting address (an IP or CS:IP) is given on the command line, the

trace starts from that point.  It should be noted, however, that setting

either CS or IP clears out the information for the "Where was I?" (the W)

command.

Each instruction traced displays registers and instruction BEFORE

execution.  This is the opposite of DEBUG, which displays the registers

and instruction AFTER the instruction that was traced.

To stop Trace before the specified number of instructions, use the ESC

key.  If less than the requested number of instructions are executed, because

of either a user break (ESC key) or encountering an invalid instruction, the

number of instructions executed is displayed.

Examples:

T - Trace program execution for 1 instruction.

T 34 - Trace program execution for 34 (hex) instructions.

T DX - Trace program for the value of DX instructions.

T=1234 - Trace program execution, starting with IP=1234, for 1

instruction.  The information for the "Where was I?" (the W)

command is lost.

Error messages:

number? - The argument was not a number.

range? - The value specified for either IP or CS was invalid.

Page  71

U [[seg:]startoffset [{endoffset Lleng}]] - Unassemble memory.

The default address for unassembly is CS:IP at either the start of the

program, after a ZD command, or after a L command, and thereafter defaults

to 1 byte above the last unassembly.

The ending offset is optional and may be specified as either an offset

or as a length.  If an absolute address is used and is less than the

starting offset, the unassembly will wrap around in the current segment.

If the ending offset or a length is not specified, the default is a length

of 20.

Note that VMDEBUG may unassemble a few more bytes than specified,

because VMDEBUG unassembles all bytes of an instruction.  Then, the

default unassembly address is set to the 1st byte after the end of the

last instruction unassembled.

Press ESC to stop the unassembly at the end of the current instruction.

Examples:

U - Unassmeble 20 bytes starting at the current default address.

U 100 - Unassemble 20 bytes, starting at offset 100 in the current

default segment.

U 740:233 - Unassemble 20 bytes, starting at the given absolute

address.

U 100 120 - Unassemble the specified range in the current default

segment.

U 1230:108 l4 - Unassemble 4 bytes starting at the given absolute

address.

U CS:IP lCX - Unassemble CX bytes starting at the current CS:IP.

Error message:

range? - An invalid number was specified in the range.

Page  72

V - view the virtual screen.

If screen swapping is enabled, the view command will swap the VMDEBUG

debugging screen with the virtual screen.  Pressing any key will swap the

screens back.

Example:

V - View the virtual screen.  Pressing any key will return you to the

debugging screen.

Error message:

Virtual screen not available. - Screen swapping is not enabled.

Page  73

W [num] - Where was I?

VMDEBUG remembers the addresses of the last 7FF (2047 decimal)

instructions executed and unassembles these instructions in the order

executed.  Note that self-modifying code can cause the instructions at

the saved addresses to differ from what was actually executed.  Displaying

the instructions does not change anything, so another W command will

display the same information.

The default number of instructions to recall is 10 (16 decimal), and

is always limited to the number of instructions executed by the G, P, T,

and X commands since the last L, ZL, or ZD command, or since CS or IP were

manually changed; or 7FF, whichever is lower.  Thus, if 22 instructions

have been executed since the last ZD command, W FFFF displays only 22

instructions.  If 8000 instructions have been executed, W FFFF only

displays the last 7FF instructions.

Examples:

W - Display the last 10 instructions executed.  If fewer instructions

were executed since the last reset, display as many as possible.

W 200 - Display the last 200 instructions executed.

Error messages:

number? - The parameter given was not a number.

No instructions have been executed. - There is nothing to display.

Page  74

X [=[CS:]IP] [T] [Qnumber] [N{insttype "instruction string"}]

[[seg1:]ofs1[,endofs1][=count1] ... [seg20:]ofs20[,endofs20][=count20]]

- eXecute program.

The program is interpreted starting at the current or specified CS:IP

with hardware interrupts on.  If screen swapping is enabled, screens are

swapped at the beginning and end of the command.  Breakpoint ranges may be

specified as for the G command.  Use the hotkey to stop the interpreter.

When done, the number of instructions executed is displayed.

The X command is slower than the G and P commands, because of the extra

processing done for the additional breakpoint types.

If a starting address (an IP or CS:IP) is given on the command line,

program execution starts from that point.  Two things should be noted,

however:

1) Setting either CS or IP clears out the information for the

"Where was I?" (the W) command.

2) If breakpoints are specified along with a starting CS value, the

default segment for the breakpoints will be the starting CS value,

not the current CS value.

Three additional parameters on the command line must be specified before

any breakpoints.  Any, all, or none may be present.  If the same parameter

is specified more than once, VMDEBUG will use the last copy on the command

line.

The first parameter is T, execute in Trace mode.  The use of this option

changes the nature of the eXecute command.  Instead of executing with

interrupts watched, like the G (Go) command, it executes without watching

interrupts, like the T (trace) command.  As a result, the ESC key must be

used to return to the command line, instead of the hotkey.  Also, things like

disk I/O will not execute properly, as they will not receive the expected

interrupts from the hardware.  Pending interrupts are executed in Trace mode.

When in trace mode, VMDEBUG displays the registers and instruction about to

be executed at each step.  Also, the user is informed of hardware interrupts.

This is very useful when a trap is done on the address of a hardware interrupt

vector, which is not an obvious occurrence when examining the location at

which VMDEBUG stopped.  The screen is not swapped for execute in trace mode,

even if screen swapping is enabled.

The difference between eXecute in Trace mode and the Trace command is that

breakpoints are active.

The next parameter is Qnumber, where number specifies how many

instructions to execute before causing a breakpoint, and may be as large

as 7FFFFFFF.  Larger numbers are accepted, but execution stops after 1

instruction.  Execution may be stopped before the requested number of

instructions if a breakpoint or an invalid instruction is encountered.

If an error occurs after a known number of instructions, the user may stop

at the desired point before the error.

Page  75

The third parameter is N, and is accompanied by either insttype, which

is the type of instruction on which to stop, or "instruction string",

which stops VMDEBUG when the string generated by instruction decode matches

the entered string to the length of the entered string.  Either type of

break occurs before execution of the instruction.

When instruction type is used, the instruction types are the mnemonics

listed in chapter 5.  Note that the use of segment prefixes for the

instruction name causes an error.  If a break is set to a prefix instruction,

such as LOCK, REPZ, and REPNZ, VMDEBUG will not break when the prefix is

encountered, as prefixes are not executed, merely unassembled.

Also, VMDEBUG cannot make distinctions on the instruction arguments.  If

a break is set on a MOV instruction, any MOV instruction, such as

MOV AX,1 or

MOV DS,WORD PTR [1234] or

MOV BX,BP or...

will cause the break.

This break type is useful for instructions that have unique uses, such

as IN, OUT, IRET, INT, XLAT, LDS, etc.

When an instruction string is used, VMDEBUG will generate strings

during instruction decode, and compare the given instruction string to

the generated instruction string, up to the length of the entered

string.  This gives break on instruction type the ability to distinguish

instruction arguments, and to break on both segment override prefixes

and prefix instructions.

When entered, the string is changed to upper case, and the spacing

between the opcode and the first argument is modified to what VMDEBUG

generates.  However, no checking is done on the string for a legal

instruction type, that is left to the user.

The use of breakpoint on instruction string will slow down VMDEBUG by

about half, due to the necessity of generating and testing strings for

every instruction.

Examples:

X Q7FF - Execute the next 7FF instructions, and then stop.  Note that this

just fills the "where was I?" (the W command) queue.

X T 40:3F 40:1C 0:20,3F - Execute in trace mode, while watching the floppy

motor status byte (40:3F), the keyboard tail pointer (low byte, 40:1C),

and the interrupt pointers for the PC-XT hardware interrupts (0:20,3F).

X Niret - Execute the program until a IRET instruction is encountered.

The break occurs before the execution of the instruction.

X N"MOV AX,[" - Execute the program until an instruction moves memory

to the AX register.

X N'AD' - Execute the program until the next ADC or ADD instruction.

X=1234 - Set IP to 1234, and execute memory.  Setting IP will lose the

information for the "Where was I?" (the W) command.

Error messages:

number? - The value specified with the Q parameter was not a number.

string? - The string specified with the N parameter was not an

instruction name, or had unbalanced quotes.

range? - An invalid range was specified, or the IP or CS was invalid.

Page  76

Z7

The Z7 command toggles the WAIT instruction between acting as a NOP to

VMDEBUG (the default), and acting as a breakpoint.

The WAIT instruction tells the CPU to wait for the 80x87 chip to finish

before proceeding to the next instruction.  However, as there is no 80x87

available, it will never be ready.  Thus, what should be done when a WAIT

is executed is not obvious.

The default action is to treat WAIT as a NOP, like the other 80x87

instructions (and like a PC does when a 80x87 is not installed), so that

VMDEBUG simply continues.  However, VMDEBUG can also be made to execute a

breakpoint on a WAIT instruction, so that the user can see when a program is

expecting a return from an 80x87 instruction.

When the Z7 command is used, the user is informed of the new status of

the toggle.

Example:

Z7          Toggle the WAIT instruction between a NOP and a breakpoint.

Page  77

ZB [port1] ... [port20] - Toggle Break on I/O status.

Break on I/O watches for the execution of IN, OUT, INSB, INSW, OUTSB,

or OUTSW instructions, and stops VMDEBUG if the port has a breakpoint set

on it.

Breakpoints on ports may be set in one of two ways.  The first is that

all ports will be watched, in which case the ZB command acts as a toggle

to turn Break on I/O on or off.

The second is that up to 20 (decimal) ports may be specified on the

command line, in which case breakpoints are set on those ports only, and

Break on I/O is enabled.  To turn off Break on I/O in this case, use ZB

with no parameters.

Break on I/O status is disabled at the start of VMDEBUG and is changed

only by the ZB command, although break on I/O is ignored by the T command.

When the status is changed, the new status is displayed.

Examples:

ZB - Toggle the Break on I/O status.  The new status is displayed.

ZB 123 - Break on I/O to port 123 only.  The new status is displayed.

Error message:

number? - The specified value was not a number.

Page  78

ZC - Toggle PC-XT clone status.

PC-XT clone status allows access to the I/O ports of a PC or clone.  The

default at the start of VMDEBUG is clone status disabled.  If clone status

is disabled, all OUT instructions dump their values in the "bit bucket", and

all IN instructions return a value of FF (or FFFF for word ports),

indicating nothing at that port.  If clone status is enabled, access to the

I/O ports is allowed, with the processing described in chapter 4.

Software emulation of the 8259 interrupt controller is not enabled by this

command.  Use the ZD command to turn on the "8259".

PC-XT clone status is automatically enabled by the ZD command.  When

changed via the ZC command, the new status is displayed.

Example:

ZC - Toggle PC-XT clone status.

Page  79

ZD [commandline] - load DOS, and set the PSP fields from the optional

command line.

VMDEBUG requires that there be 64K of free memory beyond the location of

the PSP (program segment prefix) in virtual memory to load a virtual copy

of DOS.  This allows the program being debugged to have sufficient space to

execute.

If less memory is available, VMDEBUG reports an error.  VMDEBUG also

copies all possible memory to the virtual memory area, and sets the

registers and current instruction so that virtual memory is set to reboot

itself as best it can.  This allows virtual DOS to have a different

organization (RAM disks, resident programs, etc) than the real DOS, so it

will fit into virtual memory.

This command always performs the following functions:

* Clears pending interrupts.

* Resets the information for the W command (where was I?)

* Enables software emulation of the 8259 interrupt controller.  The "8259"

is not enabled by the ZC command, as it could turn on interrupts before

the software is ready for them.

* Resets the default dump address to DS:100 and the default unassemble

address to CS:100

* Copies low memory, starting at 0000:0000, to the corresponding

locations in virtual memory up to pspaddress:100 (and beyond usually,

as memory is moved in 32K chunks) OR until virtual memory is full,

whichever is less.  Note that any program in virtual memory is thus

destroyed.

* Adjusts the memory size at 40:10 and 40:13-14 in the virtual BIOS data

area to reflect the size of virtual memory.

* Copies the current contents of screen memory and the screen display

mode to internal storage.  Although the values are always saved,

they are only used if screen swapping is enabled.

* Set PC-XT clone status, to allow access to the I/O ports.

* If enough virtual memory is available, perform the following:

* Adjust the DOS allocation chain at pspaddress-1:0 and the memory

size at pspaddress:2-3 to show the size of virtual memory.

* Adjust the Disk Parameter Table pointed to by Interrupt 1E in

virtual memory so that the head settling time is 0 milliseconds,

and the motor start time is 0 seconds.  These are reasonable

values, given the speed of VMDEBUG.

* If DOS has been patched (pre-DOS 3.0), re-apply the patch.

* Clear out the command line length and the FCB's in the PSP (program

segment prefix), and if a command line has been specified, place

it and its length in the PSP, and parse the parameters into the

FCB's.  Note that, unlike DOS, the AX register is not set to reflect

the validity of drive names entered on the command line.  This may

cause programs like FORMAT.COM to behave strangely.

* Put a CD20 (INT 20, return to DOS) at virtual pspaddress:100,

set CS=DS=SS=ES=pspaddress, set IP to 100, set SP to FFFE, put

zeros at FFFE, FFFF, and set the interrupt flag.  This simulates

a trivial .COM program that quits to DOS.

Page  80

* If virtual memory is too small, perform the following:

* Report an error of "Not enough memory!"

* Set the code segment to the top of memory minus 1 paragraph, and

the instruction pointer to 0.  At this location, put a CD19 (INT 19,

boot system) instruction.

* Set the stack segment to the top of memory minus 200 paragraphs,

and the stack pointer to 1FFE.  This stack will be used in the

system boot.

DOS may be loaded any number of times, whenever a "fresh start" is

desired.

Loading a virtual copy of DOS will change the SP, IP, CS, DS, ES, SS,

and flag registers, but will leave the AX, BX, CX, DX, BP, SI, and DI

registers alone.

The ZD command gives a command line to the program being debugged, rather

than the N command (as in DEBUG).  Unlike DEBUG, there is no "fixed" PSP

for programs to use, so anything could be in the memory occupied by the PSP

when the N command is used.  Setting up the PSP fields would destroy that

memory.  The ZD command, on the other hand, sets up a PSP for programs to

use, so that formatting the PSP fields is safe.

Also, note that in DOS 3.0 or higher, the name of any program that is

Loaded is not put into the environment space.  Some programs, mostly copy

protection schemes, check the name of the program in the environment to

make sure of who is executing.  If you run across a program like this, you

may either set the name in the environment space yourself (use DW 2C L1

immediately after a ZD command to find the segment address of the

environment, use the D command to scan its contents, and the E command to

set the program name), or load the program from the virtual DOS prompt.

The AT BIOS enforces a minimum disk motor start time despite the values

in the Disk Parameter Table, so that doing the first access to a floppy

will take a long time.  The impatient programmer might want to try setting

the value at 40:3F to reflect a running motor and turn the motor on via

the O command as discussed above.

Examples:

ZD - Load DOS as described above.

ZD alpha.dat - Load DOS, and give the program to be loaded a command line

of alpha.dat.  This is a valid filename, so the first FCB at offset 5C

in the PSP is formatted.

Error message:

Not enough memory! - There is not enough memory to set up a virtual copy

of DOS.  Try again with a smaller RAM disk, fewer resident programs,

etc.  If this does not work, you need more memory.

Page  81

ZE [E [num_seq num_crt num_attrib num_graphics]]

VMDEBUG notes the presence of an EGA or VGA card and uses the appropriate

version of screen swapping automatically.  However, some cards have a mode

in which CGA and MDA modes are emulated at the hardware level instead of in

software.  This mode, however, is not detectable to VMDEBUG, which tries to

use EGA/VGA screen swapping in those modes and sends the video into an

unknown state.  The ZE command toggles between EGA or VGA and CGA/MDA screen

swapping, so VMDEBUG may be run under hardware emulation modes and programs

that swap modes may be run under VMDEBUG.

NOTE:  If EGA/VGA screen swapping is not desired, the ZE command should

be used to turn off EGA/VGA mode BEFORE the ZD command is used!  The ZD

command saves EGA/VGA information, and must use mode swapping to do so.

This mode swapping leaves the screen in an undefined state.

Using the ZE command will force a reset of the video parameters but will

not clear the screen.  If the EGA or VGA card is in the wrong mode, this

could result in a scrambled screen.  If this occurs, use ZE to switch back

to the correct mode, and in extreme cases the use of the V command to force

a screen swap may be necessary.

When EGA or VGA versus CGA/MDA modes are toggled, the results are

reflected in the ZR command, which dumps the current status of the video

registers.

The use of the ZE command does not switch the type of display adapter.

Thus, a EGA/VGA display in Color mode will switch to a CGA display, and a

EGA/VGA in Monochrome mode will switch to an MDA display, and vice versa.

If an EGA or VGA card is not installed, this command will do nothing,

including not resetting the video parameters.

When a VGA is installed, it may seem natural to want to go from VGA to

EGA to CGA/MDA, and then back to VGA.  However, most VGA boards are not EGA

compatible enough to handle this, and trying to address them as an EGA

leaves the screen in an undefined state.

The ZE command can also be given parameters to inform VMDEBUG of the

presence of an extended EGA or VGA adapter, and how to deal with it.

The first parameter is the letter E, which toggles VMDEBUG's use of the

extended ports (at 3CBh, 3CDh, and 3BFh) when swapping screens.

The second through fifth parameters must all be entered if any are to

be used (and the first parameter, E, must be entered as well).  They tell

VMDEBUG the maximum register to swap for each chip during screen swapping.

The normal register count (in hex) for each set (and the minimum that will

be accepted as valid by VMDEBUG, as well) is:

Sequencer:  4 (registers 00 - 04)

CRTC:      18 (registers 00 - 18)

Attribute: 13 (registers 00 - 13, 00 - 14 for VGA)

Graphics:   8 (registers 00 - 08)

Page  82

The maximum register count, in hex, for each set is:

Sequencer:  9 (registers 00 - 09)

CRTC:      27 (registers 00 - 27)

Attribute: 31 (registers 00 - 31)

Graphics:  13 (registers 00 - 13)

The standard swapping count will work for EGA cards, and the VGA cards

from Paradise, Compaq, and IBM.  The VGA card by Tecmar (which is OEM'ed

by several vendors--check out the copyright at C000:0000 for the actual

board maker, or check for the Tseng labs chip set) will work with the

standard swap count for most swapping, but some hi-res modes (modes 10h

and 12h, for example) need to use extended regsiters.  For the Tecmar,

use the command:

ZE E 7 25 14 8

If your VGA board requires special screen swapping, check out function 1Ch,

subfunctions 1 and 2 for information on what registers need to be swapped.

This requires you to dive into your ROM BIOS, or talk to your board

manufacturer.

Although extended ports and additional registers will help with non-

standard boards, they are not a general solution.  VGA turns out to be

very sensitive to the programming order, and some vendors boards use

"lock registers" to permit/deny access to their extended registers.

Since VMDEBUG can't know the details of your board, it may or may not

be able to do a proper screen swap of your board.

If a non-standard swapping count is desired but not extended ports,

first set the port counts and then use the ZE E command to toggle

extended ports off.

Examples:

ZE - If an EGA is installed, toggle between EGA and CGA/MDA screen

swapping, else do nothing.

ZE E - Leave EGA/VGA swapping alone, but toggle the use of extended

ports (3CB, 3CD, 3BF).

ZE E 4 18 14 9 - Toggle use of the extened ports, and set the register

swap counts to normal for a VGA.

Error messages:

EGA/VGA not present. - There is no EGA or VGA card present in the system,

and nothing to toggle.

number? - a parameter is missing, or is not a number.

Page  83

ZF [filename]

It is often desireable to look at some other file, such as the

source code or a map file, while debugging.  The ZF command allows

you to browse through a text file without leaving VMDEBUG and losing

your place in debugging.

The filename is optional, and if not specified the previous file will

be re-opened to the previous file position.  The filename may contain

a drivename and pathname.

When the ZF command is used, the screen is cleared and used to

display a header line with the available commands, and the file itself.

When leaving the ZF command, the original screen is NOT restored.

When in the ZF command, the file is displayed a page at a time (23

lines).  Long lines are wrapped to the next line, but the last line

is truncated so that only 23 lines are displayed at any time.  The

remainder of a truncated line is the start of the next page.

Several subcommands are available when browsing a file.  They are:

F - Find text.  You will be prompted for a text string, and if

it is not blank, the file will be searched for the text

starting at the current page and continuing to the end.  If

the text is found, the page with the first occurance is

displayed, else the current page is re-displayed and a

"text not found" message is given.

A - Again.  If the last search string was not blank, go to the

next page and try to find the text again, as described above.

Note that leaving and re-entering the ZF command clears the

search string.

The following keys on the cursor pad allow you to move through the file:

Home - Go to the start of the file.

End - Go to the end of the file.

PgUp or up-arrow - Go up to the previous page.

PgDn or down-arrow - Go to the next page.

To exit the ZF command, press the Esc key.

When paging up in a file, you may notice that it does not return to

exactly the same place it found when paging down.  This is a result

of truncating long lines at the end of the page, and should not be

a problem.

Page  84

Examples:

ZF MYFILE.LST    Look through the .LST file associated with the file

being debugged.

ZF C:\C\AB.C     Check out the source file of the current program.

ZF               Look at the last program again.  The file is

re-entered at the same page it was left.

Error message:

No file - The drive, path, or file could not be found, or there was a

problem opening the file.  Or, no name was given and there

was no previous name.

Page  85

ZG [filespec]

It is often useful to see what files are available while in VMDEBUG.

The ZG command will display a list of any or all files in any directory

on any disk.  The user is informed if no files are available.

The filespec parameter is optional, and if given may contain a drivename,

and/or a pathname, and/or a wildcard filename.  If only a drivename and/or

a pathname is given, all files in the specified directory will be shown.

The directory list includes all hidden, system, and read-only files, as

well as all subdirectories.

Pressing Esc at any time will stop the directory list and return to the

command prompt.

Examples:

ZG             Show all files in the current directory.

ZG D:          Show all files in the root directory of the D: drive.

ZG *.COM       Show all .COM files in the current directory.

ZG \FILES\*.C  Show all files with a .C extension in the \FILES

directory on the current disk.

Error message:

No files - The drive and/or path could not be found, or there were no

matching files.

Page  86

ZH byte - Set the hotkey.

Set the hotkey (used to return from the executing virtual program to

VMDEBUG) to the combination Right-Shift plus (user selected key).  The

byte given by the user is the scancode of the desired hotkey.

The Right-Shift is set within VMDEBUG and may not be changed, but the

hotkey itself may be selected.  Some suggestions are:

scancode

(in hex)   key

--------   ----------

01         ESC         (default hotkey at start of VMDEBUG)

0E         Back Space

1C         Return

39         Space Bar

The scancode must be less than 80, indicating a key make rather than

a key break.  A value of 36 is not allowed, as that would be

Right-Shift and Right-Shift.

A second hotkey is also established as Left-Shift and (same user

selectable key).  This is used to stop the interpreter without swapping

screens (if enabled via the ZS command) or returning to the VMDEBUG

command level, and waits for a keypress.  When stopped, interrupts are

NOT trapped to be sent to the interpreter.

Pressing ESC will return to the VMDEBUG command level, and any other key

will continue from where the interpretation stopped.

Examples:

ZH E - Set hotkey combination to Right-Shift and Back Space

ZH 1 - Set hotkey combination to Right-Shift and Esc (The default at

program start).

Error message:

number? - The value given by the user was not a number, was greater

than 7F, or was equal to 36.

Page  87

ZI [name] - Redirect the input to come from the specified file or device.

The input is redirected to come from the specified file or device, or from

CON (the keyboard) if no name is specified.  This allows you to use files as

command macros for executing a series of commands in batch mode.

Note that some input is taken directly from the keyboard, and thus is not

affected by the ZI command.  This input is the single character input used

to test for ESC during command execution (such as during the U, D, H, T, or

other commands) and the hotkey used to interrupt an executing program.

When input has been redirected, pressing ESC still aborts such commands

as U, D, H, T, etc, skipping down to the next command in the file.  If ESC

is pressed during a command that cannot be interrupted by ESC (such as R, B,

E, etc.) input is redirected to CON (the keyboard).

When input has been redirected, either a EOF from the file or device or a

blank input line redirects input to CON (the keyboard).  Thus, you do not

need a ZI command to end a command file.

Redirection is accomplished within VMDEBUG, not at the DOS level.  Using

ZD to load DOS will not redirect the input inside the virtual DOS.

If you are using command line re-direction with VMDEBUG, you need at least

FILES=6 in your CONFIG.SYS file.  A minimum of FILES=7 is recommended to

allow echoing of output via the ZO command at the same time.

Examples:

ZI input.dat - Take input from the INPUT.DAT file, if it exists, until

a ZI command, a blank line, or the end of the file.

ZI - Take input from the CON device (the keyboard).

ZI PRN - Attempt to take input from the printer.  As the printer will

not return any input, VMDEBUG will restore input to the console (the

keyboard).

Error messages:

ERROR: File not found. - The specified file or device does not exist.

ERROR: Path not found. - The specified path for the file does not exist.

ERROR: Too many open files. - DOS does not have an open entry in its

internal tables to open the input file.  You need to specify a larger

number of files in your CONFIG.SYS file.

ERROR: Access denied. - You do not have permission to read the specified

file.  You may have to change the attributes of the file.

ERROR: DOS internal error.  Code=xx - DOS has returned an error that

should not be returned from the functions called.  The code displayed

is the return value from function 3D of INT 21.  Consult your DOS manual

for information on the problem, and contact DDI.

Page  88

ZK - Toggle passing of the Keyboard interrupt to the program running

under VMDEBUG.

The ZK command is used to toggle the passing of the keyboard interrupt to

programs running under VMDEBUG.  The new state is displayed.  The default is

to pass the interrupt into VMDEBUG.

This command is used when the user does not want ANY extra interrupts to

affect the program, such as the release of the return key when starting

program execution (see chapter 9 for details), or any accidental keypress.

Example:

ZK - Toggle the passing of the keyboard interrupt.

Page  89

ZL - Load the saved state of the virtual machine from disk.

WARNING:

Loading a saved image means that DOS file buffers are also restored,

and as the information that they contain is not current, their use

by DOS can destroy information on your hard disk!  For example, if one

of the buffers contains part of the FAT (file allocation table) for the

hard disk, DOS will assume that it is still current, when in truth some

or all of the space that is marked free in the old FAT is in use by

other files!

If the image file was just saved and restored, the space used by the

image file is not recorded in the FAT in the buffers, and if the

image is older (days/weeks/months), there could be dramatic changes

between the saved FAT and the disk's current status.

For more information on this subject, see chapter 7.

The ZL command is the complement of the ZW command, which writes the

state of the virtual machine to disk.  The filename to be loaded is

specified by the ZN command.

The restored image contains the state of the monochrome, CGA, EGA, and

VGA displays, the saved video memory, the registers, flags, and virtual

memory.  The information for the W (where was I?) command, the PC clone

status, and screen swapping status are not restored.  For a description of

the contents of an image file, see appendix A.

The software emulation of the 8259 is not restored by this command.  Use

the ZD first command if you need hardware interrupts.  Note that PC clone

status is also set by the ZD command.

The hardware state of the machine is not restored, which could include

the floppy disk motor running, the status of the sound port, or the DMA

address if the image was saved while a disk I/O operation was in progress.

The samed image must be loaded into a virtual memory that is at least

as large as the saved image.  If loaded into a smaller virtual memory,

VMDEBUG will report an error.

If the load fails, use the ZD command immediately to reset the state of

the virtual machine.  The video data is loaded first, so a failed load

means that the video data internal to VMDEBUG has been scrambled.

When the ZL command is used, pending interrupts are cleared and the

previous information for the W command is lost, even if the load fails.

If the load is successful, VMDEBUG responds with OK.

Page  90

Example:

ZL - Load the virtual state of the machine.  Remember to use a ZD command

first to initalize the "8259" and PC clone status, or PC clone status

may be via the ZC command.  If screen swapping is desired, the ZS

command is needed.

Error messages:

No image name. - A name has not been specified with the ZN command.

ERROR: Read past end of file. - The file is too small to fill virtual

memory and is probably not an image file.  Use the ZD command to

reset VMDEBUG.

ERROR: File not found. - The specified image file does not exist.

ERROR: Path not found. - The specified path for the image file does not

exist.

ERROR: Too many open files. - DOS does not have an open entry in its

internal tables to open the image file.  Specify a larger number of

files in the CONFIG.SYS file.

ERROR: Access denied. - You do not have permission to read the specified

file.  You may have to change the attributes of the file.

ERROR: Image too big for memory. - The image file is larger than the

available virtual memory space.

ERROR: DOS internal error.  Code=xx - DOS has returned an error that

should not be returned from the functions called.  The code displayed

is the return value from either function 3D or 3F of INT 21.  Consult

your DOS manual for information on the problem and contact DDI.

Use ZD to make sure that VMDEBUG is in a legal video state.

Page  91

ZM [seg] - display virtual memory size and RAM mapping.

The size of virtual memory is displayed in paragraphs (10 (16 decimal)

bytes each).  ZM is useful for seeing if sufficient memory exists for

loading DOS, or running a given program.

The size reported will always be a multiple of 1000, which indicates

a multiple of 64K.  A conversion chart from paragraphs to K is:

paragraphs   size in K

----------   ---------

1000          64K

2000         128K

3000         192K

4000         256K

5000         320K

6000         384K

7000         448K

8000         512K

9000         576K

A000         640K

The mapping of "non-existant" ram is displayed as a segment address

aligned on a 64K boundary, and is restricted to C000, D000, E000, or F000.

The default mapping is at D000.

If a segment address of C000, D000, E000, or F000 is given, memory will

be re-mapped, and both memory size and the new segment will be displayed.

If any other value is entered, VMDEBUG will respond with a "number?" error.

For a description of what "non-existant" memory is, see chapter 3.

Examples:

ZM - Displays number of paragraphs in virtual memory, and the segment

for "non-existant" memory.

ZM E000 - Remap the "non-existant" memory to the 64K segment at

E000:0000 to E000:FFFF.

Error message:

number? - The value given was not a number equal to C000, D000, E000,

or F000.

Page  92

ZN imagename - Set the image name for the ZL and ZW commands.

The ZN command is like the N command, except that the name specified is

used by the ZW and ZL commands.  A separate name is used for these commands

to prevent accidentally over-writing a file.

Example:

ZN program.img - Set the name for ZW or ZL to program.img.

Error message:

Image name? - A name was not specified on the command line.

Page  93

ZO [name] - Echo output from VMDEBUG to the specified file or device.

This command allows the screen output from VMDEBUG to echo to a file or

device.  If a name is not specified, or is sent to CON (the display screen),

any previous ZO is canceled.

If output is sent to a file, VMDEBUG will open the file and append the

output to the end of the file.  If the file does not exist, VMDEBUG will

attempt to create it.

The ZO command echos rather than redirects the output.  You can observe

the output, correcting typing errors or any unexpected result.

If you are using the ZI command as well, you should have at least FILES=7

in your CONFIG.SYS file.  See chapter 7 for details.

Output echo is accomplished within VMDEBUG and not at the DOS level.

Using ZD to load DOS does not echo the output from the virtual DOS.

Any error encountered while echoing output causes termination of echoing,

and gives an error message before the next VMDEBUG prompt.  The most common

error is a "disk full" message, although others can occur.  The message

will be "Output echoing aborted," followed by an explanation.

Output echoing should be avoided if the virtual application or virtual

DOS will be writing information to the same disk.  See "File buffers and

disk conflicts" in chapter 7.

Examples:

ZO output.dat - Echo the output from VMDEBUG to OUTPUT.DAT.

ZO - Stop any echoing of output.

Error messages:

Output echoing aborted. - The output echoing has been aborted within

VMDEBUG.  An explanation follows on the next line.

ERROR: Disk full. - The file is too big to fit on the disk.  Try moving

the file to a hard disk.

ERROR: Path not found. - The specified path for the output file does not

exist.

ERROR: Too many open files. - DOS does not have an open entry in its

internal tables to open the image file.  You need to specify a larger

number of files in your CONFIG.SYS file.

ERROR: Access denied. - You do not have permission to write to the

specified file.  You may have to change the attributes of the file.

ERROR: DOS internal error.  Code=xx - DOS has returned an error that

should not be returned from the functions called.  The code displayed

is the return value from function 3C, 3D, or 40 of INT 21.  Consult

your DOS manual for information on the problem, and contact DDI.

Page  94

ZP seg1:ofs1 [seg2:ofs2] ... [seg20:ofs20]

A bug exists (sort of... only a program like VMDEBUG would cause it) in

COMMAND.COM prior to DOS 3.0.  What happens is that setting up virtual

memory essentially lowers the top of available memory, and the top of

memory is where the transient portion of COMMAND.COM is kept.  When control

returns to DOS, the resident portion of COMMAND.COM does a checksum on the

transient portion, and if the checksum is not valid, re-loads the transient

portion and checksums it again.  If still not valid, an error message is

issued, and if COMMAND.COM is being loaded from a hard disk, the system is

halted as COMMAND.COM in memory thinks that COMMAND.COM has been trashed

on the disk.

What is really happening is that the resident portion of COMMAND.COM is

doing a checksum on the memory where the transient portion was, which no

longer exists (virtual memory is smaller than the real memory).  As a result,

the transient portion of COMMAND.COM is re-loaded into virtual memory at the

correct location, but the resident portion of COMMAND.COM checksums the

transient portion again at the old, invalid location.  As the checksum is

not valid, COMMAND.COM issues its warning message.

This "bug" may be avoided by simply patching the memory location that

contains the location of the transient portion of COMMAND.COM.  The

ZP command patches the location, re-applying the patch every time the ZD

command is used.

The location of the patch, unfortunately, varies with every release of

DOS, and every manufacturers IO.SYS (IBMBIO.COM for PC-DOS).  The patch

to be applied varies every time a device driver is added or deleted, or a

BUFFERS statement is changed in the CONFIG.SYS file.  As a result, there is

no way for VMDEBUG to determine the exact location of the patch.  Instead,

simple instructions are given in chapter 7 for determining the location of

the patch for a given configuration.  The patch information may be kept in

the configuration file (VMDEBUG.CFG), but be warned that changing your

CONFIG.SYS file can change the location of the patch, and configuration

files are easily ignored.

The value of the patched is automatically calculated by VMDEBUG, and so

only the location is needed.

The best solution for the DOS version problem is to upgrade your DOS.

Hard disk owners will especially benefit from the smaller cluster size,

which will free up more disk space.

Another problem that requires the Patch DOS command involves machines

such as the HP Vectra, an AT compatible machine, that reserves an area of

memory at the top of RAM for use by its BIOS.  Although the memory is

physically present, its address has changed, due to VMDEBUGs memory mapping

that protects DOS and VMDEBUG.  In this case, the references to the RAM must

be changed, and in particular for the HP Vectra INT 6F must be patched.  See

the above section on the HP Vectra for details.

Page  95

Once the patch location is determined, the locations must be entered in

the form segment:offset, with both segment and offset specified.  No checking

is done on the patch locations.  When a valid patch is entered, VMDEBUG

responds with "DOS patched."

Note that DOS does not need to be loaded to use the ZP command.  Even if

DOS is not loaded, the memory bytes are patched.

Examples:

ZP 2317:228 - The patch to be applied is in segment 2317, offset 228.

ZP 0:6F*4+3 c44:519 - Patch INT 6F (for the HP Vectra) and COMMAND.COM.

Error message:

number? - A parameter was left off the command line, or was not a number.

Page  96

ZR - Display the current contents of the video display registers.

The ZR command displays the current contents of the video display

registers, including the contents of the write-only registers.  The values

displayed are those written to the hardware ports, and are not the contents

of the BIOS save area.  Thus, the status of programs that bypass BIOS

can be examined.  Screen swapping does not need to be enabled to use the ZR

command.

The set of video registers presented correspond to the primary display

adapter at the start of VMDEBUG.  If a program uses two screens, or switches

modes during program execution (i.e. an EGA or VGA display), VMDEBUG cannot

know this, and shows the registers for the original display.

When the ZE command switches between EGA or VGA and CGA/MDA screen

swapping, the set of registers displayed changes as well.

Most registers are not accessed by individual port numbers, but instead

are accessed by sending an address to one port, and the data to another port.

VMDEBUG displays this sort of register as the name of the hardware device,

followed by a set of hexadecimal bytes which represents the registers in

index order.

Example:

ZR - Display the video display adapter registers.

Page  97

ZS - Toggle screen swapping mode.

The ZS command toggles the status of screen swapping mode.  The new mode

is displayed.  The default at the start of VMDEBUG is screen swapping

disabled.  Note that there is nothing in the video buffer at the start of

the program, and the real screen is not loaded into the buffer until a ZD

command is used.

If screen swapping is enabled, the screen is swapped with the screen

saved by the ZD command at the start and end of the G, P or X commands.

You MUST have a Monochrome Dispaly Adapter (MDA), a Color Graphics Adapter

(CGA), an Enhanced Graphics Adapter (EGA), or Video Graphics Adapter (VGA)

for this to work; otherwise your screen mode could be undefined.  Also,

if you are not in a standard BIOS mode (3 for a CGA/EGA/VGA, and 7 for

monochrome), VMDEBUG forces you into a standard mode at the start of the

program.  This allows VMDEBUG to use a standard screen for its interface.

The mode reset at program entry may be defeated by using ANY command line

parameter.

The actual state of the video hardware is saved, not just the BIOS

parameters.  Thus, if the program being interpreted sets a custom screen

mode by programming the CRTC (CRT Controller), that mode will be restored

when screens are swapped.

When screen swapping is enabled all memory references to the video

memory are mapped to the virtual screen, even if the VMDEBUG debugging

display is in use.  Thus, for example, the Dump command, when used on

video memory, will show the contents of the virtual screen, not the

contents of the VMDEBUG debugging display (which, of course, contains

the Dump).  Another example is a program which modifies the screen

contents, which may be Assembled within the video memory and run with

the Trace command (which leaves the VMDEBUG debugging display active).

The program and its results will be in the virtual screen, not the

VMDEBUG debugging screen.

The only failing of this feature involves its use with EGA/VGA cards

in graphics mode.  When in graphics mode, these cards can have their

video memory moved and the active bit planes changed in such a way that

screen swapping is necessary to properly isolate the VMDEBUG debugging

display from the virtual screen.  For this case, the P command may be used

to swap screens, execute the instruction, and restore the VMDEBUG display.

If the software under test changes default display adapters (or modes,

on an EGA or VGA display), VMDEBUG continues to use the display that was

active when VMDEBUG was started.

Example:

ZS - Toggle state of screen swapping.  The new state is displayed.

Page  98

ZT [number [irq]]

The ZT command sets the divisor for the timer (clock interrupt), or the

specified IRQ (interrupt request) to number+1. The default clock divisor is

7D0 (2000 decimal) for a PC and 1F4 (500 decimal) for a AT, corresponding

to 1 clock interrupt every 110 seconds on a PC, and to 1 clock interrupt

every 27.5 seconds on a AT.  The default divisor for the other IRQs is 0,

which corresponds to no division.

The clock divisor may be adjusted for best performance with your system

and software.   Using a smaller divisor gives a closer performance/time

match, but uses more of the interpreter's time for processing the clock

interrupt.

The IRQ divisors are available so that devices such as Bus mice (which

generate periodic interrupts like the clock interrupt) can be installed

when VMDEBUG is run, without having VMDEBUG spend most/all of its time

interpreting their interrupt drivers.  The legal values of IRQ are 02

through 0F.  Note that the clock interrupt is IRQ 0, and the keyboard is

IRQ 1, so that all interrupts except the keyboard may be divided.  It is

the responsibility of the user to insure that the correct IRQ is given to

the ZT command, as dividing something like the Hard Disk interrupt is not

recommended.

When an IRQ divisor is used, the nature of interrupt handling by

VMDEBUG changes from that discussed in chapter 4.  If this interrupt

is not passed into VMDEBUG for processing, the user may select (via the

ZU command) if the necessary interrupt processing is to be handled by

the normal system interrupt handler or by VMDEBUG.

Using FFFF for the divisor turns off the interrupt, so that VMDEBUG no

longer sends the interrupt to the interpreted system.

If no parameters are given, the current divisors are displayed.

Examples:

ZT 200 - Set the timer divisor to 200 (hex).

ZT - Display the current divisors.

ZT 50 5 - Set the divisor for IRQ 5 to 50 (hex).

Error message:

number? - The specified divisor or IRQ was not a number.

Page  99

ZU

When IRQs are divided by VMDEBUG, there is a question of what to do

with the interrupts that are not passed into VMDEBUG.  Although processing

is necessary (such as resetting the interrupt controller, the 8259), it is

not clear who should do this processing.  The ZU command allows you to

toggle handling between internal handling within VMDEBUG (the default)

and passing the interrupt to the normal system vector.

However, note that the normal system vector should be defined in any

case.  If a train of interrupts is initiated by a program running under

VMDEBUG (such as initalizing a bus mouse), the system must be able to

handle the interrupt when control returns to the VMDEBUG command level.

The purpose of this command is to allow the user to "shut off" interrupt

handlers while VMDEBUG is running a program.

Example:

ZU - Toggle who has control of IRQs not passed into VMDEBUG (unused IRQs).

Page 100

ZV [number] - Toggle long Video retrace mode.

Some programs, most notably the EGA BIOS, try to reprogram the hardware

entirely during the video retrace, and if the retrace ends before the

update is complete, will reset themselves and try again until they are

successful.

However, as VMDEBUG is slower than an actual 8088, such programs will

keep trying forever.  Occasionally, they will succeed, as the checks for

retrace will occurr on successive screen retraces, but this is not a

reliable method.

When the long video retrace mode is enabled, VMDEBUG will simulate a

video retrace mode that lasts for 18 (decimal) clock ticks (1 second)

every 36 clock ticks (2 seconds) by adjusting the values returned from

a read of the byte I/O port 3BA (monochrome) or 3DA (color).

The default state is long video retrace off, and when changed the new

state is displayed.

A number may also be given to the ZV command, and when given enables

the long video retrace and specifies its length (in clock ticks, 1/18th

of a second each).

Note that some programs, such as BASIC, change the clock tick rate.  Such

changes will force a shorter "long" video retrace mode.

Examples:

ZV - Toggle the long video retrace mode.

ZV 24 - Set long video retrace mode, with a length of 36 decimal

clock ticks (2 seconds).

Error message:

number? - The specified number of clock ticks was not a number.

Page 101

ZW - Write the state of the virtual machine to disk.

WARNING:

Saving an image means that DOS file buffers are also saved.

Since the information in the buffers is not current, their use by DOS

later can destroy information on your hard disk!  For example, if one

buffer contains part of the FAT (file allocation table) for the hard

disk, DOS assumes the information is still current, when in truth some

or all of the space that is marked free in the old FAT is in use by

other files!

If the image file was just saved and restored, the space used by the

image file is not recorded in the FAT in the buffers, and if the image is

older (days/weeks/months), there could be dramatic changes between the

saved FAT and the disk's current status.

For more information on this subject, see chapter 7.

The ZW command is the complement of the ZL command, which reads the

state of the virtual machine from disk.  The filename to be created is

specified by the ZN command.  If the filename already exists, its previous

contents will be destroyed when the ZW command is used.

The saved image contains the state of the monochrome, CGA, EGA, and VGA

displays, the saved video memory, the registers, flags, and virtual memory.

The information for the W (where was I?) command, the PC-XT clone status,

and screen swapping status are not saved.  For a description of the

contents of an image file, see appendix A.

When writing an image file, remember that the file created is very large.

The size of the image file is 5274 (decimal) bytes larger than the size of

the virtual memory.   So, if the virtual memory size is 320K (5000 paragraphs

reported by the ZM command), the image file size will be 332,954.  If the

virtual memory size is 448K (7000 paragraphs reported by the ZM command),

the image file size will be 464,026.  Thus, a 320K virtual memory image file

is the largest that will fit on a 360K DSDD floppy.

An image file should not be written while the virtual machine is in the

middle of disk access.  At such a time, an interrupt from the hardware is

expected and will not be coming when the image is restored.

If the write is successful, VMDEBUG responds with OK.

If the write is not successful, VMDEBUG tries to delete the partial file.

VMDEBUG does not check for an error on the deletion, as processing that error

could lead to an infinite loop.

Page 102

Example:

ZW - Write the current state of the virtual machine to disk.

Error messages:

No image name. - A name has not been specified with the ZN command.

ERROR: Disk full. - The file is too big to fit on the disk.  Try moving

the file to a hard disk.

ERROR: File not found. - The specified image file does not exist.

ERROR: Path not found. - The specified path for the image file does not

exist.

ERROR: Too many open files. - DOS does not have an open entry in its

internal tables to open the image file.  You need to specify a larger

number of files in your CONFIG.SYS file.

ERROR: Access denied. - You do not have permission to read the specified

file.  You may have to change the attributes of the file.

ERROR: DOS internal error.  Code=xx - DOS has returned an error that

should not be returned from the functions called.  The code displayed

is the return value from either function 3C or 40 of INT 21.  Consult

your DOS manual for information on the problem, and contact DDI.

Page 103

ZY

The ZY command toggles the remapping of DMA to and from the ROMs remapped

by the ZZ command.  The default is not to remap DMA.

Since the ZZ command moves ROM space into RAM, the remapped ROMs are

subject to being over-written.  To help overcome this problem, DMA is left

at the old ROM location, which cannot be over-written.  This means that

DMA from disk to ROM will leave the new ROMs intact, but also means that

DMA from ROM to disk will write the wrong information.  If DMA remapping

is enabled, when the ZZ command is used the DMA addresses for the remapped

ROM will be changed as well.  This means that DMA from ROM to disk will

write the correct information, but DMA from disk to ROM will destroy the

ROMs!

This remapping applies only to programs and systems running under

VMDEBUG.  The L (load file) and C (create file) commands will always

access the remapped ROMs.

It should be noted that DOS seems to move data to an internal buffer

for reading and writing before disk access is done, so that not remapping

DMA does not render the ROMs totally safe.

Example:

ZY - Toggle remapping of DMA to and from ROM during ZZ commands.

Page 104

ZZ [file1 [seg1 [file2 [seg2 [file3 [seg3 [file4 [seg4 [commandline]]]]]]]]]

VMDEBUG can load ROM replacements from files into RAM and remap virtual

memory so they appear at the correct address.  This ROM replacement can

occur at the 64K segments C000, D000, E000, and F000, so that the remapped

locations are C000:000 to C000:FFFF, D000:0000 to D000:FFFF, E000:0000 to

E000:FFFF and F000:0000 to F000:FFFF.  This command is useful for testing

ROMs before committing them to chips.

The files are read in as straight data (i.e. no .EXE relocation occurrs),

and may be any size, although VMDEBUG will read a maximum of 64K per file.

If a segment is specified (only the last file on the command line does not

need to specify the segment) it must be C000, D000, E000, or F000.  If the

segment is not specified, it defaults to F000.

Remapping ROM into RAM decreases the available RAM for virtual memory,

which is reflected in the ZM command.  As a result, the ZP command should

be re-used after the ZZ command is used.  Also, note that the image files

read and written by the ZL and ZW commands expect virutal memory to be at

least as large as the saved image, so loading an image saved without ROM

remapping after a load of ROMs by the ZZ command will fail.

If no arguments are given, ZZ will restore the original ROMs, and return

the memory used to the virtual memory space.  Also, if there is an error

in the arguments, the normal ROMs will be restored.  In both cases, the

ZZ command will respond with "Normal ROM restored." and do a ZD (load DOS)

command.

For example, on a 640K machine with no RAM disks or disk buffering, a ZZ

command of:

ZZ SOMEROM1.DAT D000 EGAROM.DAT C000 BIOSROM.DAT F000 SOMEROM2.DAT E000

will produce a memory mapping of:

Page 105

Actual Memory                              Virtual Memory

Segment ����������������ͻ                       �������������������ͻ Segment

�                �                       �  �                � maps to

0000  � DOS            �                       �  � Virtual DOS    �  7000

����������������͹                       �  ����������������͹

�                �                       �  �                �

1000  � VMDEBUG code   �                    ��Ĵ  � Available RAM  �  8000

����������������͹                    �  �  ����������������͹

�                �                    �  �  �                �

2000  � VMDEBUG data   �                    �  �  � Available RAM  �  9000

����������������͹�Ŀ                 �  �������������������͹

� Segment D000   �  �                 �  �  � "Non-Existant" �

3000  � ROM            �  �����������Ŀ     �  �  � RAM            �  D000

����������������͹�Ĵ           �     �  �  ����������������͹

� Segment C000   �  �           �     �  �  � "Non-Existant" �

4000  � ROM            �  ��������Ŀ  �     �  �  � RAM            �  D000

����������������͹�Ĵ        �  �     �  �  ����������������͹

� Segment F000   �  �        �  �     �  �  � "Non-Existant" �

5000  � ROM            �  �����Ŀ  �  �     �  �  � RAM            �  D000

����������������͹�Ĵ     �  �  �     �  �  ����������������͹

� Segment E000   �  �     �  �  �     �  �  � "Non-Existant" �

6000  � ROM            �  ��Ŀ  �  �  �  �����Ĵ  � RAM            �  D000

����������������͹�Ĵ  �  �  �  �  �  �  �  ����������������͹

� Segment 0000   �  �  �  �  �  �  �  �  �  � "Non-Existant" �

7000  � Virtual RAM    �  �  �  �  �  �  �  �  �  � RAM            �  D000

����������������͹  �  �  �  �  �  �  �  �  ����������������͹

� Segment 1000   �  �  �  �  �  �  �  �  �  � "Non-Existant" �

8000  � Virtual RAM    �  ��ĳ�ĳ�ĳ�ĳ�ĳ���  �  � RAM            �  D000

����������������͹  �  �  �  �  �  �     �  ����������������͹

� Segment 2000   �  �  �  �  �  �  �     �  � "Non-Existant" �

9000  � Virtual RAM    �  �  �  �  �  �  �     �  � RAM            �  D000

top of ����������������͹���  �  �  �  �  �     �������������������͹

RAM   ����������������͹�Ŀ  �  �  �  �  �     �������������������͹

� Video          �  �  �  �  �  �  �     �  � Video          �

A000  � Expansion      �  �  �  �  �  �  �     �  � Expansion      �  A000

����������������͹  ��ĳ�ĳ�ĳ�ĳ�ĳ����Ĵ  ����������������͹

�                �  �  �  �  �  �  �     �  �                �

B000  � Video          �  �  �  �  �  �  �     �  � Video          �  B000

����������������͹���  �  �  �  �  �     �������������������͹

� EGA BIOS and   �     �  �  ��ĳ�������Ĵ  � EGA BIOS and   �

C000  � Hard Disk BIOS �     �  �     �  �     �  � Hard Disk BIOS �  4000

����������������͹�Ŀ  �  �     �  �     �������������������͹

� ROM            �  �  �  �     ��������Ĵ  � ROM            �

D000  � Expansion      �  ��ĳ�ĳ���������     �  � Expansion      �  3000

����������������͹���  �  �              �������������������͹

� ROM            �     ��ĳ�������������Ĵ  � ROM            �

E000  � Expansion      �        �              �  � Expansion      �  6000

����������������͹        �              �������������������͹

�                �        ��������������Ĵ  �                �

F000  � BIOS           �                       �  � BIOS           �  5000

����������������ͼ                       �������������������ͼ

Page 106

When ROM is remapped into RAM, the ROM segments become writeable, so

that the "ROM" is subject to being over-written by a run-away program.

However, DMA (Direct Memory Access, used by disk I/O) is a separate mapping,

and need not be remapped to the new ROMs, so that they are safe from disk

I/O.  See the ZY command for details.

The ZZ command also does a ZD command, to reset VMDEBUG and virtual DOS.

If a command line is desired, it can be used only after all four filenames

and segments, otherwise it will be treated as a ROM filename.

Examples:

ZZ EGAROM.DAT C000 BIOSROM.DAT - Replace the EGA BIOS ROM at C000, and

the normal BIOS ROM at F000.

ZZ - Restore the normal ROMs.

Error Messages:

Invalid ROM segment. - The segment specified was not C000, D000, E000, or

F000.

Duplicate ROM segment. - The same segment was specified for more than one

ROM file.

ROM file not found. - The specified file could not be opened.

Error reading ROM file. -  Dos reported an error trying to read the

ROM file.

Page 107

The VMDEBUG.CFG configuration file

----------------------------------

The input redirection feature of VMDEBUG is used to search for a file

named VMDEBUG.CFG in the current directory at the start of the program.

If VMDEBUG.CFG exists, the contents are read and processed by VMDEBUG

as if VMDEBUG.CFG were begin processed by the ZI command.  This allows

you to set up a command file to automatically initalize VMDEBUG upon

entry to the program.

The VMDEBUG.CFG file may contain any sequence of commands, although

the most common ones would be ZD, to load DOS, ZP to patch DOS (for

Pre-DOS 3.0), and ZS to allow screen swapping.  When using ZP in the

VMDEBUG.CFG file, remember the warnings presented in chapter 7:

A change to the CONFIG.SYS file will change the location of the patch.

As with the ZI command, a blank line terminates file processing and the

Esc key may be used to terminate a command or to terminate the processing.

Also, if the file uses the G, P, or X commands, the hotkey is not read from

the file.

Page 108

Chapter 7

VMDEBUG and DOS

---------------

Finding the patch for pre-DOS 3.0 COMMAND.COM

---------------------------------------------

This section can also serve as an example on the use of VMDEBUG, as the

program is used to trace DOS itself, with no problems associated with the

non-reentrancy of DOS.

First, an explanation of the problem:

Apparently, before DOS 3.0, the designers of MS-DOS (PC-DOS for the IBM

machines) never anticipated that the memory size available would shrink

after the system boot.  As a result, COMMAND.COM cannot handle the situation

in which the top of memory is adjusted downwards by a program.

What happens is a result of the fact that there are two parts of the

COMMAND.COM file, one of which is resident in memory at all times just after

the end of DOS, and the other which is loaded into high memory whenever

needed.  This arrangement is a relic from the days when memory was expensive.

Rather than allocating 20K or so permanently to COMMAND.COM, a piece about

4K in size is permanently resident, with the other 16K or so loaded in high

memory when needed and found to be invalid.  When a program needs memory,

the program simply uses high memory and overwrites the information there.

When control returns to COMMAND.COM, the resident portion does a checksum of

the memory where it left the transient portion of the program.  If the

checksum is invalid, the resident portion assumes that a program has

over-written the transient portion of COMMAND.COM, and re-loads the transient

portion from disk.  The resident portion then checksums the transient piece

again, and if the checksum is still invalid, the transient portion assumes

that COMMAND.COM on the disk itself has been destroyed.  If COMMAND.COM is

being reloaded from a hard disk, the system is halted as there is no way to

replace the disk with another disk that has a valid COMMAND.COM.

However, there is a flaw in the checksum logic of COMMAND.COM before

DOS 3.0.  If the transient piece is found to be invalid, the resident portion

uses the BIOS data to compute the new load address for the transient piece,

loads the transient piece, AND THEN CHECKSUMS THE PREVIOUS LOCATION OF THE

TRANSIENT PIECE.  If the size of memory has changed, the load location and

the previous location will not be the same, and COMMAND.COM will never find

a valid transient piece.

In order to circumvent this for VMDEBUG, the data location in

COMMAND.COM used to store the location of the transient piece must be

modified, which is easily done.  The contents of the location is a segment

address, and must be replaced by the corresponding location in virtual

memory.  Only the high byte needs to be changed.  To create the patch,

VMDEBUG needs only the location of the high byte of the segment address.

The patch value is automatically computed by VMDEBUG.

Page 109

The location of the segment value changes in every version (and even

minor variation) of DOS.  The location itself, however, is worthless.

Due to device drivers, number of buffers used, number of files, etc, the

location changes with EVERY variation of CONFIG.SYS.  Thus, rather than

giving useless locations, a procedure for finding the location for any

version of DOS 2.x and CONFIG.SYS is given below.

THIS NEEDS TO BE DONE ONLY ONCE FOR EACH COMBINATION OF DOS AND CONFIG.SYS!

In the following, the > is the VMDEBUG prompt, and should not be typed.

>ZD            Load a local copy of DOS

>G 0:4C        Start executing (VMDEBUG has loaded an INT 20 for you, which

returns you to DOS), and stop when the memory address for

the INT 13 vector is referenced.  This halts VMDEBUG before

the first disk access occurs.

>G 0:84        Start executing again, and this time stop when the memory

address for the INT 21 vector is referenced.  This halts

VMDEBUG at the next DOS call, which should have AH=42, the

"move file pointer" function.

Hard disk users, take note:  Due to the magic of file buffers,

it is possible that this instruction will be skipped, and

the value in AH will be 3E.  If this is the case, you can

skip down to the next "stop at DOS" interrupt below.

>G 0:4C        Start execution again, and stop at the next INT 13 (disk

access).

If you are loading COMMAND.COM from a floppy, do the following step:

>E 40:3F 0     Tell the virtual copy of DOS that the disk is not running

(the address can also be expressed as 0000:043F).  You can

skip this if the floppy drive LED is still on when you want

to continue with the next step.

Back to general instructions:

>G 0:84        Stop at the next DOS call (INT 21).  This should have AH=3E,

which is the "close file handle" function.

>P             Use the Proceed command to execute the interrupt and stop

on the instruction immediately after the call.

>T 9           Trace the next 9 instructions.  Note that this is for MS-DOS

2.11.  PC-DOS 2.0 and 2.1 require only 8 instructions.  I do

not have all versions of MS-DOS to test this on, however, and

you might have to adjust this up or down a little.

When all this is over, VMDEBUG displays an instruction of the form:

xxxx:yyyy  2E        CS:

xxxx:uuuu  8E1Ezzzz  MOV   DS,[zzzz]                  xxxx:zzzz=jjjj

The needed numbers are the values of CS (i.e. the xxxx above) and the number

in the square brackets (the zzzz).  The value to be moved, the jjjj, is

interesting but not necessary.

Page 110

The address xxxx:zzzz is the location of the segment value that must be

changed.  Remembering that the 8088 stores low byte first (i.e., doing a

D xxxx:zzzz l2  will show, for example A7 9F for 640K DOS 2.x), the byte

to change is the byte at zzzz+1.  Thus, the patch location is the address

xxxx:zzzz+1, with xxxx and zzzz the numbers found above.

To bring up a working copy of DOS, use the ZP command to apply the

patch, and the ZD command to re-load (and re-patch) a virtual copy of DOS.

For example:

ZP xxxx:zzzz+1

ZD

This is necessary as the Trace command shows the instruction BEFORE its

execution, and so the value has already been used by the time you are at

the VMDEBUG command prompt.

YOU DO NOT HAVE TO DO THIS EVERY TIME YOU WISH TO BRING UP DOS!

Now that the address is known, simply use a ZP command once at the start

of VMDEBUG to apply the patch.  The patch will be applied automatically

with every ZD command thereafter.

Remember, however, that every change to CONFIG.SYS changes the location

of the patch.  It might be prudent to have a "known" version of CONFIG.SYS

around (named, perhaps, CONFIG.VMD) that will put the patch location at a

know place.

The ZP (patch DOS) command may be included in the configuration file,

VMDEBUG.CFG, but is easily forgotten, and thus the wrong patch may be

applied.  If a patched copy of virtual DOS acts funny, be sure to check

the location of the patch before assuming the error is yours.

File buffers and disk conflicts

-------------------------------

File buffers, wonderful things which DOS could not get along without,

make disk access faster and easier.  Unfortunately, file buffers are also

a problem when used with a program like VMDEBUG.  In many ways running

VMDEBUG resembles running a computer under 2 operating systems at the same

time.  The problem appears mostly with the hard disk, as DOS realizes that

floppy disks may be changed.

When VMDEBUG is running a virtual copy of DOS, the virtual DOS has its

own virtual buffers, that reflect the state of the disk drives as loaded

from the real DOS and the changes made by virtual DOS and virtual

applications.  However, as the virtual machine has no idea that a real

machine or that VMDEBUG exists, the virtual DOS does not inform the real

DOS of changes made to the disks.

This situation is normally livable, because the virtual buffers are

accurate when initially loaded and the virtual application records its

results in the virtual buffers.  Also, the real file buffers are flushed

Page 111

by VMDEBUG before every command prompt, keeping the real copy of DOS as

current as possible.  Nevertheless, DOS insists on keeping and using the

information contained in its file buffers, notably the File Allocation

Table (the FAT). Doing a reboot after a VMDEBUG session changes the

contents of the disk is recommended (although CHKDSK seems to work well

for actually flushing all buffers).

Problems can arise, however, when the disk is modified by VMDEBUG after

the virtual DOS has been loaded.  When output is redirected via the ZO

command, the real DOS creates a file that did not exist when the virtual

DOS was loaded.  If the virtual buffers contain the FAT (File Allocation

Table), the virtual DOS may use part of the disk now occupied by your

output file.  This will destroy your output file and may cause lost

chains in the file allocation table.  Although ugly, this is not too

damaging to your disk, will not damage any existing files, and may be

repaired with CHKDSK (remember to use the /F option).

The situation is even worse with command line redirection of output, as

loading a virtual copy of DOS when command line redirection is in effect

copies the redirection into the virtual DOS, so that both the real DOS and

the virtual DOS will be attempting to write to the same file at the same

time, and allocate the same space for the file, each without telling the

other what is being done.  Again, this should not damage any other files,

but the allocation chain for the output file can be totally destroyed.  If

you encounter this, use CHKDSK with the /F option to clean up your disk.

The final straw comes, however, when the ZW and ZL commands are used.

These commands write the current state of the virtual machine, file buffers

and all, to the disk and read it back again.  The saved file buffers have

no information on the large image file and may be totally inaccurate if

moved to another machine or if major changes have occurred since the file

was written.  The virtual DOS, however, has no indication of this and uses

the saved information as if it were valid.  This can wipe out the image

file at a minimum, and may destroy large areas of data on the disk if major

changes have been made since the image file write.

These problems are also inherent when a disk caching program is used

with VMDEBUG.  To make matters even worse, the caching program keeps

large areas of the disk in memory, and has no means to flush them, as

is possible with DOS.  The virtual DOS will also have its copy of the

cached data, and this may be totally different than that used by the

real DOS, and so use of the data can destroy large amounts of data on a

disk.  Thus, in order to minimize risks, such programs should not be used.

I/O redirection

---------------

I/O redirection is supported at two levels within VMDEBUG.  At the DOS

level, the standard redirection symbols '<' (redirect input) and '>'

(redirect output) may be used.  Also, using Ctrl-P to echo console I/O

Page 112

to the printer works, and even is recommended to copy output to the

printer.  Be warned, however, that input redirection may hang you in the

VMDEBUG program, as DOS level redirection does not return control to the

keyboard at the end of the input file, and Ctrl-C and Ctrl-Break are

disabled.  Thus, if the input file does not contain a 'Q' command to exit

VMDEBUG, it may be necessary to reboot the system.

If a virtual copy of DOS is run when redirection is in effect, the

virtual copy of DOS also redirects its output, with possibly very nasty

effects, such as having two copies of DOS (one real and one virtual)

trying to write to the same file at the same time.

Doing a program terminate (the default INT 20 instruction after a ZD

command) in the virtual DOS before loading and running an application

cancels redirection for the virtual DOS (but does not cancel Ctrl-P echo),

so only the real DOS will be accessing the file.

Redirection within VMDEBUG is also supported, and is the recommended

method. Both the ZI and ZO commands are written so that redirection is

invisible to a copy of virtual DOS loaded while ZI and ZO are in effect.

Moreover, the ZI input redirection terminates whenever a blank input line

is encountered, which includes errors and the end of file.  The ZO output

echoing allows you to see the output as it goes to the file or device,

allowing you to make corrections if necessary.  In addition, error

conditions such as disk full are noted, echoing is terminated cleanly,

and the error is reported to the user.

Both command line redirection and redirection within VMDEBUG may be used

at once.  Both the VMDEBUG.CFG file and the ZI command redirect input until

terminated by VMDEBUG and then return input to the file specified on the

command line.  Output echoing via the ZO command sends output to both the

command line redirection and the file or device specified.  Note, however,

that CON is a special case to ZO, and so it may not be used to echo

redirected output to the screen.

I/O redirection uses file handles, as does command line I/O redirection.

The recommended minimum number of file handles is 7, set by FILES=7 in your

CONFIG.SYS file.

Page 113

Chapter 8

Examples

--------

Tracing a DOS command

---------------------

As the first example of the use of VMDEBUG, the DOS internal command

CTTY will be traced to find its function.  All output is copied from an

actual VMDEBUG session on a PC, run with PC-DOS 3.1, a 32K RAM disk using

VDISK.SYS, with no resident programs and no ANSI.SYS.  Having a DOS

technical reference in front of you helps things make more sense.  The

comments were not typed, but are added here to explain the steps used.

>ZD   - load DOS.

>G    - execute an INT 20, program terminate.

At this time, the screen is swapped, and after just under 2 minutes

(sigh), the C:\VMDEBUG>> prompt is up.  At this time, CTTY AUX is typed,

without a return.  Then VMDEBUG is interrupted via the hotkey.

Number of instructions executed: 0001:5E46

AX=8400  BX=0303  CX=00F9  DX=087F  SP=09EE  BP=0000  SI=00D3  DI=01B1

DS=012E  ES=012E  SS=012E  CS=012E  IP=4D88   NV UP EI NT PL ZR NA PE NC

012E:4D88  C7063D030000 MOV    WORD PTR [033D],0000             012E:033D=030D

>G 0:84    - stop at the next INT 21

Press return to end the command line to dos and wait a moment...

Number of instructions executed: 0000:07AA

AX=4000  BX=0001  CX=0002  DX=3B1B  SP=4806  BP=3981  SI=397F  DI=3B1B

DS=5B79  ES=5B79  SS=5B79  CS=5B79  IP=3333   NV UP EI NT PL NZ NA PO NC

5B79:3333  CD21         INT       21

Checking the DOS manual, this is write to file handle=1, STDOUT.

>D DS:DX LCX   - see what is being output

5B79:3B10                          -        0D 0A                       ..

it's a CR/LF

>P   - skip over this

Number of instruction executed: 0000:02C4

AX=0002  BX=...   The registers and instruction are not important.

>G 0:84    - find the next DOS call

Number of instructions executed: 0000:08F8

AX=2901  BX=3995  CX=0008  DX=3B7F  SP=4826  BP=0000  SI=3C04  DI=3E1B

DS=5B79  ES=5B79  SS=5B79  CS=5B79  IP=029C   NV UP EI NT PL NZ NA PO NC

5B79:029C  CD21         INT      21

Page 114

This is a parse filename.  What is it parsing?  Let's see.

>D DS:SI L10

5B79:3C00              63 74 74 79-20 61 75 78 0D 00 00 00       ctty aux....

5B79:3C10  00 00 00 00            -                          ....

It's the command line that was typed.

>P    - let it happen

Number of instructions executed: 0000:0213

AX=...   The registers and instruction are not important.

>G 0:84    -why not use sticky breakpoints?  So I don't have to remember

to turn them on and off.  Old habits die hard.

Number of instructions executed: 0000:0099

AX=2901  BX=439B  CX=FF04  DX=3B7F  SP=4826  BP=0085  SI=43A0  DI=005C

DS=5B79  ES=5B79  SS=5B79  CS=5B79  IP=031D   NV UP EI NT PL NZ NA PE NC

5B79:031D  CD21         INT      21

Parse filename again.  What is it this time?

>D DS:SI L5

5B79:43A0  61 75 78 00 00          -                         aux..

It's AUX this time.

>P  - do it.

Number of instructions executed: 0000:01F6

AX=...   The registers and instruction are not important.

>G 0:84   - stop on INT 21 again

Number of instructions executed: 0000:0009

AX=2901  BX=439B  CX=FF04  DX=3B7F  SP=4826  BP=0085  SI=0085  DI=006C

DS=5B79  ES=5B79  SS=5B79  CS=5B79  IP=033A   NV UP EI NT PL ZR NA PE NC

5B79:033A  CD21         INT      21

Parse filename again.

>D DS:SI L5

5B79:0080                 0D 0D 67-0D 76                         ..g.v

0D = Carriage return, end of command line.

>P    - go do it.

Number of instructions executed: 0000:0177

AX=...   The registers and instruction are not important.

Page 115

>G 0:84   - once more.

Number of instructions executed: 0000:02BA

AX=3D02  BX=FFFF  CX=0000  DX=0082  SP=4824  BP=0085  SI=0084  DI=0086

DS=5B79  ES=5B79  SS=5B79  CS=5B79  IP=1332   NV UP EI NT PL NZ AC PO NC

5B79:1332  CD21         INT      21

This is open file for read/write.  What is being opened?

>D DS:DX L5

5B79:0080        61 75 78 00 0D   -                            aux..

>P   - ok, go open the AUX device

Number of instructions executed: 0000:0657

AX=0005  BX=FFFF  CX=0000  DX=0082  SP=4824  BP=0085  SI=0084  DI=0086

DS=5B79  ES=5B79  SS=5B79  CS=5B79  IP=1334   NV UP EI NT PL NZ AC PO NC

5B79:1334  7210         JNAE    1346

No carry, the open was successful.  The handle is 5.

>G 0:84   - once again

Number of instructions executed: 0000:0003

AX=4400  BX=0005  CX=0000  DX=0082  SP=4824  BP=0085  SI=0084  DI=0086

DS=5B79  ES=5B79  SS=5B79  CS=5B79  IP=133B   NV UP EI NT PL NZ AC PO NC

5B79:133B  CD21         INT      21

Call the IOCTL function to find information about the file with the

handle in BX.  This is the AUX that was just opened.

>P     - go get the information.

Number of instructions executed: 0000:0149

AX=80C0  BX=0005  CX=0000  DX=80C0  SP=4824  BP=0085  SI=0084  DI=0086

DS=5B79  ES=5B79  SS=5B79  CS=5B79  IP=133D   NV UP EI NT PL NZ AC PO NC

5B79:133D  F6C280       TEST    DL,80

Since the requested information is in DX, look up what the test is doing.

This is checking for ISDV (is device) status, which is true.

>T   - since we are at an interesting point, trace a few instructions.

AX=80C0  BX=0005  CX=0000  DX=80C0  SP=4824  BP=0085  SI=0084  DI=0086

DS=5B79  ES=5B79  SS=5B79  CS=5B79  IP=1340   NV UP EI NT PL NZ AC PO NC

5B79:1340  750D         JNZ     134F

>T    - keep tracing.

AX=80C0  BX=0005  CX=0000  DX=80C0  SP=4824  BP=0085  SI=0084  DI=0086

DS=5B79  ES=5B79  SS=5B79  CS=5B79  IP=134F   NV UP EI NT PL NZ AC PO NC

5B79:134F  32F6         XOR     DH,DH

Page 116

>T     - keep tracing.

AX=80C0  BX=0005  CX=0000  DX=00C0  SP=4824  BP=0085  SI=0084  DI=0086

DS=5B79  ES=5B79  SS=5B79  CS=5B79  IP=1351   NV UP EI NT PL ZR AC PE NC

5B79:1351  80CA03       OR      DL,03

>T      - keep tracing.

AX=80C0  BX=0005  CX=0000  DX=00C3  SP=4824  BP=0085  SI=0084  DI=0086

DS=5B79  ES=5B79  SS=5B79  CS=5B79  IP=1354   NV UP EI NT NG NZ AC PE NC

5B79:1354  B80144       MOV     AX,4401

Since we just did an IOCTL, moving a 44 to AH sounds like setting up

another DOS call.  Check this instruction with a R before executing it.

>R       - what are we about to do?

AX=4401  BX=0005  CX=0000  DX=00C3  SP=4824  BP=0085  SI=0084  DI=0086

DS=5B79  ES=5B79  SS=5B79  CS=5B79  IP=1357   NV UP EI NT NG NZ AC PE NC

5B79:1357  CD21         INT     21

Yes, it's another IOCTL call.  This time, information is being set for

the AUX device that was opened.  The status that is being set is

ISCI (is console in, bit 0), ISCO (is console out, bit 1), EOF (bit 6),

and ISDV (is device, bit 7).  DH is 0, as it should be.

>P    - go set the status.

Number of instructions executed: 0000:0143

AX=44C0  BX=0005  CX=0000  DX=00C3  SP=4824  BP=0085  SI=0084  DI=0086

DS=5B79  ES=5B79  SS=5B79  CS=5B79  IP=1359   NV UP EI NT NG NZ AC PE NC

5B79:1359  53           PUSH    BX

This doesn't look interesting, skip to next DOS call.

>G 0:84     -skip to next DOS call.

Number of instructions executed: 0000:0004

AX=3EC0  BX=0000  CX=0003  DX=00C3  SP=4822  BP=0085  SI=0084  DI=0086

DS=5B79  ES=5B79  SS=5B79  CS=5B79  IP=1361   NV UP EI NT PL ZR AC PE NC

5B79:1361  CD21         INT     21

This is doing a close on handle 0, STDIN.  Life without it could be fun.

>P    - let DOS close it

Number of instructions executed: 0000:0198

AX=3E09  BX=0000  CX=0003  DX=00C3  SP=4822  BP=0085  SI=0084  DI=0086

DS=5B79  ES=5B79  SS=5B79  CS=5B79  IP=1363   NV UP EI NT PL ZR AC PE NC

5B79:1363  43           INC     BX

INC HANDLE could be interesting.  Let's see what's next.

Page 117

>G 0:84   - find next DOS call.

Number of instructions executed: 0000:0003

AX=3E09  BX=0001  CX=0002  DX=00C3  SP=4822  BP=0085  SI=0084  DI=0086

DS=5B79  ES=5B79  SS=5B79  CS=5B79  IP=1361   NV UP EI NT PL NZ NA PO NC

5B79:1361  CD21         INT     21

We seem to be in a loop closing file handles.  This time, the file being

closed is handle 1, STDOUT.

>P       - go close it.

Number of instructions executed: 0000:01AC

AX=3E08  BX=0001  CX=0002  DX=00C3  SP=4822  BP=0085  SI=0084  DI=0086

DS=5B79  ES=5B79  SS=5B79  CS=5B79  IP=1363   NV UP EI NT PL ZR AC PE NC

5B79:1363  43           INC     BX

Do the next iteration.

>G 0:84   - find the  next DOS call.

Number of instructions executed: 0000:0003

AX=3E08  BX=0002  CX=0001  DX=00C3  SP=4822  BP=0085  SI=0084  DI=0086

DS=5B79  ES=5B79  SS=5B79  CS=5B79  IP=1361   NV UP EI NT PL NZ NA PO NC

5B79:1361  CD21         INT     21

Close handle 2, STDERR.

>P     - go close it.

Number of instructions executed: 0000:0198

AX=3E07  BX=0002  CX=0001  DX=00C3  SP=4822  BP=0085  SI=0084  DI=0086

DS=5B79  ES=5B79  SS=5B79  CS=5B79  IP=1363   NV UP EI NT PL ZR AC PE NC

5B79:1363  43           INC     BX

Since CX=1, I think the loop is over.  Lets see what happens next.

>G 0:84   - Go to the next DOS call.

Number of instructions executed: 0000:0004

AX=4507  BX=0005  CX=0000  DX=00C3  SP=4824  BP=0085  SI=0084  DI=0086

DS=5B79  ES=5B79  SS=5B79  CS=5B79  IP=1369   NV UP EI NT PL NZ NA PE NC

5B79:1369  CD21         INT     21

Duplicate a file handle.  They are requesting a duplicate of the AUX

handle.

>P    - let them do it.

Number of instructions executed: 0000:0185

AX=0000  BX=0005  CX=0000  DX=00C3  SP=4824  BP=0085  SI=0084  DI=0086

DS=5B79  ES=5B79  SS=5B79  CS=5B79  IP=136B   NV UP EI NT PL NZ NA PE NC

5B79:136B  B445         MOV     AH,45

The file handle returned is 0, the first open handle.  This also happens

to be STDIN.  Also, mov AH,45 sounds like another DOS call.

Page 118

>G 0:84     - await the next DOS call.

Number of instructions executed: 0000:0001

AX=4500  BX=0005  CX=0000  DX=00C3  SP=4824  BP=0085  SI=0084  DI=0086

DS=5B79  ES=5B79  SS=5B79  CS=5B79  IP=136D   NV UP EI NT PL NZ NA PE NC

5B79:136D  CD21         INT     21

Yes, ask for another duplicate.  Can you guess what this one will be?

>P           - call DOS.

Number of instructions executed: 0000:0192

AX=0001  BX=0005  CX=0000  DX=00C3  SP=4824  BP=0085  SI=0084  DI=0086

DS=5B79  ES=5B79  SS=5B79  CS=5B79  IP=136F   NV UP EI NT PL NZ NA PE NC

5B79:136F  B445         MOV     AH,45

This is STDOUT, and once again it looks like they're setting up for

another DOS call.

>G 0:84      - await the DOS call.

Number of instructions executed: 0000:0001

AX=4501  BX=0005  CX=0000  DX=00C3  SP=4824  BP=0085  SI=0084  DI=0086

DS=5B79  ES=5B79  SS=5B79  CS=5B79  IP=1371   NV UP EI NT PL NZ NA PE NC

5B79:1371  CD21         INT     21

This call is no surprise.

>P      - go to DOS.

Number of instructions executed: 0000:019F

AX=0002  BX=0005  CX=0000  DX=00C3  SP=4824  BP=0085  SI=0084  DI=0086

DS=5B79  ES=5B79  SS=5B79  CS=5B79  IP=1373   NV UP EI NT PL NZ NA PE NC

5B79:1373  B43E         MOV     AH,3E

Yes, it's returned STDERR.  Now, it seems to be setting for yet another

DOS call.

>G 0:84   - go find out what it is.

Number of instructions executed: 0000:0001

AX=3E02  BX=0005  CX=0000  DX=00C3  SP=4824  BP=0085  SI=0084  DI=0086

DS=5B79  ES=5B79  SS=5B79  CS=5B79  IP=1375   NV UP EI NT PL NZ NA PE NC

5B79:1375  CD21         INT     21

Now the handle for AUX is being closed.

>P     - go close it.

Number of instructions executed: 0000:0198

AX=3E04  BX=0005  CX=0000  DX=00C3  SP=4824  BP=0085  SI=0084  DI=0086

DS=5B79  ES=5B79  SS=5B79  CS=5B79  IP=1377   NV UP EI NT PL NZ NA PE NC

5B79:1377  8E1EA33D     MOV     DS,[3DA3]               5B79:3DA3=13ED

This looks interesting.  Let's see what it is.

Page 119

>U CS:IP

5B79:1377  8E1EA33D     MOV     DS,[3DA3]

5B79:133B  1E           PUSH    DS

5B79:137C  A11800       MOV     AX,[0018]

5B79:137F  A3CB0A       MOV     [0ACB],AX

5B79:1382  B8CD02       MOV     AX,02CD

5B79:1385  50           PUSH    AX

5B79:1386  CB           RETF

5B79:1387  8E06A33D     MOV     ES,[3DA3]

5B79:138B  ....   The rest is not important.

It appears that this is the end of the routine.  Let's look for 1 more

DOS call.

>G 0:84

Number of instructions executed: 0000:0011

AX=2522  BX=0005  CX=0000  DX=0288  SP=0729  BP=0085  SI=0084  DI=0086

DS=13ED  ES=5B79  SS=13ED  CS=13ED  IP=0495   NV UP EI NT PL NZ NA PE NC

13ED:0495  CD21         INT     21

Yes, we are back in the resident portion of COMMAND.COM, and setting the

INT 22 vector (terminate address) to point to COMMAND.COM.  That's all

for this routine!

That may seem a little complicated, but consider that the alternative is

to try to get DEBUG to trace COMMAND.COM, or to unassemble COMMAND.COM and

try to figure out what is going on.  In that light, this isn't at all bad.

Debugging a resident program as it runs

---------------------------------------

Suppose, for example, you wanted an additional click when you pressed the

spacebar, and wrote the following resident program:

fun     segment

assume  cs:fun,ds:fun,es:fun,ss:fun

org     100h

clicker:

jmp     start                   ;jump to start of program

;

;  the resident interrupt handler

;

int9here:

push    ax                      ;save ax register

in      al,60h                  ;get data from keyboard port

cmp     al,38h                  ;this is a mistake.  This should

;be a compare to 39h, not 38h.

jne     dooldint                ;if not equal, keep going

in      al,61h                  ;get 8255 port B

Page 120

push    ax                      ;save it

and     al,0feh                 ;reset bit 0

out     61h,al                  ;and get direct speaker control

push    cx                      ;save count register

mov     ah,20                   ;cound 20 decimal loops

soundloop:

xor     al,2                    ;flip bit 1 to toggle speaker

out     61h,al                  ;speaker on/off

mov     cx,50h                  ;a time delay

here:   loop    here                    ;pause a moment

dec     ah                      ;count this wave

jnz     soundloop               ;if more, do them

pop     cx                      ;restore cx

pop     ax                      ;restore al=8255 port B

out     61h,al                  ;restore the port

dooldint:

pop     ax                      ;restore ax

jmp     dword ptr [oldint9]     ;and go do the original interrupt

;

;  the data area

;

oldint9:        dw      0               ;the offset of the BIOS int 9

oldint9seg:     dw      0               ;the segment of the BIOS int 9

;

;  the setup program

;

start:  xor     ax,ax                   ;set ax to 0

mov     es,ax                   ;es = 0

cli                             ;no interrupts while changing vectors

mov     ax,word ptr es:[24h]    ;get the offset for int 9

mov     word ptr oldint9,ax     ;save it for the resident

mov     ax,word ptr es:[26h]    ;get the segment for int 9

mov     word ptr oldint9seg,ax  ;save it for the resident

mov     word ptr es:[24h],offset int9here     ;set new int 9 offset

mov     word ptr es:[26h],cs    ;set new int 9 segment

sti                             ;interrupts are OK now

mov     dx,offset start         ;the last location to leave resident

int     27h                     ;terminate and remain resident

fun     ends

end     clicker

;

;  Can you spot the hidden mistake?

;

The problem is that, when you use it, not only doesn't it click, it

crashes the system at the first keypress!  Something is wrong here.

Using VMDEBUG to run the program, however, makes things easy to find.

The output below was taken from a VMDEBUG session, where the above

program was debugged.  The values of the segment registers might change,

but that will not change the results.  Do not type the comments on the

command lines.

Page 121

>ZD             - load DOS.

>N clicker.com  - name of the program to load.

>L              - load it.

>T              - trace the installation, to make sure it is ok.

AX=0000  BX=0000  CX=0053  DX=0000  SP=FFFE  BP=0000  SI=0000  DI=0000

DS=152A  ES=152A  SS=152A  CS=152A  IP=0100   NV UP EI NT PL NZ NA PO NC

152A:0100  EB2C         JMP     012E

>T              - keep tracing.

AX=0000  BX=0000  CX=0053  DX=0000  SP=FFFE  BP=0000  SI=0000  DI=0000

DS=152A  ES=152A  SS=152A  CS=152A  IP=012E   NV UP EI NT PL NZ NA PO NC

152A:012E  33C0         XOR     AX,AX

>T              - keep tracing.

AX=0000  BX=0000  CX=0053  DX=0000  SP=FFFE  BP=0000  SI=0000  DI=0000

DS=152A  ES=152A  SS=152A  CS=152A  IP=0130   NV UP EI NT PL NZ NA PO NC

152A:0130  8EC0         MOV     ES,AX

>T              - keep tracing.

AX=0000  BX=0000  CX=0053  DX=0000  SP=FFFE  BP=0000  SI=0000  DI=0000

DS=152A  ES=0000  SS=152A  CS=152A  IP=0132   NV UP EI NT PL NZ NA PO NC

152A:0132  FA           CLI

>T              - keep tracing.

AX=0000  BX=0000  CX=0053  DX=0000  SP=FFFE  BP=0000  SI=0000  DI=0000

DS=152A  ES=0000  SS=152A  CS=152A  IP=0133   NV UP DI NT PL NZ NA PO NC

152A:0133  26           ES:

152A:0134  A12400       MOV     AX,[0024]                       0000:0024=C149

>T              - keep tracing.

AX=C149  BX=0000  CX=0053  DX=0000  SP=FFFE  BP=0000  SI=0000  DI=0000

DS=152A  ES=0000  SS=152A  CS=152A  IP=0137   NV UP DI NT PL NZ NA PO NC

152A:0137  A32A01       MOV     [012A],AX                       152A:012A=0000

>T              - keep tracing.

AX=C149  BX=0000  CX=0053  DX=0000  SP=FFFE  BP=0000  SI=0000  DI=0000

DS=152A  ES=0000  SS=152A  CS=152A  IP=013A   NV UP DI NT PL NZ NA PO NC

152A:013A  26           ES:

152A:013B  A12600       MOV     AX,[0026]                       0000:0026=F000

>T              - keep tracing.

AX=F000  BX=0000  CX=0053  DX=0000  SP=FFFE  BP=0000  SI=0000  DI=0000

DS=152A  ES=0000  SS=152A  CS=152A  IP=013E   NV UP DI NT PL NZ NA PO NC

152A:013E  A32C01       MOV     [012C],AX                       152A:012C=0000

>T              - keep tracing.

AX=F000  BX=0000  CX=0053  DX=0000  SP=FFFE  BP=0000  SI=0000  DI=0000

DS=152A  ES=0000  SS=152A  CS=152A  IP=0141   NV UP DI NT PL NZ NA PO NC

152A:0141  26           ES:

152A:0142  C70624000301 MOV     WORD PTR [0024],0103            0000:0024=C149

Page 122

>T              - keep tracing.

AX=F000  BX=0000  CX=0053  DX=0000  SP=FFFE  BP=0000  SI=0000  DI=0000

DS=152A  ES=0000  SS=152A  CS=152A  IP=0148   NV UP DI NT PL NZ NA PO NC

152A:0148  26           ES:

152A:0149  8C0E2600     MOV     [0026],CS                       0000:0026=F000

>T              - keep tracing.

AX=F000  BX=0000  CX=0053  DX=0000  SP=FFFE  BP=0000  SI=0000  DI=0000

DS=152A  ES=0000  SS=152A  CS=152A  IP=014D   NV UP DI NT PL NZ NA PO NC

152A:014D  FB           STI

>T              - keep tracing.

AX=F000  BX=0000  CX=0053  DX=0000  SP=FFFE  BP=0000  SI=0000  DI=0000

DS=152A  ES=0000  SS=152A  CS=152A  IP=014E   NV UP EI NT PL NZ NA PO NC

152A:014E  BA2E01       MOV     DX,012E

>T              - keep tracing.

AX=F000  BX=0000  CX=0053  DX=012E  SP=FFFE  BP=0000  SI=0000  DI=0000

DS=152A  ES=0000  SS=152A  CS=152A  IP=0151   NV UP EI NT PL NZ NA PO NC

152A:0151  CD27         INT     27

Everything looks good in the installation, so let's see what it does

when it receives a keystroke.

>G 0:24         - break on the next reference to the INT 9 vector, whether

it is from hardware or software.

After this has executed for a few moments, press the space bar.

Number of instructions executed: 0000:32BD

AX=89DB  BX=0014  CX=19E6  DX=AFA3  SP=0723  BP=0000  SI=0734  DI=0100

DS=5B79  ES=13ED  SS=13ED  CS=13ED  IP=0481   NV UP EI NT NG NZ NA PE NC

13ED:0481  E2F8         LOOP    047B

Note that the reason for the interrupt is not displayed.

>T              - give the interrupt another chance.

AX=89DB  BX=0014  CX=19E6  DX=AFA3  SP=0723  BP=0000  SI=0734  DI=0100

DS=5B79  ES=13ED  SS=13ED  CS=13ED  IP=0481   NV UP EI NT NG NZ NA PE NC

13ED:0481  E2F8         LOOP    047B

Doing hardware interrupt # 09

Now we can see what is happening!

>T        - start tracing our interrupt handler.

AX=89DB  BX=0014  CX=19E6  DX=AFA3  SP=0723  BP=0000  SI=0734  DI=0100

DS=5B79  ES=13ED  SS=13ED  CS=152A  IP=0103   NV UP DI NT NG NZ NA PE NC

152A:0103  50           PUSH    AX

Page 123

>T        - keep tracing.

AX=89DB  BX=0014  CX=19E6  DX=AFA3  SP=0721  BP=0000  SI=0734  DI=0100

DS=5B79  ES=13ED  SS=13ED  CS=152A  IP=0104   NV UP DI NT NG NZ NA PE NC

152A:0104  E460         IN      AL,60

>T        - keep tracing.

AX=8939  BX=0014  CX=19E6  DX=AFA3  SP=0721  BP=0000  SI=0734  DI=0100

DS=5B79  ES=13ED  SS=13ED  CS=152A  IP=0106   NV UP DI NT NG NZ NA PE NC

152A:0106  3C38         CMP     AL,38

Here we see our first mistake.  The scancode in AL that represents the

spacebar is a 39, but we have assembled a 38.

>T        - keep tracing.

AX=8939  BX=0014  CX=19E6  DX=AFA3  SP=0721  BP=0000  SI=0734  DI=0100

DS=5B79  ES=13ED  SS=13ED  CS=152A  IP=0108   NV UP DI NT PL NZ NA PO NC

152A:0108  751B         JNZ     0125

>T        - keep tracing.

AX=8939  BX=0014  CX=19E6  DX=AFA3  SP=0721  BP=0000  SI=0734  DI=0100

DS=5B79  ES=13ED  SS=13ED  CS=152A  IP=0125   NV UP DI NT PL NZ NA PO NC

152A:0125  58           POP     AX

>T        - keep tracing.

AX=89DB  BX=0014  CX=19E6  DX=AFA3  SP=0723  BP=0000  SI=0734  DI=0100

DS=5B79  ES=13ED  SS=13ED  CS=152A  IP=0126   NV UP DI NT PL NZ NA PO NC

152A:0126  FF2E2A01     JMPF    WORD PTR [012A]                 5B79:012A=125B

The watchful reader will notice something suspicious here.

>T        - keep tracing.

AX=89DB  BX=0014  CX=19E6  DX=AFA3  SP=0723  BP=0000  SI=0734  DI=0100

DS=5B79  ES=13ED  SS=13ED  CS=8CFC  IP=125B   NV UP DI NT PL NZ NA PO NC

8CFC:125B  FFB70202     PUSH    WORD PTR [BX+0202]              5B79:0216=3E83

It is now rather obvious what the other problem was.  The memory referenced

for the JMPF was relative to the DS register, but DS was never set by the

interrupt handler.  What should have been done was to use a CS: segment

override, so that the memory used for the address would be in the correct

segment.  In the original assembly code, the line should have read:

jmp     dword ptr cs:[oldint9]  ;and go do the original interrupt

instead of:

jmp     dword ptr [oldint9]     ;and go do the original interrupt

It is problems like this that kill ordinary debuggers.  When a mistake

is in an interrupt handler, that interrupt is no longer available.  Even if

the mistake does not crash the system, as this one does, a dead keyboard

will prevent any normal debugger from working.

Page 124

Debugging a device driver as it runs

------------------------------------

The trick to working with a device driver is not debugging it, but

finding it.  Since device drivers are loaded by DOS at boot time, they

are installed as part of the operating system, and cannot be loaded for

execution under a normal debugger.  Also, if the faulty code is in the

initalization code of the device driver, you cannot boot the system with

the device driver installed.

VMDEBUG, however, allows you to boot a machine virtually, so that the

device driver will be installed and executed where it cannot crash the

operating system.   Even so, finding the device driver as it is loading

(or after it has loaded) can present a problem.  An example device driver

is given below, with examples of various ways of finding the device

driver, and an explanation of when each method would be useful.

Booting DOS virtually

---------------------

If the device driver cannot be installed during the boot of the normal

system, it will be necessary to boot the system virtually under VMDEBUG

to load and debug the device driver.  This is easily accomplished by

using one of lesser-known BIOS functions, the bootstrap loader (INT 19).

When VMDEBUG loads a virtual DOS (via the ZD command), it checks the size

of virtual memory available, and if it is sufficient to hold a virtual DOS

(the requirements are discussed with the explanation of the ZD command in

chapter 6), it sets the program to an INT 20 (terminate program)

instruction.  To have VMDEBUG boot the computer instead, simply use the

commands:

A          - start the assembler

INT 19     - assemble the interrupt instruction

- a blank like to terminate the assembler

to enter an INT 19 instruction, and use the G command.

If there is not sufficient virtual memory (again, see the discussion

of the ZD command in chapter 6), VMDEBUG will automatically set the

instruction to an INT 19, so that you can boot the system in a smaller

virtual memory size.

In either case, you should have a boot floppy in the A drive with the

device driver to be debugged included in the CONFIG.SYS file as a DEVICE=

command.  (Note:  If you have a system, such as a Zenith Z150, that will boot

directly from a hard disk without first trying to boot from a floppy, you

will save time by putting the DEVICE= command in the CONFIG.SYS on your hard

disk and doing a virtual boot from the hard disk.  Remember to remove the

device driver from CONFIG.SYS before trying to reboot the system.  Also,

the 1.2M floppy drives on ATs and compatibles will sense the lack of a

disk in the drive, and will immediately boot from the hard disk.)

Page 125

Finding the device driver

-------------------------

The example device driver presented below, NULLDEV.ASM, shows 4 methods

of finding the device driver.

;

;  This file defines a character device driver that does nothing.

;

;  This device driver does not need an .EXE header, so use EXE2BIN after

;  compiling.

;

RETF    macro

db      0cbh            ;force a RET FAR without a PROC FAR

endm

fun     segment byte

assume  cs:fun,ds:fun,es:fun,ss:fun

;

;  Set up equates so I don't have to refer to absolute offsets

;

commandloc      equ     2       ;location of command byte in request header

statusloc       equ     3       ;location of status word in request header

returnbyteloc   equ     13      ;location of the returned byte for code 5

residentofs     equ     14      ;location of the returned resident offset

residentseg     equ     16      ;location of the returned resident segment

errorstatus     equ     8103h   ;Return code for an error of unknown command

;bit 15 (8000h) = error flag

;bit 8 (0100h) = done flag

;bits 0-7 = error code

donestatus      equ     0100h   ;as above, this simply sets the done flag

;

;  Here, I'm going to pull a sneaky trick and fake labels in another

;  segment.

;

org     0bch

at0bch  label   word

org     0beh

at0beh  label   word

;

;  The origin of a device driver is at 0, not 100H.  Remember, there is no

;  PSP associated with a device.

;

org     0

;

;  Here is the device header

;

nulldev:

dw      -1              ;ofs of link pointer

dw      -1              ;seg of link pointer

dw      8000h           ;attribute, character device

dw      strategy        ;pointer to the strategy routine

dw      interrupt       ;pointer to the interrupt routine

db      'ADEVICE '      ;8-byte device name, upper case so

;that it can be found

Page 126

;

;  Here starts the local data

;

packetofs:      dw      0               ;offset

packetseg:      dw      0               ;segment

oldint2fofs:    dw      0               ;old int 2fh offset

oldint2fseg:    dw      0               ;old int 2fh segment

ss_save:        dw      0               ;save area for SS

sp_save:        dw      0               ;save area for SP

message_1:      db      0dh,0ah

db      'This is some initalization message.',0dh,0ah,'$'

message_2:      db      'And this is another initalization message.'

db      0dh,0ah,'$'

dw      128 dup (0)     ;a local stack

localstack      label   word            ;pointer to the stack

functiontable:  dw      initalize       ;code 0, init

dw      exit_ok         ;code 1, media check

dw      exit_ok         ;code 2, build bpb

dw      fake_input      ;code 3, ioctl input control string

dw      fake_input      ;code 4, input (read)

dw      fake_nowait     ;code 5, nondestructive no-wait input

dw      do_status       ;code 6, input status

dw      exit_ok         ;code 7, input flush

dw      fake_output     ;code 8, output (write)

dw      fake_output     ;code 9, output (write), with verify

dw      do_status       ;code 10, output status

dw      exit_ok         ;code 11, output flush

dw      fake_output     ;code 12, ioctl output control string

dw      exit_ok         ;code 13, device open  (DOS 3.x)

dw      exit_ok         ;code 14, device close  (DOS 3.x)

dw      exit_ok         ;code 15, removable media  (DOS 3.x)

dw      unknown         ;code 16, undefined

dw      unknown         ;code 17, undefined

dw      unknown         ;code 18, undefined

dw      exit_ok         ;code 19, generic ioctl request

;  (DOS 3.2)

dw      unknown         ;code 20, undefined

dw      unknown         ;code 21, undefined

dw      unknown         ;code 22, undefined

dw      exit_ok         ;code 23, get logical device (DOS 3.2)

dw      exit_ok         ;code 24, set logical device (DOS 3.2)

;

;  Here is the strategy routine

;

strategy:

;

;       db      0f1h                    ;This is an undefined opcode

;

;                       Since VMDEBUG will stop when it encounters an

;                       undefined opcode, this will halt VMDEBUG at the start

;                       of the strategy routine.  This is one way of finding

;                       the device driver.

;

;                       Note that this will stop VMDEBUG at EVERY call to

;                       the device driver strategy routine!

;

Page 127

mov     word ptr cs:packetofs,bx        ;save packet offset

mov     word ptr cs:packetseg,es        ;save packet segment

RETF                                    ;and quit

;

;  Note that a P command used when CS:IP point to the RETF instruction will

;  stop VMDEBUG at the PUSH AX instruction below, the start of the interrupt

;  routine.

;

;

;  Here is the interrupt routine

;

interrupt:

push    ax                      ;save AX

push    bx                      ;and BX

push    dx                      ;and DX

push    si                      ;and SI

push    ds                      ;and DS

push    es                      ;and ES

les     bx,dword ptr cs:packetofs        ;get pointer to the packet

mov     al,byte ptr es:[bx+commandloc]   ;get the command

cmp     al,24                   ;legal command?

ja      unknown                 ;nope, skip it

cbw                             ;make into word

mov     si,offset functiontable ;base address of jump table

add     si,ax                   ;base + function

add     si,ax                   ;base + 2*function (word address)

jmp     word ptr cs:[si]        ;jump to the function

unknown:

mov     word ptr es:[bx+statusloc],errorstatus

;status is error, done, error = unknown command

jmp     short and_quit          ;go quit

fake_nowait:

mov     byte ptr es:[bx+returnbyteloc],0

;return a 0 for character input

fake_input:

fake_output:

do_status:

exit_ok:

mov     word ptr es:[bx+statusloc],donestatus   ;set to done

and_quit:

pop     es                      ;restore ES

pop     ds                      ;and DS

pop     si                      ;and SI

pop     dx                      ;and DX

pop     bx                      ;and BX

pop     ax                      ;and AX

RETF                            ;return to calling routine

;

;  If the device driver can be successfully installed but has problems with

;  its operation, you can use the multiplex interrupt (INT 2F) to have the

;  device driver tell you where the device driver is.

;

Page 128

int2fhere:

cmp     ah,0f0h                 ;Define ourselves as multiplex 0F0h

;

;                        Remember to test and see if the multiplex is

;                        already in use if you are going to use the

;                        multiplex interrupt in a program of yours.

;                        See the DOS Technical Reference for details.

;

jne     continue_int_2f         ;not us, let it pass

cmp     al,0                    ;Function = 0, get installed state

;STANDARD FUNCTION REQUEST

jne     address_check           ;no, see if address check

mov     al,0ffh                 ;say we are installed

;STANDARD RESPONSE

iret                            ;and return to caller

address_check:

cmp     al,1                    ;Function = 1, get segment address

;THIS IS OUR DEFINITION

jne     just_return             ;if not, just do nothing

mov     ax,cs                   ;return our segment in AX

just_return:

iret                            ;and return

continue_int_2f:

jmp     dword ptr cs:[oldint2fofs]      ;continue the interrupt chain

;

;  Here is the Initilization routine.  This is called only ONCE per device.

;

initalize:

;

;        db     0f1h                    ;an undefined opcode

;

;                        This undefined opcode serves the same purpose as the

;                        one above, but will only stop VMDEBUG during the

;                        initalization routine, rather than at every call to

;                        the device driver.

;

cli                             ;do not disturb

mov     word ptr cs:ss_save,ss  ;save current SS

mov     word ptr cs:sp_save,sp  ;and SP

mov     ax,cs                   ;move CS

mov     ss,ax                   ;to SS

mov     sp,offset localstack    ;and set SP, as well

sti                             ;interrupts are OK again

;

mov     ds,ax                   ;set DS to the current segment

mov     dx,offset message_1     ;DS:DX = a string to output

mov     ah,9                    ;function = write string

int     21h                     ;call DOS

mov     dx,offset message_2     ;another string

mov     ah,9                    ;fn = write string

int     21h                     ;call DOS

Page 129

;

;  This is yet another way to find the device driver.  If the initilization

;  code works correctly, interrupt VMDEBUG when you see the 1st string being

;  displayed, and put a breakpoint in the INT 21 vector to stop the next

;  DOS call.  This will break VMDEBUG when it tries to display the 2nd

;  string, and the device driver has then been found in memory.

;

xor     ax,ax                   ;AX = 0

mov     ds,ax                   ;DS = 0

mov     ax,word ptr [at0bch]            ;get the offset for INT 2f

mov     word ptr cs:oldint2fofs,ax      ;save old offset

mov     ax,word ptr [at0beh]            ;get the segment for INT 2f

mov     word ptr cs:oldint2fseg,ax      ;save old segment

mov     word ptr [at0bch],offset int2fhere ;new offset

mov     word ptr [at0beh],cs            ;new segment

;

mov     word ptr es:[bx+residentofs],offset initalize

;last resident byte+1

mov     word ptr es:[bx+residentseg],cs ;resident segment

;

;  Note that the initalization code is thrown away after it is used

;

;  Also note:  According to the MS-DOS (2.11) documentation that accompanies

;  the Zenith Z150, the resident address is also returned in DS:DX.

;  The IBM PC-DOS Technical Reference Manual, however, does not say that

;  the resident address should be returned in DS:DX, but does say that

;  it is the responsibility of the device driver to preserve the machine

;  state.  Since the above code works under MS-DOS 2.11, the IBM scheme is

;  followed.

;

mov     ss,word ptr cs:ss_save  ;restore SS

mov     sp,word ptr cs:sp_save  ;and SP

jmp     exit_ok                 ;and quit

;

;  And that's all!

;

fun     ends

end     nulldev

If the device driver is so totally non-functional that DOS cannot/will

not even call its strategy routine for initilization, you must find it

by watching DOS use INT 21's to open and load the file.  To make this

easier, you can put a device driver that does nothing but print a message

on initilization in the CONFIG.SYS file before the defective device driver.

When the message appears from the 1st device driver, interrupt VMDEBUG

and place a breakpoint on the INT 21 vector.  As the next device is

loaded, you can find it and determine just where things are going wrong.

At the next level, DOS can call the strategy routine, but cannot do

anything else.  In this case, you need to trap the device driver at the

start of the strategy routine.  The easiest way to do so is by placing

Page 130

an undefined opcode in the instruction stream of the device driver.  In

the example device driver above, the commented-out DB 0F1H is such an

instruction.  When VMDEBUG encounters it, you will be returned to the

VMDEBUG command prompt.  At this time, use the command R IP IP+1 to

increment over the undefined opcode (note: remember that this will

destroy the saved information for the "where-was-I?" (the W) command),

and then trace the operation of your device driver.  When the strategy

routine is functional, is it is a good idea to re-compile the device

driver and run the new version under VMDEBUG, as the undefined opcode

will halt VMDEBUG at every call to the device driver, which is at best

annoying.

Moving furthur up the ladder to a functioning device driver, if the

strategy routine is working but the initalization code is not, the easiest

way to find the appropriate code is an undefined opcode at the start of

the initalization code (as in the above example).  This has the furthur

advantage that it will halt VMDEBUG only once (when the INIT call is passed

to the device driver), instead of at every call to the device driver (as in

the case of the undefined opcode in the strategy routine).  Remember to

step over the undefined opcode with a R IP IP+1 command to VMDEBUG.

If both the strategy and initalization routines work, finding the

device driver becomes much easier.  Among the available options are

halting VMDEBUG as strings are being displayed from the device driver

and looking for the next INT 21 call, or even having the initalization

code display the segment address of the device driver.  If you only need

the address of one particular routine, a carefully placed undefined

opcode will pinpoint the desired spot.

If it is necessary to find the device driver dynamically as some other

piece of code is running, the use of the multiplex interrupt (INT 2F) as

shown in the above example will simplify the task (you cannot just look

at the segment of the INT 2F vector, as there can be many programs or

devices in the chain, and your program cannot be guaranteed to be first).

The procedure is to use INT 2F with AX=F000 first (AH = multiplex = F0 in

this example, and AL = 00 = function request 0, is this multiplex defined?)

to see if your multiplex is defined (AL returns FF), and then call INT 2F

with AX=F001 (multiplex = F0 again, function request = 01 = get segment

address.  This function request is our definition) to return the segment

address of the device driver in AX.

If your device driver uses some interrupt vector that is not shared,

a simple method of finding the device driver is to look at the segment

address stored in the interrupt vector table for that particular interrupt.

Remember that the starting offset of a device driver is 0, not 100.

Page 131

Working with copy protected software

------------------------------------

This section, unlike the previous sections, is very high-level.  There

are two reasons for this: 1) There are many such schemes, all of which are

unique, and 2) There are reams of lawyers just waiting for me to drop

"secret" internal information on someone's protection schemes.

Disclaimer:  This material is presented only as an intellectual exercise.

Any use of this material may be a violation of federal copyright laws, which

is a felony.  Neither the author nor DDI will be responsible for any

violations.

Take a general protection scheme, called "lockproof" for want of a

better name.  Like almost all such schemes, "lockproof" requires the

insertion of a key diskette in the A: drive to bring up the program.

It is very likely that VMDEBUG will allow you to remove the protection

from the program by tracing until the actual program is decoded and

then making a copy.

The first step is to try to boot the program under VMDEBUG without

tracing the protection program at all.  This may seem like a waste of

time but will allow you to see if the program can be brought up at all

under VMDEBUG.  Some older protection schemes, mostly in use before

the appearance of AT class machines and Turbo-XT clones, used the

technique of timing events on the disk to see if this was the original

disk.  As they did not work with the faster machines, they seem to have

dropped by the wayside.

Another reason that the program may not boot under VMDEBUG is that it

was loaded directly from the VMDEBUG prompt.  As VMDEBUG does not set the

program name in the environment space (for DOS 3.0 and up), the program

may be checking that name to make sure that it is running, rather than

some debugger.  If you suspect this, either change the name in the

environment space (see the discussion of the ZD command for some

suggestions), or run the program from the command prompt of the virtual

DOS.

The next step is to try to bring up the program, and trace everything

you can.  This will take a while, but will be necessary to find the

location in the program at which the real program has been decoded.

The first few dozen instructions may appear to be garbage, because they

are trying to confuse the novice who is trying to trace the program.  They

also usually set the stage for destroying several interrupt vectors later

in the program, to prevent the programs being watched by normal debuggers.

The normal debuggers, such as DEBUG, depend on the INT 3 instruction

(also called the 1-byte interrupt) for program breakpoints, and the

INT 1 vector, which is used by the Trap interrupt (the trap interrupt is

controlled by the Trap flag in the flags register.  This flag is set by

either the POPF or IRET instructions and will cause an interrupt after

1 instruction has executed).  Hardware assisted debuggers will also use

the Non-Maskable Interrupt (which uses the INT 2 vector) to halt the program

with a "breakout" switch.

Page 132

One sneaky way that a program can destroy interrupt vectors is through

the use of the stack.  While it may be obvious that SS=0, SP=10 will

destroy the INT 3 vector in the next PUSH, it is not that obvious that

SS=FF00, SP=1010 will accomplish the same thing.  When the address is

calculated, the carry out of the high bit is ignored, and the 20-bit

address is 0:10, where the INT 3 will be destroyed.  This technique also

has a side "benefit" for the protection program, in that a breakpoint or

trace interrupt will destroy more interrupt vectors.

A common method of destroying interrupt vectors is to set the stack to

ROM, or some place where a PUSH (such as values from an interrupt) will

destroy something critical, and then using the interrupt vectors as a data

area for a routine.  This will prevent interrupts from occurring (or at

least returning from interrupts), and randomize the routines pointed to.

Playing around with the interrupt vectors will not affect VMDEBUG,

however, as the vectors that are changed are in virtual memory, which

VMDEBUG does not use for its operation.

Another technique used by protection software is self-modifying code.

If you can unassemble a program, you can modify the program to do whatever

you want.  However, if the program does not exist in readable form until

just before that section of the program is executed, the program must be

traced through the decode process to see what happens next.

If the decoding logic is something like "rotate byte left, xor byte with

sum of bytes, add byte to sum, rotate byte left, trash interrupt vectors,

do next byte" using the interrupt vector table for scratch memory, it may

be difficult or impossible to have a program like DEBUG trace through the

code, and decoding 5,000 bytes by hand is not a trivial job.  Also, if the

decode logic is directly before the code to be executed, it may not be

possible to place a breakpoint, as the self-modifying code will destroy the

INT 3 instruction, and then improperly decode the rest of the routine, as

the INT 3 instruction was not the expected value.

VMDEBUG, however, has no problems with such code.  It can keep track of

the results through the 5,000 bytes, and can place breakpoints that will

not interfere with the self-modifying code.

When the protection program is executing, it will sometimes take great

pains to be executing code somewhere other than the place you expect.

The most common method of this is to use either the clock interrupt or

the trap interrupt.  As both of these will suddenly cause a change in

CS and IP with no warning, no user input, and no tell-tale INT instruction

in the instruction stream, they can be very difficult to track down.

The clock interrupt can occur "between" two instructions traced by a

normal debugger, and the routine pointed to will quickly modify both the

trap and breakpoint interrupts, so that the debugger will never regain

control of the program.

The trap interrupt can be used by a program so that if a debugger is

using the trap interrupt for single stepping, the INT 1 vector will point

to the debugger and not the protection program, so that the protection

routine is never called and the program never runs.  Another trick that

can be used is to have the routine pointed to by the trap interrupt

immediately check the address pointed to by the trap interrupt.

Page 133

The INTO (interrupt on overflow) instruction can also be used, but since

it is in the instruction stream, its function must be hidden by repeatedly

using it in a long arithmetic or logical instruction stream.  Its usefulness

in hiding code would consist of constantly changing the address pointed to

by the INTO vector (the INT 4 vector), and hoping that the user misses the

change to the overflow flag.

Once again, VMDEBUG has no problems with such programs, as the vectors

and routines used are in virtual memory, which is not used internally to

VMDEBUG.

Eventually, the protection software will have to access the disk.  It may

be through DOS calls (through function calls to INT 21, or by using the

absolute disk I/O routines at INT 25 and INT 26), which will in turn use

the INT 13 BIOS routines, or it may call the INT 13 BIOS routines directly.

It is very unlikely, but possible, that the protection program will call the

ROM BIOS routines directly.  One thing to watch out for is a protection

program re-directing the DOS or BIOS interrupts into itself, and doing more

processing before actually calling the BIOS.  This could be used to hide code

from someone trying to trace the program, as they would not think it necessary

(or possible) to trace a ROM call.

When a program is actually going to do disk I/O, VMDEBUG should not be

used in trace mode, as I/O requires watching return interrupts from the

disk.  The best solution is to set a breakpoint where the routine would

return, and examine the contents.

One thing to be careful about is the status of the floppy motor drive byte.

This byte, at 40:3F, uses the lower 4 bits to reflect the motor on status of

floppy drives A-D.  When using the T command, the real BIOS can turn off the

disk motors when the timer byte expires.  The virtual copy of the status byte,

however, still shows the motor as running, and the BIOS will then try to

access a floppy that is not running.  The best solution is to turn on the

motor via the O command (described in chapter 6), although resetting the

drive byte and letting the BIOS turn on the motor again can be used.

When the protection program is satisfied that all is well, it will decode

the actual program.  If this is an .EXE file, it can be identified by the

signature 4D5A in the first two bytes of the header block.  If it is a

.COM file, it may be more difficult, but you should be able to find it

decoding a very large block of code.  Another way of trapping it would be

to watch the protection program set up a program segment prefix (PSP) for

the protected program, either by function 26 of INT 21, or by copying and

modifying its own PSP.

When you catch the program with the decoded program (and if relocation

has not yet taken place in an .EXE file), you can then save the decoded

program.  Set CS:IP to point to the start of the program (for an .EXE file,

this includes the header), and BX:CX to the length of the file to be

written.  If this is a small program remember to be sure that BX is 0, or

a very large program may be written.  Then use the command C filename to

write the program to disk, remembering that VMDEBUG cannot write an .EXE

file directly, so that another extension must be used and the file renamed

with an .EXE extension at the DOS prompt.

Page 134

It is a good idea to watch for the protection program setting "unusual"

interrupt vectors (i.e. not the familiar DOS interrupts).  I have seen one

program that called an unusual interrupt set by the protection program

(call it INT E0).  The purpose was not to do anything, but rather to determine

that it was the protection program that loaded the file, and not DOS.  If

the protection program was the loader, the routine existed, a return from the

interrupt occurred, and the program continued.  If DOS did the loading, the

interrupt pointed to random memory (or possibly 0:0), and the interrupt

crashed the system.  If you have strange behavior in an unprotected

program, look for an interrupt scheme like this.

Page 135

Chapter 9

Compatibility problems and solutions

------------------------------------

The HP Vectra

-------------

It has been found that the HP Vectra (an AT compatible) has a memory

mapping that is incompatible with normal memory map for a PC compatible.

In this case, the particular mapping set aside a 4K block of memory at the

top of the 640K RAM space (what should be DOS memory) for the use of BIOS.

VMDEBUG recognizes and properly maps partial RAM segments, so that the

segment containing the BIOS RAM will be available under VMDEBUG.

However, HP VECTRA users must use one additional step when starting

VMDEBUG--they must use the ZP (Patch DOS) command.  The BIOS RAM is

accessed by interrupt vector 6Fh, and this vector must be changed in

the virtual DOS copy loaded by the ZD command.  To facilitate this, the

ZP command may be used to patch up to 20 locations at a time, which should

be more than enough for this and any future situations of this type.

For interrupt vectory 6Fh, the location that must be patched is 0:1BF

(0:6F*4+3), the high-byte of the interrupt segment.  Any similiar patches

for other incompatibilities should again be on the high byte of the segment.

A problem to watch out for with machines such as the HP VECTRA and the

Zenith PC's (which also set up a special RAM area for BIOS, but in this case

it is not in the DOS memory, and does not cause a problem) is contention for

the BIOS RAM between VMDEBUG and the "real" machine.

As both the "real" operating system and the operating system running

under VMDEBUG are handling the clock and keyboard interrupts, and are

using the video screen, both the "real" and interpreted BIOS are using

the BIOS RAM area at the same time.  Any values written there by one

source may be over-written by the other source, and when the original

source returns later it will find a modified value, which can cause

unpredictable results.

Bus mice

--------

Bus mice, such as the Microsoft bus mouse and the Logitech bus mouse

need special consideration when running under VMDEBUG.  Like the clock

interrupt, they generate periodic interrupts, which are passed into

VMDEBUG so that they may be recognized and handled by the system being

interpreted.  For a real processor the overhead involved with the mouse

interrupt can be ignored, but under VMDEBUG this overhead is excessive,

taking most of the available processing power, which drags the apparent

speed of VMDEBUG to a virtual standstill.

Page 136

There are two ways of solving this problem.  The first is to run VMDEBUG

without the mouse.  Removing the mouse driver from the CONFIG.SYS or

AUTOEXEC.BAT file is sufficient, since the mouse hardware does not generate

interrupts if it is not initalized.

The second solution is to have VMDEBUG slow down the interrupt frequency.

The ZT command may be used to divide other interrupts than the clock, with

only one out of a specified number of interrupts being passed into VMDEBUG.

Note that this requires you to know the IRQ (interrupt request) number being

used by your hardware.

One problem that can arise is that the computer receives an interrupt

that it was not expecting.  For example, if the mouse hardware is

initalized under VMDEBUG, the interrupt handler will be installed in

VMDEBUGs virtual memory, and the interrupts will be passed into VMDEBUG

for processing.  However, when you return to the VMDEBUG command level,

VMDEBUG returns the interrupt vectors to the system.  If the system vector

does not point to a interrupt handler (even a null interrupt handler should

be sufficient), the next interrupt to come come along can crash the system.

Thus, the system should be set up in such a way that all IRQs are taken

care of, such as installing the mouse driver before starting VMDEBUG.

RAM disks

---------

This it not really a compatibility problem, but a common problem that

is misunderstood by many people.  When using an AT or compatible with a

RAM disk in extended memory, the RAM disk cannot be accessed by a program

running under VMDEBUG!  Trying to do so will result in VMDEBUG stopping on

an undefined opcode (reported as DB 0F).

If you are doing a virtual boot of DOS (or another operating system),

make sure that the system parameters and device drivers (such as those

in CONFIG.SYS) do not reference extended memory, such as using the device

driver VDISK.SYS with the /E option.

The reason for this is that extended memory is only accessable in

protected mode, not the real mode of DOS, and the 80286 (and 80386)

protected mode instructions are not implemented in VMDEBUG.  Thus, when

the ROM BIOS tries to do a mode switch to protected virtual mode, VMDEBUG

stops with an undefined opcode.

Known Quirks

------------

A program of this nature is bound to have some unusual behavior under

some circumstances, due to timing problems if nothing else.  These have been

fixed when possible, and described here where no fix is available.  I have

found one occurrence of a program that does not run due to timing problems

(a program to read a hardware clock), but that is an extreme case mentioned

below.

Page 137

When DOS is loaded, all BIOS parameters are loaded as well.  As a result,

if the cursor is not where it was in when DOS was loaded, executing DOS

results in the cursor re-appearing where it was when DOS was loaded.

The easiest way to handle this is to simply let the screen scroll, so the

cursor is in the bottom left corner after every instruction.  This only

applies when screen swapping is not enabled, as swapping screens also

swaps cursor positions.

A fairly common failing of software is that programs expect to be much

faster than the I/O hardware.  This expresses itself in several ways,

the most noticeable of which is timing loops.  The screen display that lasts

just a moment on a normal computer is hideously long under VMDEBUG.  Or, the

long pauses as the BIOS waits for the floppy motor to start up, when in

reality the motor has been spinning for 2 minutes.

Another problem has been seen in a program to set the system time and date

from a clock-calender card.  There is a port on the card (status port) that

contains a 1 if the information has changed from the last port read.  The

program in question reads information, does some minimal processing, then

checks the status to make sure that the information is still current.  If

not, it will loop back for 32 attempts to get a "current" set of information.

However, as the information includes the time down to 1/1000 of a second,

VMDEBUG cannot do the required processing before something changes.  After

running out of attempts, the program assumes it cannot read the correct

information as there is no clock at the I/O addresses, and reports that

the real-time clock is not installed.

Another problem that has not been seen, but could very well exist, is a

program that sends the I/O hardware a command, wanders off for a while, and

then sets a flag and waits for the completion interrupt that has already

been processed.  Such a scenario would go like this:

Send a command to controller xxxx    (program assumes that xxxx is slow)

Write a nice, fancy message to screen

Set the controller status byte to 0, and enter loop waiting for it

to change as the result of the return interrupt from xxxx

However, as VMDEBUG is slower than a real 8088, the interrupt came along while

the routine was writing to screen, the flag was set, and the I/O operation

was completed.  The status byte is cleared by the program upon entry to the

loop and will never again change, so the loop is infinite.

As mentioned before, VMDEBUG does buffering of multiple keyboard

interrupts, so that the user may type at his/her own speed.  There are

several things to be careful of in terms of the keyboard.

1) There will often appear to be an extraneous keyboard interrupt at

the start of program execution (after a G, P, or X command.)

This is the release of the "return" key and is very hard to avoid.

Either press and release the key very fast, or live with it.  I have

considered both pausing 1/2 second and then proceeding (to allow time

for the key to be released), or waiting for the key release signal

before proceeding.  Either of these approaches can cause more

problems than they solve and consequently are not implemented.

Page 138

2) Do NOT fill the type-ahead buffer.  It will not hurt anything, but

that short "beep" that signals buffer full is a pain when executed

under VMDEBUG.

3) At least one program, QEGA V0.96 that accompanies the Quadram EGA board,

causes a problem when it places itself in the keyboard queue.  When used

with an IBM-AT (old version, 6MHZ), there is an interaction between the

ROM BIOS and the QEGA program that destroys the BIOS keyboard buffer

under certain circumstances.  The interaction is this:

a) Before the IRET of the keyboard routine, interrupts are enabled

again by resetting the 8259 and setting the interrupt enable

flag.  With VMDEBUG running, it is now possible to process the

next keyboard interrupt.  If this is the key release signal,

there is no problem.  If there are more keypresses to be

processed, they are processed BEFORE the IRET from the FIRST

keyboard interrupt.  This does not cause an re-entrancy problem,

but does lead to the problem below:

b) The QEGA program seems to assume that ONLY ONE key could have

possibly been pressed between entry to the QEGA program, the

trip to the ROM BIOS, and the return to the QEGA program.  As

a result, it truncates the additional keypresses from the

keyboard queue.  Thus, it appears that the keypresses after the

first have disappeared.

Interestingly, this problem does not appear on a AT-clone that uses

the Phoenix BIOS.

4) VMDEBUG appears to work with Sidekick (at least version 1.11C),

although Sidekick is well known for being nasty about sharing

the keyboard interrupt, which VMDEBUG sets as you enter the G, P,

and X commands, and resets as you leave.  Note, however, that you

cannot bring up the virtual copy of Sidekick, as the "real" copy

of Sidekick will not pass along the proper keystrokes.

Not being able to invoke the virtual copy of a resident program

may be generic to programs that expect a hotkey.  Running VMDEBUG

under VMDEBUG, however, you can give each program a different

hotkey.

5) Some AT-clones generate keyboard click internally, as the keystroke

BIOS is executing.  When using VMDEBUG, there will be 2 clicks for

each key, one as the "real" BIOS processes the keystroke, and one

as the BIOS under VMDEBUG processes the keystroke sometime later.

This sounds weird, but it does not hurt anything.

6) If the hotkey is pressed in the middle of a shift key sequence, the

return to VMDEBUG will occur, but when control is returned to the

interpreted program, the keyboard will remain in a shifted state until

the right shiftkey is pressed and released again.  This is due to the

keyboard flags saved in the virtual BIOS area.

7) When you return to DOS after exiting VMDEBUG, the sequence stored in

the DOS buffer (called up by key F3) is the last sequence typed

into DOS running under VMDEBUG, not the last command in which VMDEBUG

was executed.  This is due to the buffer stored in the transient

portion of COMMAND.COM, which is modifed by DOS running under VMDEBUG.

Page 139

It is a bad idea to interrupt VMDEBUG while floppy disk or hard disk I/O

is going on, as the hardware will signal completion when it is ready, and

if VMDEBUG is not available to pass the interrupt to the interpreted program,

when execution resumes it will be waiting for an interrupt that never comes.

When trying to boot Concurrent CPM-86 (CCPM, also known as Concurrent

PC-DOS) under VMDEBUG, note that CCPM assumes that the interrupt vectors

have their original, ROM values.  If DOS is loaded, the interrupt vectors

will have their PC-DOS (or MS-DOS) values, which usually point into RAM,

which is overwritten by CCPM as it boots, but it does NOT set the

interrupt vectors before using them.  This presents itself in the printer

table vector (INT 17h) and probably others I haven't bothered to trace down

yet on a Zenith Z150, although CCPM will boot on a different machine

(a Standard Brand 286 AT clone) under VMDEBUG.

Note that VMDEBUG will not run under CCPM, though CCPM does run under

VMDEBUG.

Page 140

Appendix A

Format of VMDEBUG image files

-----------------------------

A VMDEBUG image file is a series of binary records containing the port

information for the monochrome adapter, the port information for the color

graphics adapter, the port information for the EGA, the registers, the screen

image, and the virtual memory.  There are no header records, the information

is retrieved by reading a fixed number of bytes at a known location within the

file.  The only exception is the virtual memory of the computer itself, whose

size varies.  This data area is recorded at the end of the file, with a known

starting position.

The records and their contents are:

1st record:  monochrome display adapter CRTC registers, 26 bytes long

The registers are recorded in the same sequence as the hardware

registers.  This set of registers serves dual purposes: when used

with a MDA card, the first 18 bytes are significant, and have the

meanings listed below.  When used with a EGA card in monochrome mode,

the first 25 bytes are significant, and have the meanings listed in

the 2nd set of definitions below.

Byte 0:  Horizontal total characters - 1

Byte 1:  Horizontal displayed characters

Byte 2:  Horizontal synch position (up/down centering)

Byte 3:  Horizontal synch width in characters

Byte 4:  Integer Vertical total lines - 1  [int(lines*scan lines/char)-1]

Byte 5:  Fraction Vertical total lines - 1

Byte 6:  Vertical displayed rows

Byte 7:  Vertical synch position (row)

Byte 8:  Interlace mode

Byte 9:  Maximum scan line address

Byte 10: Cursor starting scan line

Byte 11: Cursor ending scan line

Byte 12: Memory address MSB (bits 0-5: half the offset to be at top left)

Byte 13: Memory address LSB (half the offset to be at top left)

Byte 14: Cursor address MSB

Byte 15: Cursor address LSB

Byte 16: reserved for light pen

Byte 17: reserved for light pen

EGA definitions:

Byte 0:  Horizontal total characters, including retrace

Byte 1:  Horizontal dispaly enable end

Byte 2:  Start horizontal blanking

Byte 3:  End horizontal blanking

Byte 4:  Start horizontal retrace pulse

Byte 5:  End horizontal retrace

Byte 6:  Vertical total

Byte 7:  CRT Controller overflow

Byte 8:  Preset row scan

Page 141

Byte 9:  Maximum scan line address

Byte 10: Cursor starting scan line

Byte 11: Cursor ending scan line

Byte 12: Memory address MSB (8-bits)

Byte 13: Memory address LSB (8-bits)

Byte 14: Cursor address MSB

Byte 15: Cursor address LSB

Byte 16: Vertical retrace start

Byte 17: Vertical retrace end

Byte 18: Vertical display enable

Byte 19: Offset (Logical line width)

Byte 20: Underline location

Byte 21: Start vertical blanking

Byte 22: End vertical blanking

Byte 23: Mode control register

Byte 24: Line compare

Byte 25: Reserved (currently unused)

2nd record:  monochrome display adapter ports, 8 bytes long

Byte 0: port 3B4 (addressable as 3B0, 3B2, 3B4, or 3B6 after the hardware)

Byte 1: port 3B5 (addressable as 3B1, 3B3, 3B5, or 3B7 after the hardware)

Byte 2: port 3B8 (mode control register)

Byte 3: port 3B9 (reserved)

Byte 4: port 3BA (status register)

Byte 5: port 3BB (reserved for light pen)

Byte 6: port 3BF (Configuration Data Switch, for Hercules-type MDA cards)

Byte 7: Reserved (currently unused)

3rd record:  color graphics adapter CRTC registers, 26 bytes long

The registers are recorded in the same sequence as the hardware

registers.  This set of registers serves dual purposes: when used

with a CGA card, the first 18 bytes are significant, and have the

meanings listed below.  When used with a EGA card in color mode,

the first 25 bytes are significant, and have the meanings listed in

the 2nd set of definitions below.

Byte 0:  Horizontal total characters - 1

Byte 1:  Horizontal displayed characters

Byte 2:  Horizontal synch position (up/down centering)

Byte 3:  Horizontal synch width in characters

Byte 4:  Integer Vertical total lines - 1  [int(lines*scan lines/char)-1]

Byte 5:  Fraction Vertical total lines - 1

Byte 6:  Vertical displayed rows

Byte 7:  Vertical synch position (row)

Byte 8:  Interlace mode

Byte 9:  Maximum scan line address

Byte 10: Cursor starting scan line

Byte 11: Cursor ending scan line

Byte 12: Memory address MSB (bits 0-5: half the offset to be at top left)

Byte 13: Memory address LSB (half the offset to be at top left)

Byte 14: Cursor address MSB

Byte 15: Cursor address LSB

Byte 16: light pen MSB

Byte 17: light pen LSB

Page 142

EGA definitions:

Byte 0:  Horizontal total characters, including retrace

Byte 1:  Horizontal dispaly enable end

Byte 2:  Start horizontal blanking

Byte 3:  End horizontal blanking

Byte 4:  Start horizontal retrace pulse

Byte 5:  End horizontal retrace

Byte 6:  Vertical total

Byte 7:  CRT Controller overflow

Byte 8:  Preset row scan

Byte 9:  Maximum scan line address

Byte 10: Cursor starting scan line

Byte 11: Cursor ending scan line

Byte 12: Memory address MSB (8-bits)

Byte 13: Memory address LSB (8-bits)

Byte 14: Cursor address MSB

Byte 15: Cursor address LSB

Byte 16: Vertical retrace start

Byte 17: Vertical retrace end

Byte 18: Vertical display enable

Byte 19: Offset (Logical line width)

Byte 20: Underline location

Byte 21: Start vertical blanking

Byte 22: End vertical blanking

Byte 23: Mode control register

Byte 24: Line compare

Byte 25: Reserved (currently unused)

4th record: color graphics adapter ports, 6 bytes long

Byte 0: port 3D4 (addressable as 3D0, 3D2, 3D4, or 3D6 after the hardware)

Byte 1: port 3D5 (addressable as 3D1, 3D3, 3D5, or 3D7 after the hardware)

Byte 2: port 3D8 (mode control register)

Byte 3: port 3D9 (color select register)

Byte 4: port 3DA (status register)

Byte 5: port 3DB (clear light pen latch)

5th record: EGA registers, 8 bytes long

Byte 0: port 3C2 (miscellaneous register)

Byte 1: port 3xA (3BA for monochrome, 3DA for color modes. Feature control)

Byte 2: port 3C4 (sequencer address)

Byte 3: port 3CC (graphics I position)

Byte 4: port 3CA (graphics II position)

Byte 5: port 3CE (graphics SI address)

Byte 6: port 3C0 (attribute address, used with attribute flip flop below)

Byte 7: Reserved (currently unused)

Page 143

6th record: SI registers, 10 bytes long

The registers are recorded in the same sequence as the hardware

registers.

Byte 0: Set/Reset

Byte 1: Enable set/reset

Byte 2: Color compare

Byte 3: Data rotate

Byte 4: Read map select

Byte 5: Mode

Byte 6: Miscellaneous

Byte 7: Color don't care

Byte 8: Bit mask

Byte 9: Reserved (currently unused)

7th record: Sequencer registers, 6 bytes long

The registers are recorded in the same sequence as the hardware

registers.

Byte 0: Reset register

Byte 1: Clocking mode

Byte 2: Write map mask

Byte 3: Character map select

Byte 4: Memory mode

Byte 5: Reserved (currently unused)

8th record: Attribute registers, 34 bytes long

The registers are recorded in the same sequence as the hardware

registers.  Although only 20 registers are defined for attribute

values, a register address of 32 (bit 5) is used to enable memory

access to the color palette, and so the extra registers are saved

to make sure that the attribute address is always valid within VMDEBUG.

Byte 0:  Palette register 0

Byte 1:  Palette register 1

Byte 2:  Palette register 2

Byte 3:  Palette register 3

Byte 4:  Palette register 4

Byte 5:  Palette register 5

Byte 6:  Palette register 6

Byte 7:  Palette register 7

Byte 8:  Palette register 8

Byte 9:  Palette register 9

Byte 10: Palette register 10

Byte 11: Palette register 11

Byte 12: Palette register 12

Byte 13: Palette register 13

Byte 14: Palette register 14

Byte 15: Palette register 15

Byte 16: Mode control

Byte 17: Overscan color

Byte 18: Color plane enable

Byte 19: Horizontal pel panning

Bytes 20-33: not used

Page 144

9th record: Attribute flip flop, 2 bytes long

Byte 0: 0 = next write to port 3C0 is address,

1 = next write to port 3C0 is data

Byte 1: always 0

10th record: VGA register information, 1024 bytes long

Bytes 0-1023: VGA according to BIOS function 1Ch, subfunction 1.

This is the video registers and DAC registers. The

BIOS RAM data area is part of the virtual memory, and

does not need to be saved again.

11th record: 8088 registers, 28 bytes long

All registers are stored low byte first.  The ordering of registers is

according to opcode value.

Bytes 0-1:    AX register (AL byte 0, AH byte 1)

Bytes 2-3:    CX register (CL byte 2, CH byte 3)

Bytes 4-5:    DX register (DL byte 4, DH byte 5)

Bytes 6-7:    BX register (BL byte 6, BH byte 7)

Bytes 8-9:    SP register

Bytes 10-11:  BP register

Bytes 12-13:  SI register

Bytes 14-15:  DI register

Bytes 16-17:  ES register

Bytes 18-19:  CS register

Bytes 20-21:  SS register

Bytes 22-23:  DS register

Bytes 24-25:  flags register

Bytes 26-27:  IP register

12th record: saved screen memory, 4096 bytes

The saved screen memory corresponds to page 0 of a 25 by 80 text screen,

and is taken from whichever video adapter is the default at the start of

VMDEBUG.

13th record: saved virtual memory, size varies in 64K blocks

The saved virtual memory is the total contents of the virtual memory used

by the VMDEBUG program.  It starts at offset 5274 (149A hex) within the image

file, and takes up the remainder of the image file.  If you need to address

the virtual memory within the image file, the desired location may be found

by adding 0149 to the segment address, and 000A to the offset.  This is the

desired byte location within the image file.

Image loader program

--------------------

It is possible to restore an image saved from VMDEBUG into the real

system memory, and to execute it.  The procedure is dangerous, however. The

warnings on file buffers and disk caching from chapter 7 apply here as well.

Page 145

When an image is restored to real memory, the image overwrites the TOTAL

contents of the system, including DOS.  This means that DOS is now in the

state it was while running in VMDEBUG, including memory size.  Thus, as far

as DOS is concerned, it is running in a much smaller machine than it actually

is.  The additional memory will be ignored.

Also, note that the hardware may not be in the state that the image

assumes.  The floppy drive may not be running, sound may not be enabled,

the DMA addresses may not be set (if the image was saved during a disk

I/O operation), among other things.

The program presented here assumes that the CGA card will receive the

saved video information, and that all of DOS (DOS, file buffers, device

drivers, but not necessarily COMMAND.COM) will fit in the first 65280 (FF00

hex) bytes of memory.  What makes this program interesting is that DOS calls

must be used to read the file, and that DOS is the first thing that is

normally loaded from the image file.  One solution to this problem is

presented in the program below, and the advanced user can find more.

Page 146

RETF    macro

db      0cbh                    ;force MASM to assemble a RET FAR

;without a PROC FAR

endm

fun     segment

assume  cs:fun,ds:fun,es:fun,ss:fun

;

;  Program to load a VMDEBUG image file to real memory.

;

;   Assumptions:

;     640K machine, DOS fits in 65280 bytes, will restore CGA screen

;

org     100h                    ;this is a .COM file

loader: mov     si,offset startmoved    ;DS:SI = next routine

mov     ax,8800h                ;new program segment

mov     es,ax                   ;ES = new segment

mov     di,si                   ;ES:DI = new location for next routine

mov     cx,offset progend       ;end of program to move + 1

sub     cx,si                   ;difference is # of bytes to move

cli                             ;do not disturb

cld                             ;up

rep     movsb                   ;move it!

push    ax                      ;push new segment

mov     bx,offset startmoved    ;get new offset

push    bx                      ;push it

RETF                            ;Do a far return to the moved code

errorabort:

int     20h                     ;error, return to DOS

;note that INT 20 needs CS pointing to

;the PSP

startmoved:

mov     ss,ax                   ;and SS

mov     sp,offset startmoved    ;and set SP to below program

push    ds                      ;save old CS(=DS) for error abort

mov     ds,ax                   ;set DS

sti                             ;allow interrupts again

mov     ax,03d00h               ;set up for DOS call

mov     dx,offset imagename     ;filename of image file

int     21h                     ;call DOS

jae     loadit                  ;no carry = no error, go load it

mov     ax,offset errorabort    ;time to quit

push    ax                      ;quit offset to stack

RETF                            ;far return back to the exit

;

;   It's now too late for error aborts

;

loadit: mov     word ptr [filehandle],ax  ;save the file handle

mov     bx,ax                   ;handle to BX for read

mov     cx,34                   ;read 34 bytes, the MDA parameters

mov     dx,offset thedump       ;into the dump area

mov     ah,3fh                  ;fn = read

int     21h                     ;call DOS

Page 147

;

;   The next read will be the CGA parameters, which will be kept

;

mov     bx,word ptr [filehandle]  ;handle to BX for read

mov     cx,32                   ;read 32 bytes, the CGA parameters

mov     dx,offset videostuff    ;into the video info buffer

mov     ah,3fh                  ;fn = read

int     21h                     ;call DOS

;

;   Next, read past the EGA/VGA parameters

;

mov     bx,word ptr [filehandle]  ;file handle to BX

mov     cx,8+10+6+34+2+1024     ;read past the next 6 records

mov     dx,offset thedump       ;into the disposal area

mov     ah,3fh                  ;fn = read

int     21h                     ;call DOS

;

;   Read the registers

;

mov     bx,word ptr [filehandle]  ;file handle to BX

mov     cx,28                   ;read 28 bytes, the registers

mov     dx,offset regs          ;into the register buffer

mov     ah,3fh                  ;fn = read

int     21h                     ;call DOS

;

;   Restore the video memory

;

mov     bx,word ptr [filehandle]  ;file handle to BX

mov     cx,4096                 ;size of video buffer

mov     ax,0b800h               ;video segment

mov     ds,ax                   ;move to DS

mov     dx,0                    ;load location is DS:0

mov     ah,3fh                  ;fn = read

int     21h                     ;call DOS

push    cs                      ;move CS

pop     ds                      ;back to DS

;

;   Now read in the DOS information from the virtual memory

;

mov     bx,word ptr [filehandle]  ;get the handle

mov     cx,0ff00h               ;a big number

mov     dx,9000h                ;use 9000h

mov     ds,dx                   ;as the segment

mov     dx,0                    ;9000:0 is the transfer address

mov     ah,3fh                  ;fn = read

int     21h                     ;call DOS

;

;   Now, to read the rest of memory

;

Page 148

readloop:

mov     bx,word ptr cs:[filehandle]  ;get the handle

mov     cx,0ff00h               ;big number

lds     dx,dword ptr cs:[loadofs]    ;get the load address

mov     ah,3fh                  ;fn = read

int     21h                     ;call DOS

mov     dx,ax                   ;save bytes read

shr     ax,1                    ;/2

shr     ax,1                    ;/4

shr     ax,1                    ;/8

shr     ax,1                    ;/16

add     word ptr cs:[loadseg],ax     ;add to segment

mov     ax,dx                   ;get bytes read again

and     ax,000fh                ;mod 16

add     word ptr cs:[loadofs],ax     ;add to offset

cmp     dx,0                    ;were 0 bytes read?

jne     readloop                ;no, keep going

mov     ax,cs                   ;get CS

mov     ds,ax                   ;into DS

;

;   now start the final setup

;

mov     dx,3d4h                ;6845 index register

mov     cx,18                  ;need 18 bytes

mov     bx,offset videostuff   ;base of video data

videoloop1:

mov     ax,18

sub     ax,cx                  ;18-cx = index to 6845 regs

mov     si,ax                  ;si = index into video data

out     dx,al                  ;select 6845 register

inc     dx                     ;point to 6845 data register

mov     al,byte ptr [bx+si]    ;get the data for this port

out     dx,al                  ;out data to video register

dec     dx                     ;back to 6845 index register

loop    videoloop1             ;do next register

add     bx,26                  ;add offset to port data

mov     cx,2                   ;only 2 ports are needed

mov     dx,3d8h                ;starting CGA register

videoloop2:

mov     si,4

sub     si,cx                  ;4-cx = index to port values

mov     al,byte ptr [bx+si]    ;get port data

out     dx,al                  ;and out it

inc     dx                     ;address of next port

loop    videoloop2             ;do next port

;

;   Now move DOS

;

Page 149

xor     ax,ax                  ;AX = 0

mov     es,ax                  ;ES = 0

mov     di,ax                  ;DI = 0

mov     si,ax                  ;SI = 0

mov     dx,9000h               ;DX = 9000h, DOS buffer segment

mov     ds,dx                  ;DS:SI = loaded DOS, ES:DI = real DOS

mov     cx,0ff00h              ;big number again

cli                            ;DO NOT DISTURB!

cld                            ;UP!

rep     movsb                  ;new DOS

mov     ax,cs                  ;mov CS

mov     ds,ax                  ;to DS

mov     bx,offset regs         ;pointer to register information

mov     ax,word ptr [bx]       ;set AX register

mov     cx,word ptr [bx+2]     ;set CX register

mov     dx,word ptr [bx+4]     ;set DX register

mov     sp,word ptr [bx+8]     ;set SP register

mov     bp,word ptr [bx+10]    ;set BP register

mov     si,word ptr [bx+12]    ;set SI register

mov     di,word ptr [bx+14]    ;set DI register

mov     es,word ptr [bx+16]    ;set ES register

mov     ss,word ptr [bx+20]    ;set SS register

push    word ptr [bx+24]       ;push FLAGS

push    word ptr [bx+18]       ;push CS

push    word ptr [bx+26]       ;push IP

mov     ds,word ptr [bx+22]    ;set DS register

mov     bx,word ptr cs:[bx+6]  ;set BX register

iret                           ;and start saved program!

;

;   start of data

;

filehandle:     dw      0

loadofs:        dw      0

loadseg:        dw      0ff0h

imagename:      db      'PROGRAM.IMG',0

videostuff:     db      32 dup (0)

regs:           db      28 dup (0)

thedump:        db      0

;

;   the program end

;

progend:

fun     ends

end     loader

Page 150